3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208746r02

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3 Conclusion update

Document for:	Approval

Agenda Item:	9.12

Work Item / Release:	FS_5GSATB / Rel-18

Abstract: Conclusion update for KI#3.

1. Introduction

1. Adjust the conclusion structure as following:

1. Assumption

2. On-board PSA UPF or UL CL/BP UPF/local PSA selection

3. URSP configuration

4. User plane configuration

Some sentences related to, e.g. UPF as ULCL/BP, are not belongs to “on-board UPF acing as PSA” case, thus should be moved.

2. Add description on user plane configuration to enable local data switch.

- For on-board UPF acting as PSA case:

- If SMF selects the on-board UPF as PSA of UE’s PDU Session, the SMF configures the UE’s N4 session to forward/detect packet to/from the internal interface.

  - To detect/forward packets from “access side” (i.e., packets received from the UE) towards other UEs for UE-to-UE communication, the PDR contains Source Interface set to "access side", the FAR contains the Destination Interface set to internal interface.

  - To detect/forward packets from the UPF internal interface (i.e. packets sent to the UE by other UEs) towards the current UE, the PDR contains Destination Address set to IP address of the UE, the FAR contains Outer Header Creation indicating the N3/N9 tunnel information, and Destination Interface set "access side".

- Based on pre-configuration, the SMF establishes the N19 tunnel between the two UPFs and corresponding UPF-level N4 rules to support local data switch.

  - To detect/forward packets (from internal interface to N19) towards UEs anchored at other UPFs, the PDR in the UPF-level rules contains the Destination Address set to IP address(es) of the target UPF address pool, the FAR contains Outer Header Creation indicating the N19 tunnel information, Destination Interface set to "core side".

  - To detect/forward packets (from N19 to internal interface), the PDR contains Source Interface set to "core side", and CN Tunnel Information set to N19 tunnel header (i.e. N19 GTP-U F-TEID); and the FAR contains Destination Interface set to internal.

Figure 1 presents an example of how the PSA UPF on-board forwards IP packets, including packets from UE1 to UE2 under the same PAS UPF and packets from UE1 to UE3 under different PAS UPFs.

Figure 1. User plane handing for on-board UPF acting as PSA case

For Packets from UE1 to UE2:

- The PDR of UE1' N4 session contains Source Interface set to "access side", the FAR forward packets to the internal interface.

② The PDR of UE2' N4 session detects that Dst IP address of packets is UE2's address, the FAR forward the packets through N3/N9 tunnel to UE2 side.

For packets from UE1 to UE3, UE1 and UE3 are under different PAS UPFs:

- . The PDR of UE1' N4 session contains Source Interface set to "access side", the FAR forward packets to the internal interface.

③. The PDR of UPF-level session detects that Dst IP address of packets belongs to the PAS UPF2's address pool, the FAR forward the packets through N19 tunnel to PSA UPF2.

④. The PDR of UPF-level session detects packets from N19, the FAR forward the packets to the internal interface.

⑤. The PDR of UE3 N4 session detects packets to UE3, the FAR forward the packets through N3/N9 to UE3 side.

In summary, the IP address(es) in UPF address pools are utilized to detect UE-to-UE communication packets which are sent to the internal interface and optional N19 for local data switch.

- For on-board UPF acting as UL CL/BP UPF/local PSA case:

-	Based on 1) AF request including IPs/DNNs of UEs which require UE-to-UE communications; or 2) target IP reported by on-ground PSA UPF as current reporting mechanism defined in TS 23.501 [1], SMF determines the UEs are anchored on the same or different on-board Local PSA.

-  SMF configures Local PSA(s) based on the IP address(es) of target UEs, then UE-to-UE communication packets are detected and sent to the internal interface and optional N19 for local data switch. The local data forwarding rules on L-PSA can be configured based on 5GVN user plane handing mechanism.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-27.

# * * * * First change * * * *

8.3	Conclusions on Support of Local Data Switching via UPF on-board

1. Assumption:

-	Only a single SMF is supported for Local Data Switching and N19 via UPF on-board, i.e. both UEs are served by the same SMF.

-	The UE to UE traffic may be locally routed by UPF on-board to the target UE without traversing back to the satellite gateway on the ground;

-	N19 tunnel may be established between two UPFs on-board different satellites to locally switch UE to UE traffic.

2. On-board PSA UPF or UL CL/BP/local PSA selection is performed as following (Same mechanism can be reused to KI#2):

-	For each GEO satellite ID, a DNAI value is assigned by the operator. SMF is configured with DNAI corresponding to each GEO satellite ID. The SMF determines DNAI based on the configuration and GEO satellite ID received from AMF.

-	On-board UPF selection is done based on the DNAI as current UPF selection defined in TS 23.501 [2] and TS 23.502 [3].

3. URSP configuration:

For on-board UPF acting as PSA case:

-	A combination of DNN/S-NSSAI is assigned by the operator to the services deployed on GEO satellite. Based on GEO satellite backhaul category from AMF, PCF determines URSP rule including the DNN/S-NSSAI for such services. PCF sends the URSP rules to UEs allowed to access the services on GEO satellite. (Same mechanism can be reused to KI#2)

4. User plane configuration:

-	The UE to UE traffic may be locally routed by UPF on-board to the target UE without traversing back to the satellite gateway on the ground;

-	For the on-board UPF acting as a PSA case, SMF creates N4 rules on the on-board UPF to enable local data switching as following

Based on pre-configuration, the SMF establishes the N19 tunnel between the two UPFs and corresponding UPF-level N4 rules to support local data switch. The IP address(es) in UPF address pools are utilized in the UPF-level N4 session to detect UE-to-UE communication packets which are sent to  pre-configured N19 for local data switch.

-	N19 tunnel may be established between two UPFs on-board different satellites to locally switch UE to UE traffic.

-	For on-board UPF acting as ULCL/BP/local PSA case:

-	Based on 1) AF request including IPs/DNNs of UEs which require UE-to-UE communications, which will re-uses an existing NEF API and no impacts are foreseen; or 2) target IP reported by on-ground PSA UPF as current reporting mechanism defined in TS 23.501 [1], SMF determines the UEs are anchored on the same or different on-board Local PSA.

-  SMF configures Local PSA(s) based on the IP address(es) of target UEs, then UE-to-UE communication packets are detected and sent to the internal interface and optional N19 for local data switch. The local data forwarding rules on L-PSA can be configured based on 5GVN user plane handing mechanism.

# * * * * End of changes * * * *