3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208747

Elbonia, 10 - 17 Oct, 2022

Source:	Huawei, HiSilicon

Title:	KI#8, Evaluation and conclusion of KI#8

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: This document provide the evaluation and conclusion for KI#8.

# 1. Introduction

There are 5 candidate solutions proposed to address key issue#8, i.e. solution#22/#30/#31/#32/#33, these solutions can be categorized as follows:

-	Category#1) The location service continuity for UE moves between NG-RAN nodes, i.e., solution#22/#30.

-	Category#2) The location service continuity between EPS and 5GS (bi-direction), i.e., solution#31/#32/#33.

This document provides the evaluation and conclusion for KI#8.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.1	Evaluation of KI #8

There are 5 candidate solutions proposed to address key issue#8, i.e. solution#22/#30/#31/#32/#33, these solutions can be categorized as follows:

- Category#1) The location service continuity for UE moves between NG-RAN nodes, i.e., solution#22/#30.

- Category#2) The location service continuity between EPS and 5GS (bi-direction), i.e., solution#31/#32/#33.

7.1.2. Location service continuity between EPS and 5GS (bi-direction)

Solution# (31, 32, 33) focus on the location service continuity between EPS and 5GS (bi-direction), the solutions comparison is shown in Table 7.1.2-1 based on following evaluations:

A. Solutions principle

(a) Solution#31

-	 For periodic/deferred location service continuity from 5GS-EPS, 5GC determines the corresponding location QoS can be applicable to EPS, LMF sends these parameters to UE.

-	For both periodic/deferred location service continuity and non-periodic deferred location service continuity, after UE handover to EPS/5GS, the AMF/MME sends the handover notification and the location information including the applicable location QoS to GMLC to trigger the location service procedure in EPS/5GS.

-	For periodic/deferred location service, if UE finds periodical event is triggered, the UE sends the suitable message and location QoS which can be applicable to EPS/5GS.

(b) Solution#32

For non-periodic deferred location service continuity:

-	From 5GS-EPS, AMF forward/receive location information to/from MME to trigger positioning procedure; mapping the Provide Subscriber Location message to and from the Namf_Location_ProvideLocation request message; forward the positioning result to MME via N26.

-	Form EPS to 5GS, MME forward/receive location information to/from AMF to trigger positioning procedure; forward the positioning result to AMF via N26.

(c) Solution#33

For periodic/deferred location service continuity from 5GS-EPS, it is unclear how 5GC determines the location QoS can be applicable to EPS and notifies to UE. It is also unclear how the UE sends the suitable event triggered message and carry the suitable location QoS after UE access to EPS.

For periodic/deferred location service continuity from EPS-5GS, it is unclear how the UE sends the suitable event triggered message after UE access to 5GS.

For both periodic/deferred location service continuity and non-periodic deferred location service continuity, after UE handover to EPS/5GS, the AMF/MME trigger to cancel the location service in 5GS/EPS. For both periodic/deferred location service continuity and non-periodic deferred location service continuity, after UE handover to EPS/5GS, the AMF/MME sends the location information to GMLC, GMLC determines the location QoS can be applicable to EPS/5GS to trigger the location service procedure start in EPS/5GS.

B. For whether can be applied to Periodic/deferred location service

Both solution#31 and solution#33 can be applied to Periodic/deferred location service. However, solution#33 cannot guarantee the location service continuity in the “location QoS with multiple QoS class” case. For 5GS-EPS/EPS-5GS, AMF trigger the location cancellation in step 15, i.e., after UE access to EPS/5GS, it is not possible to remove the location context in UE, therefore, the UEs may still report the incorrect event triggered message and incorrect location QoS which are not applicable to EPS/5GS. Besides, considering the UE may move back to 5GS/EPS, the more feasible way could be using the existing procedure to abort the on-going location session instead of cancel the location service, i.e., AMF/MME notify LMF/E-SMLC to abort the LPP/NRPPa/ SLmAP Transaction after UE access to EPS/5GS.

C. For whether can be applied to non-periodic/deferred location service

All solutions #31/32/33 can be applied to nonPeriodic/deferred location service.

D. MME impact

Both solution#31 and solution#/33 propose the MME need indicate to GMLC/E-SMLC the handover to EPS/5GS, it has less MME impact. Compared with solution#31 and solution#33, solution#32 has more MME impact, in solution#32, the MME need map the location signalling message between 5GS/EPS and EPS/5GS, MME need use the handover signalling message to forward the location formation from 5GS to EPS/from EPS to 5GS, and the MME need indicate to E-SMLC /GMLC the handover to 5GS/EPS.

E. N26 interface impact

Solution#31 and solution#33 decoupling the location service procedure with the interworking procedure, therefore both solution#31 and solution#33 have no N26 interface impact. While solution#32 coupling the location service procedure with the interworking procedure, which has the N26 interface impact.

F. Location QoS mapping

Both solution#31 and solution#33 hold the principle bring minimum impact to EPS and propose the location QoS mapping.

For the “location QoS with multiple QoS class” case, due to the reason the "LCS QoS class" is defined as an enumerate data type and MME only understand the enumerated value {"assured" and "best effort"}, it does not understand the enumerated value "multiple qos". Thus, solution#31 further use examples to clarify how the location QoS mapping is and why the mapped location QoS can be transparent and unaware by EPS, for examples: choose only one set of { “hAccuracy” , “vAccuracy” } from the "LocationQoS". The choosing principle could be e.g., choose the most stringent values pairs of { “hAccuracy”  and “vAccuracy” } from the "LocationQoS" and set the "LcsQoSClass" in "LocationQoS" as the "Best Effort".

Table 7.1.2-1: Solutions comparison for location service continuity between EPS and 5GS (bi-direction)

| Solution | Sol#31 | Sol#32 | Sol#33 |
| --- | --- | --- | --- |
| N26 interface impact | No | Yes | No |
| MME impact | Less impact | More impact | Less impact |
| Coupling the interworking procedure | No | Yes | No |
| LCS client aware | No | No | No |
| Applicable to non-periodic/deferred location service | Yes | Yes | Yes |
| Applicable to Periodic/deferred location service | Yes | No | Yes, but may incur location service interruption in the “location QoS with multiple QoS class” case. |
| Location service continuity | Yes | Not guaranteed in the Periodic/deferred location service, including the “location QoS with multiple QoS” class case | Not guaranteed in the Multiple QoS class case |

In general, the principles proposed in solution#33 can satisfy the stringent location service continuity requirements, i.e., it can avoid the abnormal case:

- caused by the UE report the incorrected event trigged message dure to move to another system (this not relevant to the Location QoS and this should also not be reckon as a rare case, UE need report the correct event triggered message)

- caused by the “multiple QoS class” (the multiple QoS is not the rare case and should be categorised in the general case, since it is not realistic to forbidden the LCS client to request the location QoS with multiple QoS class)

Also, solution#33 utilize a more elegant way to keep the location service continuity, i.e., it does not restart the session when UE handover between different systems.

The principles proposed in solution#31 can satisfy the loosen location service continuity requirements, i.e., it tolerant the abnormal case happened in the network for the multiple QoS case (although this should not be reckoned as a rare case) or caused by the UE report the incorrected event trigged message due to UE move to another system. Also, it is allowed some location session disruption during the UE moving.

# * * * * Next change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Support of location service continuity in case of UE mobility

8.1.1	Conclusions

The following conclusions is proposed for KI#8:

Based on the location service requirement and operator’s policy, if stringent location service cotunity is needed, the following principle should be adopted:

Categary A: Continuity between EPS and 5GS (bi-direction)

-	For periodic/deferred location service continuity from 5GS to EPS:

- The LMF should determine whether UE can move between 5GS and EPS and notify UE the corresponding location QoS can be applied to EPS.

- After the location event is triggered, the UE should determine and send the suitable event triggered message and carry the suitable location QoS which can be applicable to EPS.

-	 For periodic/deferred location service continuity from EPS to 5GS:

- After the location event is triggered, the UE should determine and send the suitable event triggered message which can be applicable to 5GS.

-    For both non-periodic/deferred location service continuity and periodic/deferred location service continuity:

- The LMF should determine the location information including the location QoS can be applicable to EPS, GMLC notify this information to E-SMLC to trigger the location procedure start in EPS.

- After handover complete, the AMF/MME should notify GMLC.

Based on operator’s policy, if loosen location service cotunity requirement is needed, the following principle should be adopted:

For 5GS -> EPS mobility

-	(During Hand-over) The source AMF shall cancel the LCS session (e.g. to LMF), responding to GMLC with a new cause for HO and with the target MME ID

-	GMLC then initiates a new location request to the target MME identified in AMF message cancelling the LCS session over 5GS possibly translating location parameters such as QoS, user Id (no privacy check is needed)

-	The location procedure takes place in EPS ; location estimation is returned to GMLC which returns the estimate to LCS Client

NOTE:	This ensures a common procedure for all types of location and positioning methods (and regardless of where the hand-over takes place in the on-going location procedure) with no impacts to EPS

For EPS -> 5GS mobility

-	(During Hand-over) The source MME shall cancel the LCS session over EPS as currently specified, and responds to GMLC with a new cause for HO and with the target AMF ID

-	GMLC then initiates location request to target AMF identified in MME cancel message possibly translating location parameters such as QoS, user Id (no privacy check is needed)

-	The location procedure takes place in 5GS ; location estimation is returned to GMLC which returns the estimate to LCS Client

For both direction of mobility between 5GS and EPS, the GMLC used over 5GS (respectively EPS) can issue location request over EPS (respectively 5GS).

# * * * * End of changes * * * *