SA WG2 Meeting #S2-153E	S2-2208771r09

10 - 17 Oct, 2022, E-Meeting

Source: 	Ericsson, Qualcomm, LG Electronics

Title: 	KI#6: Evaluation and conclusion of Key Issue #6

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an evaluation and a conclusion of Key Issue #6.

# Discussion

There are 10 solutions in TR 23.700 v1.1.0 addressing key issue #6. These are: Solution #7, #8, #9, #10, #13, #17, #25, #38, #39 and #47. In this contribution an evaluation of these solutions is proposed. The aims and aspects of key issue #6 are as follows:

This key issue aims to study whether the existing mechanisms for overload control in the network can support all the requirements in clause 6.41 of TS 22.261 [2] "Providing Access to Local Services" and whether any enhancements or additional mechanisms need to be defined. The following aspects will be considered:

-	How to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

-	How to minimize the impact on the UE's communication e.g. to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

NOTE:	The solution for this KI may need to consider mechanism developed for KI#5 "Enabling access to localized services via a specific hosting network".

The solutions are divided in two groups, one called "The avoidance-based method", and another called "The runtime-based method". The methods and solutions are described and evaluated, and a conclusion is proposed.

# Proposal

Add the following evaluation and conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

*** NEXT CHANGES ***

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.6	Key Issue #6: Support for returning to home network

The following principles from the evaluation are recommended to be considered during the normative work:

-  In avoidance-based method, the home network AMF can continuously negotiate the number of returning UEs per unit time with the hosting network in order to mitigate an overload at the home network in an adaptive manner based on the load condition at the home network and without making UEs to wait unnecessarily, as described in solution #47.

-	The home/serving network can re-use existing mechanisms for Control Plane Load Control, Congestion and Overload Control described in clause 5.19 of TS 23.501 [3] when load level reaches a certain threshold and overload control mechanism are triggered.

-	Additional mechanisms can be implemented to ensure spreading of return of the UEs to home network without any normative impacts to 3GPP specifications.

NOTE :	Means to describe how to spread returning UEs can be captured as informative description, during normative specification work.

*** END CHANGES ***