3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208784r08

Elbonia, October 10 – 17, 2022

Source:	Thales

Title:	Conclusions proposal for TR23.700-28

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: a conclusion is proposed.

# 1. Introduction/Discussion

This pCR provides a structure for the conclusions and conclusions.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

# 8	Conclusions

## 8.1	Conclusion on general mobility management and/or power saving

The conclusions for solutions to the requirements R1, R2 for KI#1 and R5 and R6 for KI#2 are the following:

Principles of system behaviour:

-	NTN MEO/LEO satellite or satellite constellation that provides discontinuous coverage is considered as the satellite access in 5GS and EPS for the power saving enhancement

-	Provisioning of UE unreachability period to both UE and AMF/MME shall be specified

-	UE unreachability period provided to the UE and AMF/MME shall include timing information when UE moves out/in of NTN coverage

-	Both network centric and UE centric procedures to determine the UE unreachability time are specified. These two approaches are not mutually exclusive, they serve different use cases and they can co-exist in the same network. In Network centric procedure, the AMF/MME determines the PSM parameters taking into account the UE unreachability period. In UE centric procedure, the UE informs the network of the UE determined UE unreachability period.

AMF/MME information retrieval:

- AMF/MME shall not determine UE unreachability period based on satellite orbit data by itself. The AMF/MME receives UE unreachability time to be used for setting up the power saving parameters or handling mobility management of NTN UE

- AMF/MME obtains UE unreachability period information (timing information when UE moves out/in of NTN coverage) from other NF such as NWDAF or new NF or AF (or a 3rd party server via NEF) or OAM which calculates the UE unreachability period using satellite orbit information.

Editor’s Note: who will provide the coverage information depends on the discussion.

The role of AMF/MME:

- The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and include:

- Periodic registration update timer

- MICO mode with Active Time

- MICO mode with Extended Connected time(only for AMF case)

- eDRX

- taking into account the UE unreachability period in order to make sure the UE in power saving mode out of network coverage, to avoid attempting to page the UE during this time and that the UE need not initiate NAS procedures excessively frequently compared to the Expected UE behaviour, if known by the AMF/MME .

- The mobility management and Power saving mode (PSM) parameters include:

  - Periodic registration update timer

  - MICO mode with Active Time

  - MICO mode with Extended Connected time(only for AMF case)

  - eDRX

- The AMF/MME sets an implicit detach timer based on the coverage information to avoid de-registering or detaching the UE when it is in discontinuous coverage.

- If the AMF/MME detects that the UE in CM-CONNECTED is about to leave the current satellite coverage based on the coverage information, the AMF/MME may trigger N2/S1 release procedure to move UE into CM-IDLE state.

- Optionally, AMF can provide a backoff timer to prevent the UE from sending MO data/signalling while out of coverage.

Regarding which information UE has and how UE retrieves it:

- If possible, accurate coverage information is shared with UE coverage information retrieves from the network with enhanced solution compared to R17

- Satellite coverage data may be transferred to a UE from an external server or an AF (internal or external to PLMN)

- Following release-17, the UE has coverage information from RAN broadcasts

Editor’s Note: LS need to have feedback from CT1 on whether UP or NAS via an AMF/MME (CP) will be used for the transfer depends on the discussion.

Regarding the role of the UE:

- The capability for the UE to be able to determine requested NAS timer values and PSM parameters (e.g. request for eDRX, MICO mode with Active Time) based on its predicted unreachability periods needs to be specified. The UE initiates NAS procedure, with sufficient time to complete the NAS procedure before the start of coverage gap, and negotiates timer values and power saving parameters with AMF/MME as per existing specifications.

- Determination of satellite coverage gaps may be supported by UEs based on available satellite coverage data taking into account UE mobility/trajectory. In this case, UEs will indicate, with sufficient time to complete the NAS procedure before the start of coverage gap, an impending satellite coverage gap duration, when known, to an AMF or MME as described by unreachability period and will notify the AMF or MME when the satellite coverage gap has ended.

Editor's note: Whether the AMF/MME is expected to negotiate the UE determined NAS timer value to some different value is TBD.

## 8.2	Conclusion on overload impacts to a target RAT/PLMN

The conclusions for solutions to the requirement R4, overload impacts to a target RAT/PLMN, for KI#1, are:

Editor’s Note: The conclusion is for discussion during the meeting, depending on conclusion in S2-2208984

## 8.3	Conclusion on alternative RAT/PLMN selection

The conclusions for solutions to the requirement R3, alternative RAT/PLMN selection, for KI#1, are:

Editor’s Note: The conclusion is for discussion during the meeting, depending on conclusion in S2-2209184.

# * * * * End of changes * * * *