3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208802r01

Elbonia, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell, Docomo

Title:	Way forward proposal for Conclusions and Evaluations

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3/ Rel-18

Abstract:  This paper proposes evaluations and conclusions for KI#3

1. Introduction/Discussion

This paper proposes a comprehensive evaluation and conclusions proposal for KI#3.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-41

# * * * * First change * * * *

8.X	Conclusions for Key Issue #3

Editor's note: these are interim conclusions

For support of limited AoS slices not matching deployed TAs it is proposed that:

Option1: secondary TAs per cell based solution

-	If the access to the network slice can be limited to only supporting devices (i.e. the slice customer can control the UE population): to retain the same behaviour as in current system (homogenous support in TA) a new feature is introduced in the system to handle secondary TAs for supporting UEs and networks.

Editor's note, the above bullet is FFS and subject of feedback from RAN WGs.

Option 2: reuse existing URSP rules with per cell level granularity

-	Clarify that URSP rules allows a per cell level granularity that the UE is required to validate before using a URSP including an S-NSSAI, and also that the UE to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. This would then ensure cell level granularity of service availability without any protocol impacts, unless URSP rules is to be explicitly enhanced with new indication that the UE is to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic.

NOTE: 	How the handover can be optimized to prevent the UE from leaving the slice service area (or entering into the slice service area) will be considered during normative phase based on RAN WG feedback.

Option 3: reconfiguration of TAs

-	If the access to the network slice cannot be limited to only supporting devices (i.e. the slice customer cannot control the UE population): reconfiguration of TAs is recommended to keep the end-to-end significance of slice unchanged, but an operator could decide to selectively use the resource allocation based on the approach in solution 26/45 (e.g. if NSAC does not apply for the network slice) and if the charging issues identified for Sol 26/45 related to session status in the CSAN (is it so the PDU session is still fully active outside the AoS?) and admission control (NSAC) are resolved. There are also wasted MO and MT attempts to use the slice as both the UE and AMF are not aware in idle mode whether the UE is inside or outside the AoS as the UE does not know the Slice support at cell level, while the RAN and AMF don't know the UE location at cell level

Editor's note: It is FFS which option are adopted for normative work.

For improved support of temporary network slices:

Option 1: Sending “Timing Information” about the configured/allowed slices to the UE

- “Timing Information” as described in Solution #24 can be used to track the start time, end time, and periodicity of the availability of the network slice, including any related temporary TA. A solution will be standardized that enabled the UE to be updated with timing information related to the availability of the configured/allowed slices. It is proposed to specify that the UE can be updated with timing information about the configured/allowed slices and this same timing information can also be provided from the RAN to the AMF when the serving PLMN RAN is configured with the timing information. The timing information can be associated to TAs, S-NSSAIs for temporary slices that also require deployment/support of temporary TAs. If the termination of a network slice is HPLMN initiated, then this information is passed to UE and RAN UE context in addition to AMF and SMF. If both VPLMN and HPLMN timing information applies the most constraining timing determines a slice availability.

- When the timer associated with a S-NSSAI expires, then the UE and network removes the S-NSSAI locally from the allowed NSSAI if the S-NSSAI present in the allowed NSSAI.

-	When a slice is periodically available/unavailable, the principles of Solution #24 can be followed so that the associated PDU session can be retained and restored.

-	The UE sends capability that it supports timing information to the network. The network provides timing information to the supported UE.

Option 2: URSP rule is enhanced with indication to request the UE to re-evaluation the URSP rules.

-	For timing restriction of the use of a network slice, URSP rules are enhanced with the requirement for the UE to re-evaluate the URSP rules periodically as per a standardized value or according to a value indicated by the PCF, and for the UE to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. As to enable enforcement in the network, the SMF enforces the UP of PDU Sessions and deactivates the UP according to the timing information that the SMF receives from PCF as part of the SM Policy.

Option 3: reusing existing URSP rule

-  The PCF should generate URSP based on Temporary network slices related information stored in the UDR. UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in URSP and manage the PDU Sessions for the related S-NSSAI accordingly based on the current design as described in TS 23.503 [12], clause 6.6.2.3.

Editor's note: It is FFS which option are adopted for normative work.

For the graceful and gradual termination aspect:

Option 1:

-	For decomission of a network slice, OAM and and NF implementation (e.g. by updating URSP rules and to deregister not used S-NSSAIs) can handle the functionality without further standardization. Whether OAM provides timing information to an NF as to allow the NF to apply a change of resource utilization in advance would be up to SA5.

-  UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in existing URSP and release the PDU Sessions for the related S-NSSAI gracefully based on the current design as described in TS 23.503 [12], clause 6.6.2.3.

Option 2:

-	When a slice is decommissioned, the PDU Sessions of the slice should be gracefully (for supporting UEs of the timing information) and gradually released (for no supporting UEs of the timing information). :

-	If the UE supports the timing information indicating time of network slice availability, the network may provide the timing information to the UEs so the UE knows in advance when a network slice ceases to be supported. In this case, the UE can take the necessary actions to prepare for the slice not becoming available..

Editor's note: the above bullet is FFS and depends on the conclusion of temporary network slices

-	In addition, the AMF, for non-supporting UEs and for the case of UE not performing any actions despite of the timing information provided by the network, may be triggered by the UDM or directly via the OAM to start gradually terminating PDU Session(s) associated with S-NSSAI subject to be terminated. The AMF releases PDU Session(s), associated with the S-NSSAI subject to be terminated, based on operator’s policy available at the AMF.

Editor's note: the above bullet is FFS and depends on the conclusion of temporary network slices

# * * * * End of changes * * * *