3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208821r01

Elbonia, October 10 – 14, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	Conclusion on KI#5

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: this paper is to propose partial conclusion of key issue #5.

1. Introduction/Discussion

2. Text Proposal

Based on the analysis above, it is proposed to capture the following changes vs. TR 23.700-41 040.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.5	Conclusions for Key Issue #5

-	AMF provides information to the UE that the S-NSSAI is rejected due to the S-NSSAI is supported in the current TA but not supported in current AMF. If the UE does not move outside of the RA and wants to use this rejected S-NSSAI for service, it may request this S-NSSAI in registration request, without including the old 5G-S-TMSI or GUAMI in RRC message, so the NG-RAN can reselect a different AMF.

# * * * * End of changes * * * *