SA WG2 Meeting #S2-153E(e-meeting)	S2-2208897

Electronic, October 10 – 17, 2022

Title:	[Draft] reply LS on Multicast MBS session Deactivation and Reactivation

Response to:	S2-2208136/C4-224417

Release:	Rel-17

Work Item:	5MBS

Source:	SA2

To:	CT4

Cc:	RAN3

Contact Person:

Name:	Fenqin Zhu

E-mail Address:	zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.247 CR0134

1. Overall Description:

SA2 thanks CT4 for their LS and would like to answer the questions raised by CT4 as follows:

Q1:	During the MBS session deactivation procedure, does the MB-SMF instruct the MB-UPF to stop the forwarding of DL MBS data towards the N3mb DL F-TEIDs configured in the MB-UPF for the MBS session?

SA2 Response for Q1

Please see the attached CR.

Q2:	If the answer to Q1 is yes, when the multicast traffic resumes (i.e. first packet arrives at MB-UPF), does the forwarding of MBS data towards the N3mb DL F-TEIDs resume only after the MB-SMF instructs the MB-UPF to forward the packets at step 15 of the MBS session activation call flow (Figure 7.2.5.2-1), i.e. after the MB-SMF activates the multicast MBS session towards the AMF(s) (and NG-RAN nodes) and receives a successful response from the AMF (and a first NG-RAN node)?

SA2 Response for Q2

Please see the attached CR.

Q3:	Can SA2 confirm that during an MBS session deactivation procedure, the SMF need not and shall not request the UPF(s) to maintain the N19mb tunnels and that, after an N19m tunnel (of the MBS session) is removed in the UPF, the SMF shall not send a ContextUpdate request message towards the MB-SMF to report the removal of the N19mb tunnel in the UPF?

SA2 Response for Q3

After MBS session deactivation, there are two handling possibilities of the N19mb tunnel used between the UPF and MB-UPF.

- Option 1: the N19mb tunnel is decided to be released by SMF. In this case the SMF need send a ContextUpdate request message towards the MB-SMF to release the tunnel between UPF and MB-UPF. At the next time MBS session activation, the SMF notifies the MB-SMF to re-establish the N19mb tunnel if it is needed.

- Option 2: the N19mb tunnel is decided to be kept by SMF but the delivery of the data to the UE's PDU session is suspended. In this case the SMF notify UPF to drop the received MBS session data. At the next time MBS session activation, the SMF notifies the UPF to forward the MBS session data if it is needed.

Depending on implementation the SMF decides either to keep or release the N19mb tunnels. If the N19mb tunnel (of the MBS session) is decided to be removed from the UPF, the SMF need send a ContextUpdate request message towards the MB-SMF to report the removal of the N19mb tunnel in the UPF.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly requests CT4 to take above information into account.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | Country |
| --- | --- | --- | --- |
| SA2#154 | November, 14-18th | Toulouse | France |
| SA2#154AH | January, 16-20th | Any (e-meeting) | Elbonia |