SA WG2 Meeting #S2-153E(e-meeting)	S2-2208897

Electronic, October 10 – 17, 2022

# Title:	[Draft] reply LS on Multicast MBS session Deactivation and Reactivation

# Response to:	S2-2208136/C4-224417

# Release:	Rel-17

# Work Item:	5MBS

Source:	SA2

To:	CT4

Cc:	RAN3

Contact Person:

#### Name:	Fenqin Zhu

E-mail Address:	zhufenqin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.247 CR0134

1. Overall Description:

SA2 thanks CT4 for their LS. SA2 agreed the attached CR to clarify the points raided by CT4 and would like to answer the questions raised by CT4 as follows:

Q1:	During the MBS session deactivation procedure, does the MB-SMF instruct the MB-UPF to stop the forwarding of DL MBS data towards the N3mb DL F-TEIDs configured in the MB-UPF for the MBS session?

SA2 Response for Q1

If session activation is to be detected by the MB-UPF, the MB-SMF requests the MB-UPF to perform Buffered Downlink Traffic detection and thus to stop forwarding data. If session activation is to be based on an AF request, the MB-SMF also configures the MB-UPF to stop forwarding and possibly to buffer MBS data

Please see the attached CR.

Q2:	If the answer to Q1 is yes, when the multicast traffic resumes (i.e. first packet arrives at MB-UPF), does the forwarding of MBS data towards the N3mb DL F-TEIDs resume only after the MB-SMF instructs the MB-UPF to forward the packets at step 15 of the MBS session activation call flow (Figure 7.2.5.2-1), i.e. after the MB-SMF activates the multicast MBS session towards the AMF(s) (and NG-RAN nodes) and receives a successful response from the AMF (and a first NG-RAN node)?

SA2 Response for Q2

It is up to implementation when to execute this step 15. Applying buffering until the MBS session is activated at the majority of RAN nodes avoids packet loss. However, waiting for responses from all AMFs and SMFs may delay the activation and also does not guarantee that the MBS session is activated at all RAN nodes

Please see the attached CR.

Q3:	Can SA2 confirm that during an MBS session deactivation procedure, the SMF need not and shall not request the UPF(s) to maintain the N19mb tunnels and that, after an N19m tunnel (of the MBS session) is removed in the UPF, the SMF shall not send a ContextUpdate request message towards the MB-SMF to report the removal of the N19mb tunnel in the UPF?

SA2 Response for Q3

During MBS session deactivation, the MB-SMF keeps the tunnels for data forwarding towards UPFs established.

After MBS session deactivation, there are two handling possibilities of the N19mb tunnel used between the UPF and MB-UPF, depending on SMF implementation.

- Option 1: the N19mb tunnel is released by SMF. In this case the SMF sends a ContextUpdate request message towards the MB-SMF to release the tunnel between UPF and MB-UPF. At the next time MBS session activation, the SMF notifies the MB-SMF to re-establish the N19mb tunnel if it is needed.

- Option 2: the N19mb tunnel is kept by SMF but the delivery of the data to the UE's PDU session is suspended. In this case the SMF instructs the UPF to drop the received MBS session data. At the next time MBS session activation, the SMF instructs the  the UPF to forward the MBS session data if it is needed.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly requests CT4 to take above information into account.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | Country |
| --- | --- | --- | --- |
| SA2#154 | November, 14-18th | Toulouse | France |
| SA2#154AH | January, 16-20th | Any (e-meeting) | Elbonia |