3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208904r10

Elbonia, October 10 – 17, 2022

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell, CATT

Title:	KI#1: Further evaluation and conclusion update

Document for:	Discussion/Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract of the contribution: This document provides the further evaluation and conclusion update to KI#1.

# 1. Introduction

For KI#1 it can be categorized as three issues, i.e.

- Assistance information provision (sol#1, 3, 6, 26, 27): how to enable the group member UE receive the multicast MBS data in RRC-inactive state.

- MBS session activation/release (sol#4, 18, 19, 22, 23): inform the multicast group members of the MBS session activation/release for the indicated MBS session.

- Mobility procedure (sol#5/#19/#20/#21/#28):  the mobility procedure for UE in RRC-inactive state multicast MBS data reception.

For the MBS session activation/release and mobility procedure, further progress needs the feedback from RAN side. Due to that, this paper focus on the Assistance information provision issue.

As concluded at S2#152E, there are two level assistance information, i.e. the MBS session and UE session level assistance information.

Explicit/Implicit Parameter

For MBS session level assistance information, there are some discussion on whether the existing MBS session QoS information can be used for that purpose and feedback from RAN side is expected.

For UE level assistance information, one proposal is to reflect the UE level assistance information via the MBS session QoS flow information. If that it means the associated information is not 1:1 mapping from the MBS session QoS information. Hence it adds some complexity at the NG-RAN side, as some UE associated QoS flow information can be mapped from MBS session information, some are not. For those not able to be mapped from the MBS session QoS flow information, it also needs to consider whether they share the same associated QoS flow information or not considering they may be from different PCF/SMF. The mobility to the non-supporting NG-RAN node also need be considered. On the other hand, if the UE level assistance information is a separated explicit parameter as mentioned in other proposal, it is easy for NG-RAN handling. Hence it is proposed to use one explicit parameter to reflect the UE level assistance information.

Proposal 1: The UE level assistance information is reflected as an explicit parameter.

MBS session level assistance information provision from AF to 5GC

For the MBS session level assistance information, it seems the only way to transmit from AF to 5GC is via the MBS session creation procedure.

Assistance information provision from 5GC to NG-RAN node

For the MBS session level assistance information, it is better to be conveyed as other MBS session information parameter, e.g. QoS information. Then it is proposed to be delivered to NG-RAN node as part of the shared delivery tunnel establishment procedure.

For the UE level assistance information, all solution proposes to use the PDU session level signaling to deliver the parameter to the NG-RAN node.

Proposal 4: The MB-SMF provides the MBS session level assistance information to NG-RAN during the shared tunnel establishment procedure.

Proposal 5:  The SMF provides the received UE level assistance information from UDM to NG-RAN node as part of the PDU session information in N2 SM Info and sent to NG-RAN via AMF.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# * * * * Next change * * * *

## 8.1	Key Issue #1:	MBS session reception in RRC Inactive

Editor's note:	Feedback from RAN WGs is required to conclude KI#1 for the issue which nee RAN input.

### 8.1.1	Interim conclusions

The following conclusions is proposed for KI#1:

-	Depending on RAN WG feedback, it may be possible to keep some UEs within the same MBS session in RRC_CONNECTED and some in RRC_INACTIVE state. NG-RAN nodes take the responsibility to determine (e.g. during congestion) which UE(s) within an MBS multicast session will be moved from CM-CONNECTED with RRC CONNECTED to CM-CONNECTED with RRC Inactive state and still receive MBS session data.

-	The 5GC provides information about the MBS session as specified for Rel-17 and may provide additional assistance information to help NG-RAN to determine whether to apply delivery enabling reception by UEs in RRC_Inactive state for an MBS session and which UE(s) to be moved to RRC Inactive state.

-	Depending on RAN WG feedback, the assistance information may include recommendations whether to enable delivery for reception in RRC_Inactive state for an MBS session and information about UEs that should preferably be kept in RRC_Connected state, i.e. the MBS session level and UE level assistance information, and may be provided by the AF to 5GC and then to NG-RAN.

Editor's note:	What kind of assistance information is needed in the RAN will be confirmed by RAN WGs.

- NG-RAN use the MBS session level and UE level assistance information as help for the decisions on whether to enable delivery for reception in RRC_INACTIVE state for an MBS session and/or on which UEs to keep in RRC_CONNECTED or. RRC_INACTIVE state. How NG-RAN performs those decisions is up to NG-RAN implementation.

NOTE 1:	How the NG-RAN handles the situation without assistance information for RRC Inactive multicast MBS data reception is to be determined by RAN WGs.

NOTE 2: What is defined in clause 5.3.3.2.5 of TS 23.501 for "RRC Inactive Assistance Information" is sent by AMF to NG-RAN and may be used by NG-RAN together with any other MBS session level assistance information and UE level assistance information for deciding whether to send a UE to RRC Inactive state.

-	When the MBS session is activated, the UEs in RRC Inactive state in cells, where the MBS session is delivered allowing RRC-inactive reception, should be able to remain in RRC Inactive state for receiving the MBS session data.

-	For group paging, the network notifies which MBS session is to be activated.

-	How NG-RAN notifies the UE that the MBS session is re-activated and whether the MBS session is allowed to be received in RRC-inactive state will be decided by RAN WGs.

-	When the UE moves outside the RNA, the UE performs UE Triggered Connection Resume in RRC Inactive procedure to the target RAN node as per existing procedures in TS 23.502 [3].

-	When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but Connection resume fails, it follows existing procedures and transition to CM-IDLE. When the UE transition to CM-IDLE since is not able to receive the MBS multicast data at the new cell, the UE initiates the mobility registration update or Service Request procedure and activates the associated PDU session, so that the shared tunnel (if not already established) or the individual delivery can be established towards the NG-RAN node for multicast data delivery to the UE.

-	When the UE moves outside the RA, the UE performs mobility registration procedure and as per the existing procedure in TS 23.501 [2] clause 5.3.3.2.5 for "Mobile initiated NAS signalling procedure" for UEs in CM-CONNECTED with RRC Inactive state, the UE will resume the RRC Connection.

-	During the handover procedure, the SMF includes the "MBS assistance information for RRC Inactive" if any in N2 SM Info and sent to NG-RAN via AMF.

The following requirements, which need be supported by RAN WG, are concluded:

-	Backward compatibility with Rel-17 UEs not supporting the RRC_Inactive reception of MBS multicast data needs to be ensured

-	RAN WG2 define UE radio capability for MBS reception in RRC_INACTIVE state.

-	NG-RAN nodes decide for which MBS sessions to apply delivery enabling reception in RRC_Inactive state. The NG RAN nodes handling RRC-Connected UE in an MBS multicast session also decides whether the UEs can transition to RRC_Inactive state and may consider assistance information from the 5G core network for that decision. How NG-RAN performs those decisions is up to NG-RAN implementation.

-	When the MBS session is activated, the UE in RRC Inactive state in cells where the MBS session is delivered in the delivery mode for RRC-inactive reception should be able to remain in RRC Inactive state for receiving the MBS session data.

-	When the UE is in RRC Inactive state and moves within the RNA, it shall be able to continue receiving DL multicast MBS data unless it leaves the MBS service area.

# * * * * End of change * * * *