3GPP TSG-SA2 Meeting #153E(e-meeting)	S2-2208907

# Electronic, October 10 – 17, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3737 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UPEAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in conclusion of FS_UPEAS study, it is proposed to add the UPEAS related description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UPEAS related general description and UPF selection is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPEAS is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.X(new), 6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Management |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.8.2.x	Data exposure via SBI interface

The UPF may expose information described in TS 23.502 [2] clause 5.2.26.2, via a service-based interface directly. The NF consumer, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF.

When the UPF support the data exposure via the SBI interface, it may register its NF profile to the NRF. The NF profile include the supported event exposure service and related event ID(s).

To get exposure data from UPF, NF consumer do the subscription to the UPF directly or indirectly via SMF. UPF selection is further described in clause 6.3.3.1.

Only NWDAF can subscribe to the UPF event exposure service directly and only for the following case:

- Data collected for UPF load analytics.

- Data collected for analytics targeting "any UE", but not related with an AoI or with a specific data flow.

To minimize the impact of event notification to UPF data processing, the event subscription may includs Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. When the related event is detected, the Reporting window defines the last reporting valid time. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

# * * * * Next change * * * *

### 6.3.3	User Plane Function Selection

#### 6.3.3.1	Overview

The selection and reselection of the UPF are performed by the SMF by considering UPF deployment scenarios such as centrally located UPF and distributed UPF located close to or at the Access Network site. The selection of the UPF shall also enable deployment of UPF with different capabilities, e.g. UPFs supporting no or a subset of optional functionalities.

For home routed roaming case, the UPF(s) in home PLMN is selected by SMF(s) in HPLMN, and the UPF(s) in the VPLMN is selected by SMF(s) in VPLMN. The exact set of parameters used for the selection mechanism is deployment specific and controlled by the operator configuration.

The UPF selection involves:

-	a step of SMF Provisioning of available UPF(s). This step may take place while there is no PDU Session to establish and may be followed by N4 Node Level procedures defined in clause 4.4.3 of TS 23.502 [3] where the UPF and the SMF may exchange information such as the support of optional functionalities and capabilities.

-	A step of selection of an UPF for a particular PDU Session; it is followed by N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3].

To collect the data from the UPF as defined in clause 5.8.2.X, the related UPF is discovered and selected as following:

-	When the NF consumer may directly subscribe to the UPF, the NF consumer queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request.

-	When the NF consumer shall subscribe via the SMF, the NF consumer gets the serving SMF information from the UDM per SUPI, or IP address and DNN+ S-NSSAI. After that, the NF consumer sends a subscription to the indicated SMF and the SMF to identities the related UPF(s) using the parameters of the subscription (e.g. target flow description, AoI, ) and transfers the related event subscription information to the idenbtified UPF(s).

# * * * * End of change * * * *