3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208914r10

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Evaluation and Conclusion Update

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: Update of KI#3 evaluation and conclusion.

# 1. Introduction/Discussion

This paper proposes to update the evaluation and conclusion for KI#3 with 2 changes.

First, the update to solution 17 (S2-2208913) clarifies that the path switching policy in both UEs can be used to determine which ProSe services are switched based on UE negotiation. This updates the evaluation of Sol#17 and further updates the conclusion part to include that the UEs negotiate to determine which ProSe services are switched based on the path switching policy.

Second, the following EN is removed and further updates to the conclusion.

Editor's note:	Whether or not to include the optional behaviour for the UEs to inform each other of their potential to establish a Uu link in the conclusion will be determined during SA2#153E.

It is strange that the UE know that the status of its peer UE’s PDU session for the specific service on the Uu path.

The conclusion part is changed based on each UE may inform the peer UE over existing PC5 connection which service can be transmitted on Uu path from its side, e.g., based on whether it can initiate PDU session establishment for the ProSe service or the path switching policy. Then if the peer UE already knows that the specific service cannot transmitted on Uu path, then the service does not need to be negotiated again when the path switch is triggered.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 7.3	Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e. non-relay case)

For Key Issue #3: "Support direct communication path switching between PC5 and Uu (i.e. non-relay case)", based on Table 6.0-1, the solutions can be summarized and evaluated as the following:

-	Sol#16 proposes provisioning policy based direct communication path switching between PC5 and Uu. Path switching policy is provided from PCF to the UE or (pre-)configured in the UE. The path switching policy indicates which path(s) is allowed for all or specific ProSe services (i.e. direct PC5 allowed, direct Uu allowed or not allowed indicated). Based on the path switching policy, a UE may establish a PDU session or a PC5 connection in the target path and switch the traffic from the source path to the target path.

-	Sol#17 proposes to use the make-before-break mechanism to reduce interruption when path switch between PC5 and Uu paths. The path switch Policy indicates whether the specific service is allowed to switch from Uu to PC5 or from PC5 to Uu. The two UEs negotiate to determine which ProSe services are switched based on the path switching policy. For path switching from PC5 path to Uu path, the two UEs may negotiate their Uu QoS based on the PC5 QoS in order to ensure consistent service experience, and may share the IP addresses used for the Uu path with each other to achieve the switch of service transmission.

-	Sol#18 uses the principle of "make before break". Before performing a path switch, the two UEs which are connected via PC5 negotiate the triggers for path switching and what service or QoS flows need to be switched. Once the negotiated triggers are satisfied, the two UEs perform the path switching between PC5 to Uu directly.

-	Sol#19 proposes to use a path permission policy, which can be (pre-)configured in the UE or provided by the PCF. The path permission policy includes the mapping of ProSe Services to the following path permission:PC5 permitted, or Uu permitted, or both PC5 and Uu permitted.

-	Sol#20 proposes to support the switching back to PC5 path from Uu path by maintaining the unicast PC5 link after switching to the Uu path and reusing the unicast PC5 link when switching back to the PC5 path. After switching from the PC5 path to Uu path it is UE implementation to keep or release the PC5 link.

-	Sol#21 proposes that the existing path selection policy, a modified path selection policy or a new path switching policy is used to control path switch between Uu and PC5. During the path switch from PC5 to Uu, PC5 signalling includes the suggested QoS rules and QoS Flow level QoS parameters for Uu path.

-	Sol#22 proposes that for path switching from PC5 to Uu, the IP addresses associated with the UE's PDU sessions are exchanged along with the PDU Session status. PDU session status piggybacked at Keep Alive procedure is used so that each UE may decide the proper time to initiate path switch to Uu with peer UE.

# * * * * Second change * * * *

## 8.3	Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e. non-relay case)

For Key Issue #3 (Support direct communication path switching between PC5 and Uu (i.e. non-relay case)), the followings are taken as conclusion:

-	A Path switching policy is used that indicates which path(s) is permitted for all or specific ProSe services (i.e. PC5 permitted, or Uu permitted, or both PC5 and Uu permitted).

NOTE 1:	Whether the path switching policy can be combined into the path selection policy as defined in TS 23.304 [3] will be determined in normative phase.

-	The path switch policy can be (pre)configured in the UE or provided by the PCF.

-	The granularity of path switch is per ProSe service.

-	UEs negotiate over the existing PC5 connection to determine whether and which ProSe services are to be switched taking into account e.g. their path switching policy, availability of Uu path etc., by the Source UE requesting switch for specific services and the Target UE confirming which services can be switched or not. Availability of Uu path for ProSe services can be share to the other UE while using existing PC5 connection (before path switch negotiation)

-	For the path switching from the a PC5 path to a Uu path, one of the UEs may notify the peer UE of being not able to perform the path switching (e.g., under congestion control or mobility restriction) or having no willing to perform the path switching to avoid the peer UE performing the path switching.

-	For the path switching from a Uu path to a PC5 path, in order to discover each other during PC5 discovery or during Layer-2 link establishment procedure, user info may be provided by the application layer before switching to PC5 path.

-	A make-before-break mechanism is used when path switching between PC5 and Uu, i.e., when path switching occurs the UEs prepare the target path to communicate with each other, then performing the path switching from the PC5 path to Uu path and vice versa and communicate over that target path, and at which point for former path may be released.

NOTE 2:	If UEs maintain a PC5 unicast link after path switching from a PC5 path to a Uu path, the UEs can determine to switch back to the PC5 path from the Uu path based on e.g. the policy related to path switching, the PC5 signal level of the maintained unicast link.

-	For the path switching from the PC5 path to Uu path, the Uu QoS each UE will request from their network using the existing 5GS mechanism is based on PC5 QoS and negotiated via the existing PC5 connection between two UEs.

-	For the path switching from the PC5 path to Uu path, optionally the IP addresses used for Uu path can be shared between the UEs as assist them to establish communication directly with each other via the Uu path.

# * * * * End of changes * * * *