3GPP TSG-WG SA2 Meeting #153e	S2-2208997r01

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #4, evaluation and conclusion update

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: This contribution proposes to update evaluation and conclusion on Key Issue #4.

1. Discussion

Different models for mapping between the PDU Session and the PIN or PINE are assumed in various solutions in this study. It is important to discuss the PIN Session Models, i.e. how to map the PDU Session with the PIN or PIN Element.

Based on these models, the behaviour of PEGC such as when to trigger the PDU Session Establishment/Modification is impacted, and the authorization and QoS control of the PDU Session related with PIN Session model is impacted.

2. Proposal

It is proposed to update the clause 7.4 Evaluation on Key Issue #4 (1st change) and the clause 8.4 conclusion on Key Issue #4(2nd change) of TR 23.700-88.

# * * * * 1st change * * * *

7.4	Evaluation on Key Issue #4

7.4.1	General

Following mechanisms for device to network relay path management have been indicated:

a)	One PDU Session of the PEGC serves multiple PINEs (solution #5, #11, #12). The PDU Session may be dedicated to relay traffic of PINE in the PIN(s) (as in Annex X.2). Alternatively, other applications in the UE hosting PEGC may share the PDU Session to relay traffic for the PIN(s) (Annex X.1).

b)	One PDU Session of the PEGC serves one PINE (solution #13). It is also illustrated in Annex X.3.

In the case of one PDU Session serves multiple PINEs, following mechanisms for SMF selection that serving the PDU Session of a PEGC for relay have been indicated:

a)	During the PDU Session Establishment procedure, an enhanced SMF supporting PIN is selected (solution #5 implies).

b)	When initial device to network relay is required from a PEGC, a PIN AF or PIN NF sends DNAI to the PCF to trigger the SMF selection for a PDU Session of the PEGC (solution #12).

In the case of one PDU Session serves multiple PINEs, following mechanisms for QoS and policy differentiation have been indicated:

a)	During UE (PEGC) initiated PDU Session Modification procedure with PINE information, which is triggered by PINE, the PCF queries information for the PINE from the UDR and devices policy for the PINE. The QoS and policy information of the PINE is provided to the PEGC (solution #5).

b)	During network initiated PDU Session Modification procedure for a PEGC, which is triggered by a PIN AF or PIN NF, the QoS and policy information related to PINEs associated with the PEGC is provided to the PEGC. The PCF is able to provide QoS and policy information related to the PINEs (solution #11, #12).

c)	The PCF obtains the QoS and policy requirements for a PINE from UDR (solution #5).

d)	The PCF obtains the QoS and policy requirements from a PIN AF or PIN NF, which may be via the SMF serving the PDU Session of the PEGC (solution #11, #12).

In the case of one PDU Session serves one PINE, following mechanisms for QoS and policy differentiation have been indicated:

a)	The PEGC maps between DSCP over direct wireless connection and 5QI over 5GS access (solution #13).

The following mechanisms for install routing information related to communication for device to device via 5GC and PIN status information for DL data notification for a PINE have been indicated:

a)	Routing information and PIN status information is provided by SMF to UPF for routing traffic of PINEs between PDU Sessions of PEGCs for a PIN (solution #12).

NOTE:	Some combination of the above principles is not possible or not able to be alternative.

There are 9 solutions (sol#5, sol#11, sol#12, sol#13, sol#16, sol#17, sol#18, sol#19, and sol #20) addressing this KI. The mains aspects include the communications between PINE and services outside of the PIN via PEGC and 5GS.

The evaluation of KI#4 is based on the following classification of solution's characteristics:

a)	So1#13 and sol#16 define QoS differentiation for PINE traffic relayed by PEGC and 5GS.

b)	Sol#17 and sol#20 define PIN routing in presence of multiple PEGCs.

c)	Sol#5&11&18, and sol#19 define QoS information for non-3GPP access.

d)	Sol#12 proposes a solution for communication of PIN.

# * * * * 2cn change * * * *

Annex X.	PIN Session Models

In this clause, we describe the PIN Session models to explain how the PDU Session of PEGC is mapped to the PIN or PIN Elements.

X.1 Shared PIN Session Model

With this model, the PIN Element with Gateway Capability (UE) relays traffic from multiple PIN Elements connected via multiple PIN networks to the Data Network through the PDU Session of the UE as depicted in the Figure X-1.

Figure X.1-1: Share PIN Session Model

With this model, Applications of the UE also can use the same PDU Session that is used for relaying the PIN traffic.

Activation or deactivation of the PIN of PEGC does not requires the PDU Session Establishment/Modification procedures. The QoS control on PIN may be limited without the PEGC engagement.

X.2 Shared PIN Session Model

With this model, the PIN Element with Gateway Capability (UE) relays traffic from multiple PIN Elements connected with one PIN network to the Data Network through one PDU Session of the UE as depicted in the Figure X-2.

Figure X.2-1: Dedicated PIN Session Model

With this model, one PDU Session is mapped to one PIN network and other applications of the UE do not share the same PDU Session.

Activation or deactivation of the PIN of PEGC can involves the PDU Session Establishment/Modification/Release procedure.

5GC can enforce PIN level QoS by applying the QoS for the PDU Session since the PIN and the PDU Session has one-to-one relationship. For example, if the operator apply the PIN level AMBR, it can enforce PIN-AMBR by enforcing PDU Session AMBR through the existing policy framework.

X.3 Dedicated PIN Element Session Model

With this model, the PIN Element with Gateway Capability (UE) relays traffic from one PIN Elements connected with PIN to the Data Network through one PDU Session of the UE as depicted in the Figure X-3.

Figure X.3-1: Dedicated PIN Element Session Model

With this model, one PDU Session is mapped to one PIN Element and other applications of the UE do not share the same PDU Session.

Association or disconnection of the PIN Element connected to the PEGC via PIN can trigger the PDU Session Establishment/Modification/Release procedure.

5GC can allow or disallow the PIN Element to connect to the Data Network by authorizing the PDU Session during the PDU Session Establishment procedure. 5GC is able to map the interested PIN Element such as XR device to a specific network slice.

# * * * * End of changes * * * *