3GPP TSG-WG SA2 Meeting #153e	S2-2208997r03

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #4, evaluation and conclusion update

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: This contribution proposes to update evaluation and conclusion on Key Issue #4.

# 1. Discussion

Different models for mapping between the PDU Session and the PIN or PINE are assumed in various solutions in this study. It is important to discuss the PIN Session Models, i.e. how to map the PDU Session with the PIN or PIN Element.

Based on these models, the behaviour of PEGC such as when to trigger the PDU Session Establishment/Modification is impacted, and the authorization and QoS control of the PDU Session related with PIN Session model is impacted.

# 2. Proposal

It is proposed to update the clause 7.4 Evaluation on Key Issue #4 (1st change) and the clause 8.4 conclusion on Key Issue #4(2nd change) of TR 23.700-88.

# * * * * 1st change * * * *

# * * * * 2cn change * * * *

# Annex X.	PIN Session Models

## X.1 General

In this Annex, we describe the PIN Session models with the assumption that one PDU Session serves one PIN.

5GC can enforce PIN level QoS by applying the QoS for the PDU Session since the PIN and the PDU Session has one-to-one relationship. For example, if the operator applies the PIN level AMBR, it can enforce PIN-AMBR by enforcing PDU Session AMBR through the existing policy framework.

## X.2 Shared PIN Session Model

With this model, the applications of the PEGC also can use the same PDU Session that is used for relaying the PIN traffic as depicted in the Figure X-1.

Figure X.2-1: Share PIN Session Model

Activation or deactivation of the PIN of PEGC does not requires the PDU Session Establishment/Modification procedures. The QoS control on PIN may be limited without the PEGC engagement.

## X.3 Dedicated PIN Session Model

With this model, the applications of the UE do not share the same PDU Session that is used for relaying the PIN traffic as depicted in the Figure X-2.

Figure X.231: Dedicated PIN Session Model

With this model, one PDU Session is mapped to one PIN network and other applications of the UE do not share the same PDU Session.

Activation or deactivation of the PIN of PEGC can involves the PDU Session Establishment/Modification/Release procedure.

# * * * * End of changes * * * *