SA WG2 Meeting #153E (e-meeting)	S2-2209000r06

October 10 – 14, 2022, Elbonia	(revision of S2-220xxxx)

Source:	NEC, LGE, Lenovo, Nokia,

Title:	KI#2: Conclusions

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This document captures conclusions for this key issue#2 based on the inputs on basic procedure from various companies.

# Conclusion document for KI#2

## 1. Key Issue #2 Background

## 5.2 Key Issue #2: Support of providing VPLMN network slice information to a roaming UE ### 5.2.1 Description As an outcome of SA WG1 EASNS (Enhanced Access to and Support of Network Slice) work, clause 6.1.2.1 of TS 22.261 [4] captures the following service requirement for a roaming UE. For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice. This key issue aims at addressing the following aspects for a roaming UE requiring a network slice not offered by higher priority VPLMN(s) but available from other network(s): - Study how and when the HPLMN provides the UE with information about slice availability per VPLMN and prioritization information of the VPLMNs with which the UE may register for the network slice. The study includes the content of the information. - Study how and when to use the information received by the UE from the HPLMN to influence automatic PLMN selection. NOTE 1: For details on PLMN selection aspects and impacts on PLMN selection of solutions of this key issue, coordination with CT1 is needed. NOTE 2: Impacts to manual selection is not in scope of the key issue. This key issue only considers the network selection procedure for the 3GPP access type. NOTE 3: Coordination with the Study Item FS_5WWC_Ph2 is required regarding the VPLMN selection procedure when non-3GPP access type is used.

Solutions; Key Issues. KI#1; KI#2; KI#3; KI#4; KI#5; KI#6. Solution #6: Extended SoR VPLMN Slice Information transfer to UEs; X. Solution #7: Enabling awareness of Network Slice availability in VPLMNs; X. Solution #16: UE assisted slice based VPLMN prioritization for Extended SoR; X. Solution #17: Slice based VPLMN Selection Policy; X. Solution #18: Sending rejected NSSAI to the UDM to assist the UDM to steer the UE to the PLMN supporting rejected NSSAI; X. Solution #19: configuring the UE with network slice aware preferred PLMNs lists; X. Solution #20: VPLMN Selection following existing SoR information; X.

Note: This various aspects of the slice based SoR procedure has been discussed among the companies and the discussion is captured in the S2-2209001

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41

# * * * * First change * * * *

## 8.X	Conclusions for KI#2

## The following principles are concluded for KI#2.

1.	A slice based SoR mechanism to deliver enhanced slice-aware SoR information will reuse the current SoR mechanism defined in TS 23.122 [7] for SoR information delivery.  The format of the enhanced slice-aware SoR information is in the CT1 remit.

2.	The SoR container (including the enhanced slice-aware SoR information) from the UDM to the UE is security protected.

NOTE 1: SA3 will further define any upgrade of security protection mechanism if needed.

3.	UDM requires UE capability to deliver enhanced slice-aware SoR information.

NOTE 2: How the UE provides the UE capability needs to be discussed in CT1.

NOTE 3: Whether the UE provides additional assistance information (for additional assistance information refer TR 23700-41) in addition to the UE capability needs to be discussed in CT1. Any UE additional assistance information is transparently forwarded by UDM to SoR-AF during the triggering procedure by UDM. The SoR-AF should not attempt to fetch any assistance information if not provided by the UE.

4.	Only a UE supporting slice based SoR feature can receive the enhanced slice-aware SoR information , which includes available slice information per VPLMN.

5.	The UE will perform the PLMN selection based on the received slice-aware SoR information.

6.	How the slice-aware SoR information is delivered is in CT1 remit. It shall be possible for the HPLMN to update the slice availability per VPLMN and prioritization information of the VPLMNs when there is it is required by HPLMN e.g. change in the UE subscription or other HPLMN trigger.

Editor's note:	It is FFS how the SoR AF gets Subscribed NSSAI from the UDM, i.e. by using push or pull method.