3GPP SA WG2 Meeting #153E	S2-2209062r04

e-meeting, October 10 – 17, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#4: Evaluation and Conclusion for ADRF/NWDAF data management

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_PH3/Rel-18

Abstract of the contribution: This paper proposes update to the evaluation and conclusion for KI#4, Category-4 in TR 23.700-81.r01 removes the “evaluation” part, as it is merged into S2-2208870

Discussion

23.700-81 clause 7.4 evaluates in “category-4”, enhancements for ADRF/NWDAF data management. This contribution adds additional insights to the evaluation and proposes an amended conclusion aligned with the evaluation. The additional insights include:

- Although data are stored in the ADRF/NWDF, the DCCF executes tasks to update the NWDAF or ADRF storage according to the desired storage approach, off-loading those tasks from the NWDAF / ADRF.

- On the issue of supporting both Consumer provided Storage Handling Information and NF (DCCF/NWDF/ADRF) specific Storage Policy:

  - Storage Handling Information provided by a consumer provides the operator a simple way to specify per-consumer policy for storage of data. For example, ADRF storage of analytics for an OA&M consumer may be longer than for an NF consumer that requires analytics only for transient 5GS optimization. A shorter time specified by a consumer saves storage resources compared to a static, consumer independent operator policy.

  - The ADRF may be provided via cloud services administered separately from other 5GS NFs. Administration of ADRF storage policy (e.g. a one year storage retention period configured by a cloud provider) may be different from the storage requirements for various operator analytics consumers. In this case a Storage Approach specified by the consumer NF is necessary so ADRF resources are not wasted.

  - Storage requirements for the same analytics may be use case dependent. The same analytics/data consumer (e.g. NF) may have longer or shorter analytics storage requirements depending on how the analytics/data are used by the consumer. The ADRF/NWDAF/DCCF has no insight into the use case supported by the analytics, and hence can not alter storage duration without further information (Storage handling information) from the consumer.

Proposal

It is proposed to add the following to TR 23.700-81.

*** Start of changes ***

8.4	Key Issue #4: How to Enhance Data collection and Storage

1.	For Managing Impact of storing data in NFp during muting, Solutions #41 and #45 are adopted for normative work.

2.	For ADRF / NWDAF Data Storage Management, a Consumer may request data deletion alert from  ADRF. NWDAF or ADRF may be configured with operator policies for data storage as defined in Solution#46.

3.	For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy. ADRF shall not duplicate the functionality provided by MLModelProvision Service.

Editor's note:	Detailed procedures, service operations and corresponding parameters on how MTLF stores the ML model in ADRF are FFS.

4.	Data records can be stored at ADRF with an associated DataSetTag. The DataSetTag can be used to retrieve the whole set of data records associated to such tag. The DataSetTag may be included in the service requests from AnLF to MTLF to indicate the data set  used by the AnLF for inference, so that MTLF can use the data for further training of ML models..

5.	Data can be stored at and retrieved from ADRF using specific data compression technique..

NOTE X:	How data consumer and ADRF share the data compression info is up to Stage-3 design. For example, data compression method can be conveyed in the existing HTTP2 header.

6. To optimize the reporting of network data, data can be reported according to a non-fixed sampling ratio. Data consumer can update the data sampling rate dynamically by updating the data subscription.

7.	Normative change (only impacting stage 2) is recommended to allow NWDAF to register in the UDM for the served UE, also for Analytics IDs that are not UE-related, as defined in Solution #65.

*** End of changes ***