SA WG2 Meeting #153e	S2-2209092r01

# 10 - 17 October 2022, Electronic meeting        		            (revision of S2-220xxxx)

Source: 	Lenovo, Motorola Mobility, Apple, Broadcom, Tencent

Title: 	Evaluation for KI#2

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# 1.	Proposal

It is proposed to add the following changes to TR 23.700-53.

Start of Changes

## 7.1	Evaluation for KI #2: New steering functionalities for non-TCP traffic

The main classes of solutions related to KI#2 are DCCP-based (#2.1) and QUIC-based (#2.2 and #2.3). Additionally, solution #2.4 complements some of those main solutions with methods to reduce header overhead.

Both classes of solutions support a “Low-Layer” mode, where the payload of the MA-PDU session is an IP packet or an Ethernet frame. The QUIC-based class of solutions also supports “UDP proxying”, where the payload of the MA-PDU session is a UDP payload (#2.2). Solution #2.4 details an aspect of “Low-Layer” solutions, which enables bringing their user plane performance up to par with the “UDP proxying” solution.

### 7.1.1	User Plane Performance Aspect

While the main solutions differ from per-packet overhead standpoint, solution #2.4 describes how inner header compression can be used in the tunnel between UE and UPF to limit overhead in some cases (#2.1 and #2.3). The estimated per-packet overhead of all 3 solutions (#2.1, #2.2, and #2.3) becomes equivalent to each other if header compression is used with #2.1 and #2.3.

Solution #2.2 does not require header compression to achieve its low per-packet overhead. RoHC (or EHC) can be implemented with Solutions #2.1 and #2.3 to perform inner header compression in MA-PDU session endpoints (UE and UPF).

### 7.1.2	Evaluation of steering functionalities

For KI#2, three solutions have been proposed:

- Solution #2.1 (aka MP-DCCP-LL);

- Solution #2.2 (aka MPQUIC-UDP); and

- Solution #2.3 (aka MPQUIC-IP).

Solution #2.1 is based on the DCCP protocol and its multipath extension (draft-ietf-tsvwg-multipath-dccp). This solution supports the transport of UDP flows, IP flows and even Ethernet flows, over DCCP.

In contrast, Solutions #2.2 and #2.3 are both based on the QUIC protocol, its multipath extensions (draft-ietf-quic-multipath) and additional extensions defined in IETF for supporting "UDP proxying over HTTP" and "IP proxying over HTTP" respectively (see RFC 9298 and draft-ietf-masque-connect-ip). These solutions support the transport of UDP flows and IP flows respectively, encapsulated in HTTP/3.

Solutions #2.2 supports UDP traffic only.

Solutions #2.2 supports IP and UDP traffic.

Solution #2.1 support IP, UDP and Ethernet traffic

Note: Ethernet traffic support was not agreed to be in scope of the key issue but is requested functionality by BBF.

#### 7.1.2.1 Co-existence with MPTCP and ATSSS-LL

All three solutions #2.1, #2.2 and #2.3 are in principle capable to co-exist with MPTCP and ATSSS-LL and can be chosen using the solutions defined N4 and ATSSS rules.

Solutions #2.1 does not necessarily need any additional steering functionality as it can handle all kind of traffic.

#### 7.1.2.2 Evaluation of the impact of User-plane overhead

##### 7.1.2.2.1 Header Size Considerations summary

Table 1 shows header sizes introduced by MP-QUIC based solution#2.2 and solution#2.3 for carrying either UDP or IP using Datagram mode 1, Datagram mode 2 or Stream mode, and the MP-DCCP solution.

| Solution | Add. Header size [Bytes] | Comment |
| --- | --- | --- |
| MP-QUIC UDP Datagram mode 1 | 46 – 67 average 55 | - Datagram Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. They determine the range, which is only under optimal, non-expectable, conditions 46 Byte. More likely is an average value of 55 Byte. - No overhead considered for path latency information exchange to be used in the reordering process. QUIC and extension does not provide this feature. - Impact of header/payload ratio due to typical lower MTUs for QUIC not considered. |
| MP-QUIC IP Datagram mode 1 | 54 - 75 average 63 | - Datagram Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. They determine the range, which is only under optimal, non-expectable, conditions 54 Byte. More likely is an average value of 63 Byte. - No overhead considered for path latency information exchange to be used in the reordering process. QUIC and extension does not provide this feature. - Impact of header/payload ratio due to typical lower MTUs for QUIC not considered. - Further gain through inner IP header compression possible |
| MP-QUIC UDP Datagram mode 2 | 38-52 average 46 | As Datagram mode 1 but without Sequence number |
| MP-QUIC IP Datagram mode 2 | 46-60 average 54 | As Datagram mode 1 but without Sequence number Further gain through inner IP header compression possible |
| MP-QUIC UDP Stream mode | 41-68 average 56 | As Datagram mode 1 but with more variable length encoded fields. An average value of 56 Byte overhead is assumed to be likely. |
| MP-QUIC IP Stream mode | 49-76 average 64 | As Datagram mode 1 but with more variable length encoded fields. An average value of 64 Byte overhead is assumed to be likely. Further gain through inner IP header compression possible |
| MP-DCCP-LL | 55-65 | Depending on short/long DCCP header and the type of delay information exchange for reordering. 55 Byte for short header and DCCP timestamp option, 65 Byte for long header and MP_RTT option. There is no reason to select the high overhead constellation. - Further gain through inner IP/UDP or IP/TCP header compression possible |

Table 1: MP-QUIC solutions and MP-DCCP-LL overhead introduced by extra network protocol headers.

Basically, it can be concluded that MP-QUIC proxying mode has no significant difference over MP-DCCP encapsulation. Especially if it is considered that MP-QUIC at the moment does not provide path latency information exchange for the purpose of reordering and path latency difference compensation. Adding such functionality to MPQUIC would increase its overhead by at least 6 Bytes.

Without further considerations, MP-QUIC solution for IP is already equal to MP-DCCP-LL and would be worse when latency information is handled in addition.

In case MP-DCCP is combined with inner header compression, this further increases the advantage with regard to MP-QUIC solutions.

While Datagram mode 2 resembles ATSSS-LL functionality it introduces at least 38/46 Byte (UDP/IP) overhead. As this are minimum values, more likely is an average overhead 46/54 Byte (UDP/IP) due to variable length fields.

Stream mode introduces at least 41/49 Byte (UDP/IP) but more likely 56/64 Byte (UDP/IP). In that it is close to Datagram mode 1 header sizes.

While QUIC can multiplex PDUs in one packet, QUIC has limitations in regard to MTU size as no fragmentation is allowed per specification. This significantly reduces the payload transferable within one datagram, that is why those absolute header sizes do not give the full picture but more important the relation to the MTU needs to be taken into account. QUIC is typically limited to 1200 Bytes (https://blog.cloudflare.com/unlocking-quic-proxying-potential/, https://datatracker.ietf.org/meeting/114/materials/slides-114-maprg-a-quick-look-at-quic-geoff-huston-00), while for  MP-DCCP there are no such limitations. Additional header sizes introduced by the solutions have therefore a bigger impact to efficiency when lower MTU applies as less payload can be carried.

##### 7.1.2.2.2 Detailed analysis on header sizes

MP-DCCP related information can be found in two documents - MP-DCCP draft [ ] and RFC4340 (DCCP) [].

MP-QUIC solutions requires multiple documents to investigate:

QUIC RFC9000 [6], MP-QUIC draft-ietf-quic-multipath [10], QUIC Datagram RFC 9221[9], Connect-UDP RFC 9298 [27], Connect-IP draft-ietf-masque-connect-ip [31], HTTP/3-Datagram RFC 9297 [29]

Basically, the following list gives an impression how the individual solutions are composed of. It should be noted that the multipath extension of QUIC does not leave a header footprint.

- MP-QUIC UDP Datagram mode:

UDP + QUIC + MP extension + QUIC DATAGRAM + HTTP Datagram

- MP-QUIC IP Datagram mode:

UDP + QUIC + MP extension + QUIC DATAGRAM + HTTP Datagram + UDP

- MP-QUIC UDP Stream mode:

UDP + QUIC + MP extension + STREAM Frame + HTTP Datagram

- MP-QUIC IP Stream mode:

UDP + QUIC + MP extension + STREAM Frame + HTTP Datagram + UDP

- MP-DCCP-LL:     DCCP + MP extension + IP + UDP

For the MP-QUIC solution multiple nested documents needs to be skimmed and solution specific parameters needs to be considered. Below is the extra size added by the MP-QUIC UDP solution.

UDP QUIC Short    DATAGRAM         HTTP Datagram      HTTP Info

1-RTT Packet, Type, [Length],  Quarter Stream ID, Context ID, Seq No.,

8   26            1     {1,2,4,8}  {1,2,4,8}          {1,2,4,8},  8?       = 46/67 Bytes

QUIC Short: RFC9000

DATAGRAM: RFC9221

HTTP DATAGRAM: RFC9297

HTTP Info Context ID: RFC9298

HTTP Info Seq. No.: Solution specific

Datagram Length parameter is required to support multiplexing capability

Stream ID is required for differentiating QoS flows and steering mode within one MP-QUIC connection

MP extension has no impact to the header size as it only impacts the meaning of the RFC9000 defined packet number.

Seq. No. is reasonably assumed to be 8 Byte, however it is not specified so far.

Length, Quarter Stream ID and Context ID are variable length encoded fields varying between 1, 2, 4, or 8 Bytes. In the case of Datagram mode 2, Context ID is a 1 Byte value.

In case MP-QUIC Stream mode is applied, the HTTP Datagram is capsuled in a Datagram Capsule element which itself is part of a STREAM Frame. Compared to Datagram modes, HTTP Info field is not required as ContextID and Seq. No. are not required.

UDP QUIC Short    STREAM Frame                           Datagram Capsule  HTTP Datagram

1-RTT Packet, Type, Stream ID, [Offset] , [Length] ,  Type, Length,    Quarter Stream ID

8   26            1     {1,2,4,8}  {1,2,4,8}  {1,2,4,8}   1     {1,2,4,8}  {1,2,4,8} = 41/76 Bytes

QUIC Short: RFC9000

STREAM Frame: RFC9000

Datagram Capsule: RFC9297

HTTP DATAGRAM: RFC9297

STREAM FRAME Length parameter is required to support multiplexing capability

STREAM FRAME Offset parameter is required to keep send order

For the MP-QUIC IP solution an extra 8 Bytes needs to be added to all header sizes as the UDP header of the e2e data persists.

MP-DCCP-LL encapsulates IP data, therefore compared to the MP-QUIC UDP solution the inner IP/UDP header of the e2e data counts.

Data Packet
MP_RTT for Reordering
---DCCP------- + -----MP-DCCP--- + Payload

Generic Header + MP-SEQ + MP-RTT + IP + UDP
12 or 16       + 9      + 12     + 20 + 8 = 61/65 Bytes

DCCP Timestamp for reordering

---DCCP------------------- + MP-DCCP + Payload
Generic Header + timestamp + MP-SEQ  + IP + UDP
12 or 16       + 6         + 9       + 20 + 8 = 55/59 Bytes

#### 7.1.2.3 Evaluation of the impact on performance because of Encryption

As specified in RFC9001 (Using TLS to Secure QUIC), each QUIC connection is established using TLS v1.3 handshake and all QUIC packets are encrypted, and integrity protected using the negotiated TLS keys and algorithms. As RFC9001 is mandatory for QUIC transportation, this also applies to the MP-QUIC solutions #2.2 and #2.3. However, in case of ATSSS, where QUIC is applied between the UE and UPF and where access-level security mechanisms are used to protect the user-plane traffic, the additional security mechanisms in the QUIC layer is not needed.

Whether QUIC encryption can be removed to avoid impact on battery consumption, throughput and latency is unclear as SA3 has not yet started their work on this issue. And even if SA3 Stage 3 would specify a non-IETF conform 3GPP proprietary solution, it is doubtful if this is adopted into OS provided MP-QUIC libraries.

In contrast, MP-DCCP-LL is not affected by encryption.

##### 7.1.2.3 UPF resources impact

The number of MP connections across solutions #2.1-#2.3 is at least determined by the number of QoS flows. For the MP-QUIC solution one port can be used to serve many MP-QUIC connections from an UE, the “connectionID” parameter is required to distinguish the connections leading to additional header space. MP-DCCP makes use of a unique port per connection which is a clean way to separate transport flows in the Internet. It seems to be obvious, that both approaches require resources to keep states, independent on which layer this is implemented (Layer 4 port or “connection ID”). It is rather a disadvantage to use “connectionID” instead of port numbers.

In case same QoS level with different Steering modes is requested, MP-DCCL-LL has to establish a connection per QoS level and Steering Mode. This increases the number of connections over the one of MP-QUIC based solutions as there Steering Modes can be handled as separate streams within one MP-QUIC connections. On the other hand, MP-QUIC based solutions requires a UDP (#2.2) or IP (#2.3) flow ID per end-to-end UDP or IP connection to assign packets of an end-to-end connection to the right MP-QUIC stream ID. The effort for this seems to be comparable with NAT tables which have to be processed extensively for each data packet. This is not required for MP-DCCP-LL solution.

#### 7.1.2.3. How to treat out-of-order delivery caused by per packet-splitting

Independent if UDP, IP or Ethernet is addressed in the solutions #2.1 - #2.3, any end-to-end traffic characteristics can be expected in the range from: no requirements (simply UDP) up to strict in-order delivery (QUIC stream mode) with smooth end-to-end latency (BBR congestion control) demand. S2-2203965 studied these different demands in regard to ATSSS splitting feature and concludes that for enhanced throughput experiences the following key components are required on the ATSSS receiving end (UE or UPF):

- Sequence number based re-ordering

- Dynamic expiration timer for assuming packet loss

- Fast packet loss detection using connection and path sequencing information

- Path latency difference compensation

If not all of these ingredients are implemented, throughput is falling below single-path experience. A KI#2 solution has therefore to provide means to determine the order of an end-to-end stream before the splitting (e.g., connection sequence number), a path specific sequencing scheme, and the information about the path latency difference between 3GPP and non-3GPP access.

A connection sequence number is provided by all solutions #2.1-#2.3 which helps in principle to re-order an end-to-end stream.

In case packet loss occurs, however, a re-ordering process stall, unless the reordering process assumes or recognizes a missing packet lost. With the help of a path specific sequencing scheme and latency information packet loss can be reliably detected and avoid too early or too late assumption of packet loss. Solutions #2.2 and #2.3 does not provide latency information nor is it clear if the appropriate sequencing scheme is provided by the MP-QUIC IETF draft. Especially the MP-QUIC sequencing scheme is under heavy discussion there. In contrast, MP-DCCP-LL has no limitations.

In the case the carried traffic is sensitive to the jitter introduced by the simultaneous usage of 3GPP and non-3GPP path, smoothing this jitter is required. Like above for the packet loss detection MP-DCCP-LL is the only solution providing the required information.

#### 7.1.2.4 IETF support

MP-DCCP-LL solution #2.1 is composed of two documents in IETF which makes it clear to implement. RFC4340 (DCCP) is available as Standard RFC, its multipath extension is still on Experimental track. This seems however not to be limiting as 3GPP proofed adoption of Experimental IETF documents in the past, e.g., RFC 8803 for Rel. 16 ATSSS-HL solution.

In contrast MP-QUIC solutions #2.2 and #2.3 require multiple nested and dependent IETF documents as well as solution specific extensions for at least sequence number.  This complicates implementation and is exacerbated by the fact that none of the QUIC extensions have been specified with a view to interoperating in the way they are used in Solutions 2.2 and 2.3.

No prototype is known for solutions #2.2 and #2.3 which proofed functionality to achieve the goals for KI#2 and proofs that the leveraged QUIC extensions interoperate efficiently. In contrast, a public implementation is available for solution #2.1 (MP-DCCP-LL) that demonstrates suitability and was used to investigate the effect of re-ordering and latency compensation presented in S2-2203965.

### 7.1.3	Evaluation of steering functionalities for UDP traffic flows

#### 7.1.3.1	General

For supporting UDP traffic flows, two alternative solutions can be used: Solution #2.1 (MP-DCCP-LL) and Solution #2.2 (MPQUIC). The following sub-clauses discuss different aspects of these solutions.

The solution #2.3 may also be used for UDP traffic flows but this solution is more suitable when IP traffic flows (other than UDP) should be supported, thus, it should be considered separately.

#### 7.1.3.2	Allocation of UPF resources

As specified in clause 6.3.5, the MP-DCCP-LL solution requires the UPF to allocate: one IP address per access and one port per MP-DCCP connection, as shown in the figure below. The UPF allocates the IP addresses and ports for each MP-DCCP connection based on information received from SMF (MP-DCCP-LL Control Information), which indicates how many MP-DCCP connections are required. The SMF derives this information based on the PCC rules received from PCF. If additional MP-DCCP connections are required during the lifetime of the MA PDU Session, the SMF should ask UPF to allocate more resources for these additional connections and then send a PDU Session Modification message to UE including the allocated UPF resources.

Fig. 7.1.2.2-1 (a)

On the contrary, the MPQUIC solution requires the allocation of one IP address and one UDP port at the UPF, which are used for all MPQUIC connections of a UE. This is possible because the QUIC protocol can multiplex many connections on the same UDP port (using the “connection ID” parameter). The UE receives the IP address and UDP port at the UPF as part of the “proxy information” in the PDU Session Establishment Accept (exactly the same as in MP-TCP). When the UE needs to establish a new MPQUIC connection to UPF, the UE applies the received “proxy information” and does not require additional resources to be allocated at the UPF.

Fig. 7.1.2.2-1 (b)

#### 7.1.3.3	Connections between UE and UPF

The MPQUIC solution requires one MPQUIC connection per QoS flow (see clause 6.11.2). Each UDP flow transferred via an MPQUIC connection is associated with a unique QUIC stream on the connection and each QUIC stream is configured to apply a steering mode. Thus, different steering modes can be applied per MPQUIC connection.

The MP-DCCP-LL requires one connection per QoS flow and per Steering Mode (see clause 6.3.2), so it needs more connections than MPQUIC. The MP-DCCP-LL connection management (adding / removing connections) is cumbersome: Every time a new UDP flow should be transmitted using a steering mode for which no MP-DCCP connection supports, a new MP-DCCP connection should be established and the SMF should send a PDU Session Modification message with the parameters (UPF address/ports) for this MP-DCCP connection.

The MP-DCCP-LL solution specifies that the UE receives "MP-DCCP Connection Setup Information", which indicates the QoS flow and the steering mode associated with each MP-DCCP connection. However, it does not specify how the UPF knows the QoS flow, and the steering mode associated with each MP-DCCP connection. In the MPQUIC solution, the QoS flow associated with an MPQUIC connection is indicated (as a new transport parameter) during the connection setup.

The number of MP connections across solutions #2.1-#2.2 is at least determined by the number of QoS flows. For the MP-QUIC solution one port can be used to serve many MP-QUIC connections from an UE, the “connectionID” parameter is required to distinguish the connections leading to additional header space. MP-DCCP makes use of a unique port per connection which is a clean way to separate transport flows in the Internet. It seems to be obvious, that both approaches require resources to keep states, independent on which layer this is implemented (Layer 4 port or “connection ID”). It is rather a disadvantage to use “connectionID” instead of port numbers.

In case same QoS level with different Steering modes is requested, MP-DCCL-LL has to establish a connection per QoS level and Steering Mode. This increases the number of connections over the one of MP-QUIC based solutions as there Steering Modes can be handled as separate streams within one MP-QUIC connections. On the other hand, MP-QUIC based solutions requires a UDP (#2.2) flow ID per end-to-end UDP connection to assign packets of an end-to-end connection to the right MP-QUIC stream ID. The effort for this seems to be comparable with NAT tables which have to be processed extensively for each data packet. This is not required for MP-DCCP-LL solution.

#### 7.1.3.4	User-plane overhead

As shown in the figure below, the MP-DCCP-LL steering functionality implements an IP-in-IP tunneling solution, whereas the MPQUIC steering functionality implements a UDP proxying solution over a HTTP/3 tunnel. The Table 1 in section 7.1.2.2.1 summarizes user plane overhead due to additional headers for each solution.

Additional considerations are that

both the MP-DCCP-LL and the MPQUIC send Ack packets to acknowledge successful reception.

As shown in figure below, when the app provides two payloads in succession, the MPQUIC steering functionality can multiplex these payloads in the same QUIC packet (this is a feature supported by QUIC), thus, reducing the user-plane overhead by saving in this case 34 Byte . In case of MP-DCCP-LL, however, the two app payloads will be encapsulated into two IP (inner) packets and will be transmitted with two DCCP-Data packets resulting in 20 Byte overhead in this case. In addition, in case of MPQUIC, a single UDP datagram can multiplex multiple QUIC packets, which can further reduce the overhead.

NOTE: Multiplexing in this solution can work only if by chance two subsequent payload packets for same transport mode are to be scheduled at same point in time and both fit completely inside a single QUIC packet which is limited by UDP datagram and max. MTU size, typically 1200 Byte. This is a consequence of RFC 9000.

To reduce user-plane overhead, the MP-DCCP-LL solution can support header compression, e.g., with a solution similar or identical to solution #2.4. The header compression introduces additional processing.

An overhead of the MPQUIC steering functionality, which is not evident from the figure below, is the HTTP CONNECT request plus the 200 OK response, sent at the beginning of every UDP flow.

Figure 7.1.3.4-1: Example of user-plane overhead when using the MPDCCP and MPQUIC steering functionality (UL direction)

#### 7.1.3.5	Security

As specified in RFC 9001 (Using TLS to Secure QUIC), each QUIC connection is established using TLS v1.3 handshake and all QUIC packets are encrypted, and integrity protected using the negotiated TLS keys and algorithms. However, in case of ATSSS_ph3, where QUIC is applied between the UE and UPF and where access-level security mechanisms are used to protect the user-plane traffic, the additional security mechanisms in the QUIC layer is unnecessary. The MPQUIC solution contains the following EN, so this issue is expected to be addressed by SA3.

Editor's note: Whether and how encryption in the QUIC layer can be omitted is FFS and need to be verified by SA3.

The double-layer security is not an issue for MP-DCCP-LL, which does not provide cryptographic security mechanisms and relies on other security methods (e.g., IPsec or 3GPP user-plane security) to protect the traffic.

#### 7.1.3.6	Packet reordering and deduplication

Both MP-DCCP-LL and MPQUIC solutions provide the means to support packet reordering and deduplication.

The MP-DCCP-LL solution mentions that "MP-DCCP protocol itself does not specify re-ordering mechanisms but provides path sequencing (DCCP packet sequencing [11]), connection sequencing (MP_SEQ option [12]), and has inherent latency (MP_RTT option[12]) or timing (DCCP timestamp option[11]) information exchange, i.e., all properties that are necessary for the packet receiving side, i.e. UE or UPF, to be able to implement re-ordering properly." Although the MP-DCCP draft [12] states that "The details of the transmission scheduling mechanism and optional reordering mechanism are up to the sender and receiver, respectively, and are outside the scope of the MP-DCCP protocol," it is understood that such mechanisms can be considered and possibly defined in stage-3.

The MPQUIC solution can readily support reordering and deduplication by leveraging the existing QUIC stream mechanisms, which are applied in the Stream transport mode. So, when the UDP traffic can be transferred in Stream transport mode, no additional mechanisms need to be specified for supporting packet reordering and deduplication.

In addition, the MPQUIC solution supports Datagram mode 1 (see clause 6.11.3), which supports unreliable transport and inserts sequence numbers in every UDP data packet. In Datagram mode 1, every UDP packet is encapsulated into a QUIC Datagram frame, which also carries a Context ID and a sequence number. The definition of this Context ID, as well as other aspects of Datagram mode 1 (e.g., reordering mechanisms), will be considered in stage-3.

#### 7.1.3.9	IETF Support

The QUIC multipath draft is planned to be published as a Standards Track RFC, while the DCCP multipath draft is planned to be published as an Experimental RFC.

As specified in RFC 2026:

"Specifications that are not on the standards track are labeled with one of three "off-track" maturity levels:  "Experimental", "Informational", or "Historic".  The documents bearing these labels are not Internet Standards in any sense."

MPDCCP has announced working group last call for November 2022.

MPQUIC draft has no associated work group mile stone available at the moment.

NOTE1: For MPDCCP a working implementation already exists.

NOTE2: TCP Converter as specified for MPTCP proxy is based on the Experimental RFC 8803.

End of Changes