3GPP TSG-SA WG2#153E e-meeting	S2-2209118r01

Elbonia, 10-17 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	Conclusions based on the reply from ITU-T SG15 Q13/Q15 on monitoring of time synchronization and relevant parameters

Document for:	Discussion/Approval

Agenda Item:	9.7.1

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper discusses the reply from ITU-T SG15 Q13/Q15 on monitoring of time synchronization and relevant parameters.

# 1.	Discussion

## 1.1	Overview

(Ericsson comments are added as Observation ERx, also included some of the response texts from ITU-T that were omitted but we consider to be relevant to the discussion with MS User ITU-T ER)

SA2 sent an LS to ITU-T SG 15 Q13/Q15 to ask the following questions:

1. What type of information about the 5GS time source are relevant to determine that it deteriorates and may impact time synchronization services already in place for the User Equipment (UEs)?

2. What information made available by the 5GS to the UE would be sufficient to enable applications in a UE or device attached to the UE to compare the 5GS clock against other clocks that may be locally available to the applications in the UE/devices attached to the UE so that the best available clock can be selected?

It is important to note that SA2 did not ask about specific means how 5GS time would be provided by the UE to devices behind the UE.

Observation ER1: It is important to understand the applications and relevance of information like timing related data to have a solution in 3GPP that is compatible to them.

In [1] ITU-T SG 15 replies:

Depending on the timing technology used, different types of information are exchanged between the network nodes to be able to detect issues and rearrange the synchronization network.

Observation ER2: ITU-T SG 15 cannot identify a unified parameter set that can be used to detect the 5GS time source deterioration as different technologies may be used, which also impacts which information is exchanged between the network nodes. Note that SA2 relates this ITU-T SG 15 reply in the context of Q1.

In regards of Q2, ITU-T SG 15 replies:

In answer to your question, if the timing is offered to the client connected to the devices/UE via a PTP interface, and assuming the client network makes use of a specific PTP profile, information must be provided to the PTP clock in the device so that Announce messages compliant with the relevant profile (e.g., ITU-T G.8275.1, IEEE802.1AS, the SMPTE PTP profile, etc.) can be generated. In this way it is possible to “enable applications in a UE or device attached to the UE to compare the 5GS clock against other clocks that may be locally available to the applications in the UE/devices attached to the UE so that the best available clock can be selected.” As an example, this information includes value of the clockClass and other relevant parameters that should be carried by the outgoing Announce messages.

Observation 1: ITU-T SG 15 focuses only on the scenario that time is provided by the UE to attached devices using PTP. For that scenario ITU-T SG 15 comments that the information that needs to be populated by the UE in PTP Announce messages would need to be provided to the UE (clockClass and other parameters that are part of the Announce message). Unfortunately, ITU-T SG 15 does not comment on the general case that SA2 asked about.

Observation ER3: ITU-T SG 15 uses the PTP case as an example to clarify that a UE should get only minimum information relevant to perform its task; in ITU-T SG 15 example, it is to generate Announce messages. Observations regarding the general case are listed below.

Furthermore, ITU-T SG 15 replies:

Depending on the agreement between the 5GS network operator and the client network operator, a subset of the values for the applicable parameters could be sufficient. As an example, clockClass 6 could be used when the offered service meets the agreed minimum performance, and clockClass 248 when the requirement is not met. That would be sufficient for the client application to select a local clock or use the timing delivered by the 5GS.

Observation 2: By the introductory qualifier "Depending on the agreement between the 5GS network operator and the client network operator…" ITU-T SG 15 underlines the need to first conclude on the type of agreements for time synchronization and timing resiliency that are to be supported between MNOs and client networks before concluding on the type of information that may or may not be sufficient to inform the UE about the clock quality of the network.

Observation ER4: ITU-T SG 15 presumes there should be an agreement between the 5GS network operator and the client network operator where certain parameters, their specific values or certain value ranges are specified/agreed upon. Obviously, the client network operator knows requirements for the applications/services running in its network. Therefore, the ITU-T SG15 confirms that “a subset of the values for the applicable parameters could be sufficient”. As an example, ITU-T uses a PTP case where clockClass can be described by two states: 1) clockClass 6 when its quality is as agreed in the agreement, i.e., is OK; 2) clockClass 248 when it's in a free-run mode and does not meet qualities agreed in the agreement, i.e., is not OK.

Observation ER5: ITU-T SG 15 considers that a limited subset parameter values (specified in the agreement between the 5G network operator and the client network operator) should be sufficient to select between a local clock and the 5GS clock. Furthermore, ITU-T does not mention/consider any possibility to use 5GS clock despite it does not meet parameters in the agreement.

Observation 3: A discussion on the type of agreements for time synchronization and timing resiliency that are to be supported between MNOs and client networks has not happened yet in SA2. Clause 1.2 provides an initial discussion of this aspect.

Furthermore, ITU-T SG 15 replies:

It is important to note that the 5G system (including the UE) needs to estimate the overall accuracy that can be delivered to the client. However, the methods described earlier (including the aspects under study) can be used up to the gNB and not specified for the air interface between the gNB and the UE.

Related to the example of parameters mentioned in your liaison that can be delivered by SIB/RRC (e.g., uncertainty information), it is understood that in the scenarios under consideration by 3GPP, the air interface (between the gNB and the UE), as well as the characteristics of the UE clock,  have a great impact on the overall performance that can be delivered at the output of the 5GS (i.e., at the input of the connected clients). Due to that, knowledge of certain accuracy delivered at the gNB (and that could be forwarded via SIB/RRC as uncertainty of the 5GS time), could be significantly different from the accuracy actually delivered by the 5GS. Having a simplified approach as indicated in the previous example with limited set of the values for the relevant parameters (e.g., clockClass), might be beneficial in this perspective.

Observation 4: ITU-T SG 15 comments that the "the air interface (between the gNB and the UE)" has a great impact on the overall performance" and suggests that the gNB cannot consider reference time inaccuracies due to the air interface. The latter is incorrect since as of Rel-17, UEs and gNBs can measure propagation delays between gNB and UE and compensate for these delays.

Observation ER6: ITU-T SG 15 also stresses the importance of estimating the overall accuracy (and thus, the end-to-end status) of the timing that could be delivered to the client. An end-to-end accuracy, besides a 5GS part until the BS antenna, includes also an air interface part and a UE part. Yes, even within Rel-17 capabilities for some cases UEs and gNBs can measure and compensate for some delays, but it depends on RF configurations and therefore imposes certain network involvement and resources. For instance, assessment of the air interface by using one of the options in RAN propagation delay information where the UE receives this, still requires the network to be involved and allocate resources to perform measurements etc. However there exists different types of RF propagation delay methods (RTT based, TA based or even maybe not performing any at all and it must be the network to decide which method to be used dependent a total 5GS assessment since it is in control of spectrum resources etc.). Still the air interface budget is not only about RF propagation delay compensation, it is not fixed, it depends on how the network sends reference signals, their characteristics with what periodicity, BW, radio channel properties etc., and much of this has network / spectrum utilization dependencies.

Observation 5: ITU-T SG 15 is correct that the gNB is not aware of the clock characteristics of the UE and hence cannot consider the UE characteristics when providing clock quality information to the UE. However, it is not obvious why ITU-T SG 15 considers this an issue since the UE can correct the clock quality information received from the network based on its local awareness of its clock characteristics.

Observation ER7: ITU-T SG 15 confirms that characteristics of the UE clock is as important as the air interface part when it comes to the impact on the overall performance that can be delivered at the output of the 5GS. UEs internal accuracy is implementation specific, and it is not being reported to a gNB, i.e., gNB is not aware of the actual timing errors introduced by a UE due to its internal characteristics. Even now within Rel-17 specifications, a Uu time synchronization error budget, which is important component of end-to-end status as pointed out by ITU-T SG 15, is derived using a predefined values or calculated using the implementation specific means, i.e., not the actual values (see clause 5.27.1.9 of TS 23.501). Therefore, providing the exact values (not those agreed in SLA) of 5GS clock accuracy does not contribute to informing UEs about the overall accuracy that can be delivered by the 5G network to the client network.

Observation 6: It is not obvious why a limited set of parameter values would address the fact that the gNB is not aware of the UE's clock characteristics. The type of parameters to provide to the UE are rather dependent on the agreement between the MNO and the client network, as ITU-T SG 15 commented themselves (see Observation 2).

Observation ER8: Considering that two of three “sources” of timing errors (i.e., air interface part and UE part) are UE or UE location specific and rely on certain approximations/assumptions, there is no good use of providing exact values of 5GS clock accuracy. By doing that, it not only does not contribute to the knowledge about the overall accuracy that can be delivered by the 5G network to the client network, but also may raise security concerns and difficulties to control the service via subscription (see S2-2208400). As pointed out earlier, the Clock quality that is provided via SIB/RRC messages do not consider the air interface and UE characteristics. Therefore, an approach where specific clock quality values that have been agreed beforehand between the MNO and the client network would be more beneficial and sufficient to choose between local clocks or the 5GS clock. ITU-T SG 15 has a similar consideration, which is confirmed by their statement “Having a simplified approach as indicated in the previous example with limited set of the values for the relevant parameters (e.g., clockClass), might be beneficial in this perspective.”

For the reasons listed above the reply from ITU-T SG 15 is not very helpful for SA2 at this stage to address the question, which led to the LS to ITU, i.e., to determine which information needs to be provided to the UE to enable applications in a UE or device attached to the UE to compare the 5GS clock against other clocks that may be locally available so that the best available clock can be selected.

However, one aspect raised by ITU-T SG 15 should be considered by SA2: SA2 needs to first conclude on the time synchronization and timing resiliency agreements that are to be supported between MNOs and client networks before concluding on the information that needs to be provided to UEs to inform the UE about the clock quality of the network.

Conclusion: The reply from ITU-T SG 15 is not very helpful for SA2 at this stage to address the general question, which specific clock quality information to provide to the UE so that the best available clock can be selected but it has emphasized the need for SA2 to first conclude on the scenarios and agreements for time synchronization and timing resiliency that are to be supported between MNOs and client networks.

Conclusion ER: ITU-T SG 15 provided inputs are of importance. They have provided two important insights to be considered in SA2: (i) that a subset of parameter values, (e.g., clock quality) agreed between the MNO and the client network would determine whether the requirements are met or not and would be sufficient to select between local clocks or using the 5GS timing; (ii) the information contained in the SIB (e.g., accuracy) does not contain the air interface and UE parts, but these components could be decisive in which accuracy can actually be delivered by the 5GS. Therefore, ITU-T SG 15 considers that it would be beneficial to use a limited set of the values for the relevant parameters that is agreed upon in the agreement between parties.

Clause 1.2 provides an initial discussion of this aspect.

## 1.2	Timing resiliency scenarios to be supported: Ensuring flexibility for operators

Different timing service consumers have different clock quality needs, e.g., based on national regulation or operating requirements, for example:

-	Some timing service consumers may, depending on regulation, only be able to use the 5GS time when the 5GS clock is connected to a Primary Reference Time Clock (PRTC) (e.g., a PRTC traceable to GNSS).

-	Other time service consumers may prefer to use a PRTC that is traceable to GNSS but can also use 5GS time if the 5GS clock operates in hold-over as long as the network informs the timing service consumer that the 5GS clock operates in hold-over state and provides the related clock accuracy (e.g. because time stamps are generated based on 5GS time).

In other words, while some timing service consumers can only operate with a single, very specific clock quality, other timing service consumers can use 5GS time across a wider range of clock qualities as long as the network transparently informs about the clock quality at any point in time.

This implies that 5GS needs to support informing UE and AF about the 5GS clock quality, including 5GS clock quality degradation/improvement at different levels of degradation and improvement.

It is an agreement (between MNOs and client networks) that defines the range of values of clock quality for every subscription, and not a general 5GS clock quality information that covers all cases. Covering all cases in one general case (by informing about every single general change of 5GS clock quality) is not providing flexibility, it is indeed removing the possibility to differentiate between various subscription levels. While for one service offered the clock quality has a degradation, for another it might still be within the acceptable range and, thus, the information provided about the quality degradation can be considered as unnecessary, misleading, or even potentially dangerous. Actions taken under degradation/ improvement are different even in the 5GS.

Observation ER9: Different service consumers have different subscription and specific clock quality values and ranges. Therefore, the status report should be provided on a per service level and, based on agreement between MNOs and client networks, the clock quality included in this status report should be enough to select a clock. Otherwise, status reports will be provided in general even when there is no degeneration (within range) for a particular service subscribed.

# 2.	References

[1]	ITU-T SG15: Reply to Liaison on Monitoring of network time synchronization and relevant parameters. https://www.itu.int/dms_inf/itu-t/md/22/sg15/td/220919/PLEN/T22-SG15-220919-TD-PLEN-0063!!MSW-E.docx

# 3.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-25:

# >>>>BEGINNING OF CHANGES<<<<

# 8	Conclusions

# 8.X	Conclusion for KI #1: 5GS network timing synchronization status and reporting

- A time synchronization status report sent to a UE or to an AF shall be based on the parameter values specified in the agreement between the 5G network operator and the client network operator and informing whether a certain subset of the values (requested by the AF or stated in the UE’s subscription) for the applicable parameters are fulfilled or not. Clock Quality values defined in such agreement are sufficient for the client network to select among local clocks and the 5GS clock.

NOTE: In order to have an end-to-end accuracy, we need to know three components (1) UE internal, (2) Uu air interface, and (3) CN part –

- as per Rel17

  - UE internal accuracy: Unless UE reports it’s accuracy to the TSCTSF, we don’t have this. In Rel-17 specs operates with some preconfigured values or assumes that 5GS can measure it using the implementation-specific means.

  - Uu part: it could be very different and depends on UEs current location. It changes as soon as UE moves to another cell, etc., So, Uu part needs to be re-assessed.

  - 5GS (or CN) part: up till gNB antenna, preconfigured value.

- From (a) and (b) seen that we already have these 2/3 components using a simplified approach, i.e., use a preconfigured value or average assumptions about UEs characteristics.

- Therefore, it does not make sense to deliver accurate “network” component. Even if we do that, it will not contribute to the knowledge about the overall accuracy that can be delivered by the 5G network to the client network. It’s like summing up two values, where one is estimated/measured with high precision, but the other is estimated/measured loosely, and both of them have the similar “weight”. The result is as accurate as is the most inaccurate component.

As such, it would be much better to report whether or not 5GS fulfills the requirements in the agreement.

Editor’s note: How the applicable parameters are represented in a status report (i.e., whether to provide specific metrics from the agreement between the MNO and the client network operator or just an indication (e.g., OK or not OK) about whether the agreed requirements are fulfilled or not, or whether to support a combination of both approaches) is FFS.

# >>>>END OF CHANGES<<<<