# SA WG2 Meeting #S2-153E	S2-2209134r03

# 10 - 17 October, 2022, Electronic meeting

# Title:	[Draft] LS on RAN dependency for Ranging/Sidelink Positioning

# Response to:

# Release:	Release 18

# Work Item:	FS_Ranging_SL

Source:	SA2

To:	RAN1, RAN2, RAN3

Cc:

Contact Person:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

FS_Ranging_SL study has reached 85% completion up to SA2#153E in SA2, and the evaluation & conclusion for the 8 Key Issues (KI) are in process.

During the evaluation & conclusion, the following issues are identified pending for comments from RAN WGs to conclude the KIs in TR 23.700-86:

- What are the parameters required for Service Authorization to UE from RAN perspective?

- What are the parameters required for Service Authorization to NG-RAN from RAN perspective?

- To support Ranging/SL Positioning using Assistant UE, how the determination of using assistant UE and the assistant UE selection is performed from RAN perspective?

- What are the enhancements to 5G ProSe Discovery procedures and V2X Communication procedures to support Ranging/SL Positioning discovery?

- We concluded a Ranging/SL Positioning layer is introduced between Application layer and lower layers; however, whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is open for RAN decision.

- We concluded that a new Ranging/Sidelink Positioning protocol (i.e. RSPP) will be used for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE); however, whether it will be over PC5-S or PC5-U is open for RAN decision.

- We concluded that LMF may be involved when the Target UE and the Reference UE are both in the network coverage, and the protocol used between UE and LMF can be a standalone extension of LPP such that only this extension needs to be supported for UEs supporting only SL Positioning/Ranging. This extension and RSPP should be defined as common as possible. We would like to understand whether this is feasible from RAN perspective?

- For out-of-coverage we would like to understand how resource coordination and scheduling will be done to enable SL Positioning/Ranging.

- In case a constrained UE not able to support all SL Positioning/Ranging features, whether an SL Positioning Server UE can be introduced for out-of-coverage operation, providing resource coordination and scheduling, SL Positioning/Ranging method determination and positioning/ranging calculation. The role of a SL Positioning Server UE can be performed by the Target UE and SL Reference UE or the SL Positioning Server UE is a different UE all together. We would like to understand whether this is reasonable from RAN perspective.

Based on our current work plan, SA2#154AH is the last meeting for the study, hence, it is highly appreciated that RAN WGs would evaluate the above issues and give us the feedback before SA2#154AH.

2. Actions:

To RAN1, RAN2, RAN3

ACTION: 	RAN WGs give SA2 the feedback for the above issues, preferably before SA2#154AH.

3. Date of Next SA2 Meetings:

SA2#154	14th - 18th Nov 2022	Toulouse, France

SA2#154AH	16th - 20th Jan 2023	Electronic meeting