# SA WG2 Meeting #S2-153E	S2-2209134r06

# 10 - 17 October, 2022, Electronic meeting

Title:	[Draft] LS on RAN dependency for Ranging/Sidelink Positioning

Response to:

Release:	Release 18

Work Item:	FS_Ranging_SL

Source:	SA2

To:	RAN1, RAN2, RAN3

Cc:

Contact Person:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

FS_Ranging_SL study has reached 85% completion up to SA2#153E in SA2, and the evaluation & conclusion for the 8 Key Issues (KI) are in process.

During the evaluation & conclusion, the following issues are identified pending for comments from RAN WGs to conclude the KIs in TR 23.700-86:

- We have identified several RAN relevant parameters required for Service Authorization to UE, e.g.  the mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters, and we would like to understand what are the QoS parameters used at AS layer for Ranging/SL positioning. Furthermore, it is still open if any QoS parameters related to the transport of RSPP used by NG-RAN need to be authorised from PCF? Or is there any other QoS parameters to be authorised to NG-RAN?

- It is still open if any QoS parameters related to the transport of RSPP need to be authorised from PCF to the NG-RAN, and we would like to understand view from RAN WGs. Are there any other QoS parameters to be authorised to NG-RAN?

- To support Ranging/SL Positioning using Assistant UE, how the determination of using assistant UE and the assistant UE selection/reselection is performed from RAN perspective?

- On Ranging/SL Positioning discovery,  we concluded to reuse 5G ProSe Discovery procedures and V2X Communication procedures with the additional Ranging/SL Positioning parameters; however, it is not decided whether those Ranging/SL Positioning parameters are transparent to ProSe/V2X layer or not, and we would like to understand the views from RAN perspective.

- We concluded a Ranging/SL Positioning layer is introduced under Application layer; however, whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is open. We concluded that a new Ranging/Sidelink Positioning protocol (i.e. RSPP) will be used for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE and SL Positioning Server UE), which can be over either PC5-S or PC5-U The Pros & Cons are evaluated based on the following technical considerations:

- All unicast, broadcast and group-cast modes are assumed to be supported for Ranging/SL Positioning.  PC5-S doesn’t support broadcast and group-cast.

- Impact to existing protocols: a standalone extension of PC5-S is expected if PC5-S is used or RSPP transported over PC5-U as the payload.

- QoS of RSPP transportation: RAN to guarantee RSPP QoS the same as V2X/5G ProSe in case of PC5-S or 5GC to require per Application RSPP QoS in case of PC5-U.

We can’t reach the consensus between PC5-S or PC5-U, and we expect the RAN WG evaluation as the input so as to make a decision in the conclusion.

- We concluded that LMF may be involved when the Target UE and the Reference UE are both in the network coverage, and the protocol used between UE and LMF can be a standalone extension of LPP such that only this extension needs to be supported for UEs supporting only SL Positioning/Ranging. This extension and RSPP should be defined as common as possible. We would like to understand whether this is feasible from RAN perspective?

- For out-of-coverage we would like to understand how resource coordination and scheduling will be done to enable SL Positioning/Ranging.

- In case a constrained UE not able to support all SL Positioning/Ranging features, whether an SL Positioning Server UE can be introduced for out-of-coverage operation, providing resource coordination and scheduling, SL Positioning/Ranging method determination and positioning/ranging calculation. The role of a SL Positioning Server UE can be performed by the Target UE and SL Reference UE or the SL Positioning Server UE is a different UE all together. We would like to understand whether this is reasonable from RAN perspective.

Based on our current work plan, SA2#154AH is the last meeting for the study, hence, it is highly appreciated that RAN WGs would evaluate the above issues and give us the feedback before SA2#154AH.

2. Actions:

To RAN1, RAN2, RAN3

ACTION: 	RAN WGs give SA2 the feedback for the above issues, preferably before SA2#154AH.

3. Date of Next SA2 Meetings:

SA2#154	14th - 18th Nov 2022	Toulouse, France

SA2#154AH	16th - 20th Jan 2023	Electronic meeting