3GPP TSG-SA WG2#153E e-meeting	S2-2209154r02

Elbonia, 10 – 17 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated, Charter Communications, AT&T

Title:	KI#5: Conclusion update to address EAS discovery

Document for:	Discussion/Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This contribution finalizes the description of Sol. 22 option 2.

# 1.	Discussion

This document addresses the EN in the conclusion of KI#5 related to EAS discovery

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-48:

# >>>>BEGINNING OF CHANGES<<<<

## 8.5	KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed in another PLMN (i.e., the hosting PLMN in the role of a partner Operator Platform) than the one serving the UE (i.e., the serving PLMN), the following is recommended as baseline for normative work.

EAS Deployment Provision

The serving PLMN is provided with the information needed to support UEs to discover and connect to the shared EHE deployed in the hosting PLMN. In particular, the serving SMF may either store:

-	the FQDN(s), DNS Server Information, and/or DNAI(s) of the EAS Deployment Information (EDI, see TS 23.548 [x] clause 6.2.3.4-1) for each partner and use the Partner ID (e.g., PLMN ID or partner EHE ID) to identify to which partner that EDI. Such EDI information can be preconfigured in the serving SMF provided by the AF

NOTE 0:	The NEF from the hosting PLMN can act as the AF for the serving PLMN.

NOTE 1:	Whether the provision of additional parameters of the partner PLMN’s EDI (TS 23.548 [x] clause 6.2.3.4-1, e.g., DNN, S-NSSAI etc.) to the serving PLMN violates the GSMA OPG requirement of topology hiding or not will depend on feedback from GSMA OPG.

NOTE 2:	The final list of EDI parameters that can be exchanged among PLMNs will be determined in the normative phase based on feedback from GSMA OPG.

-	A list of preconfigured DNS servers per Partner ID. Such information is configured in the serving SMF and is provided to the serving EASDF (see clauses 6.22.2.2 and 6.22.3.2), to be used in case the EDI is not available.

In both cases, the serving SMF may send filter information (see 6.22.2.2 and 6.22.3.2) to the serving EASDF used to match the DNS query to the corresponding Partner ID and to select the proper partner EASDF or preconfigured DNS server..

NOTE 3:	If no filter information is provided to the serving EASDF, in case of list of preconfigured DNS servers, the serving EASDF can forward the DNS query to multiple DNS servers, and multiple DNS responses can be received by the serving EASDF.

EAS Discovery Procedure

-	The EAS discovery procedure defined in TS 23.548 [3] will be reused.

Traffic transmission between two PLMNs

-	When the traffic is transmitted between two PLMNs, the UPF in the serving PLMN should support mechanisms to facilitate low latency transmission.

Edge Relocation

Editor's note:	It is FFS how to support the Edge Relocation in GSMA OPG scenario.

Editor's note:	It is FFS how to support edge relocation between an EAS deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers.

Editor's note:	It is FFS how to support edge relocation between PLMNs.

# >>>>END OF CHANGES<<<<