3GPP TSG-SA WG2#153E e-meeting	S2-2209154r06

Elbonia, 10 – 17 October 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated, Charter Communications, AT&T

Title:	KI#5: Conclusion update to address EAS discovery

Document for:	Discussion/Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This contribution finalizes the description of Sol. 22 option 2.

1.	Discussion

This document addresses the EN in the conclusion of KI#5 related to EAS discovery

2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-48:

# >>>>BEGINNING OF CHANGES<<<<

8.5	KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed in another PLMN or partner Operator Platform (i.e., the Edge Node Sharing Scenario as described in GSMA OPG.02 [X]  the following is recommended as baseline for normative work.

EAS Deployment Provision

The serving PLMN is provided with the information needed to support UEs to discover and connect to the shared EHE deployed in the hosting network. In particular, the serving SMF may either store:

-	the FQDN(s), DNS Server Information, and/or DNAI(s) of the EAS Deployment Information (EDI, see TS 23.548 [x] clause 6.2.3.4-1) for each partner Operator Platform and use the Partner ID (e.g., PLMN ID) to identify to which partner that EDI corresponds. Such EDI information can be preconfigured in the serving SMF provided or provided by the AF.

-	The serving SMF configures EASDF according to EDI based on Rel-17 methods (see TS 23.548 [3]).

-	The DNS server can be selected by SMF by considering the UE location as described in Rel-17 TS 23.548 [3]. In addition.

EAS Discovery Procedure

-	The EAS discovery procedure defined in TS 23.548 [3] will be reused.

Traffic transmission between two PLMNs

-	When the traffic is transmitted between two PLMNs, the UPF in the serving PLMN should support mechanisms to facilitate low latency transmission. N6 routing information (as defined in TS23.501 clause 5.6.7) could be included in EDI for routing specific edge.

NOTE:

How to support charging in EHE sharing scenario will be specified by considering GSMA OPG's feedback. Some cooperation with SA5 may be needed.

Edge Relocation

Editor's note:	It is FFS how to support the Edge Relocation in GSMA OPG scenario.

Editor's note:	It is FFS how to support edge relocation between an EAS deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers.

Editor's note:	It is FFS how to support edge relocation between PLMNs.

# >>>>SECOND OF CHANGES<<<<

7.5	Evaluation for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

The solutions addressing Key Issue #5 can be split into two categories:

1.	solutions that enable the discovery of an EAS deployed in another PLMN than the one serving the UE, and

2.	solutions that facilitate the edge relocation between PLMNs.

Solutions to enable the discovery of an EAS deployed in another PLMN than the one serving the UE.

This category of solutions encompasses Solution #20, Solution #21, Solution #22, Solution #38 and Solution #40. All solutions assume that the involved PLMNs are part of an GSMA OP federation and/or that there is a controlled inter-PLMN IP connectivity between the PLMN serving the UE and the PLMN hosting the EAS.

Solution #20 is based on the usage of the newly defined Global EASDF which can directly resolve the DNS query sent by the UE to discover the shared EAS (which can be deployed in any PLMN) or can first resolve the DNS query sent by the UE to discover the EASDF able to provide the IP address of the shared EAS. Both options require the UE to be configured with the Global EASDF's URL and to resolve it to obtain the Global EASDF's IP address. Because of that these two options are inconsistent with the Rel-17 EAS discovery procedure described in TS 23.548 [3] clause 6.2.3.2.2 (which, on the contrary, assumes that the EASDF's IP address is provided by the SMF via ePCO during PDU Session establishment/modification). In addition, the second option is incompatible with the Rel-17 EAS discovery because it requires the UE to obtain the EASDF's IP address by sending a DNS query to the Global EASDF, while the Rel-17 EASDF in the PLMN has no role to play.

Solution #22 describes three alternative options: Option 0 (based on the SMF configuration to know the EAS deployment information of the EAS running on other PLMN's edge infrastructure, e.g. IP address range(s)/FQDN(s)), Option 1 (based on the usage of the Shared EASDF which stores the EDI of each EAS running in each PLMN sharing Edge Nodes) and Option 2 (which relies on inter-PLMN inter-EASDF communication to resolve the DNS Query for an EAS hosted by another PLMN). Option 2 requires storing in the NEF/UDR/SMF/EASDF an additional filtering information (PLMN ID and DNS server address/FQDN filter) for the EAS' deployed in another PLMN – such information is essential to locate the EAS during the EAS discovery procedure. Solution #22 has no impact on the UE, but, depending on the option, it affects the CN entities to different degrees.

Solution #38 relies on the AF to provide extended EAS Deployment Information (EDI) which includes N6 traffic routing information to the PLMN hosting the EHE. After having been translated by the NEF of the original PLMN and sent to the PLMN serving the UE, the extended EDI is stored in the UDR of the PLMN serving the UE. The SMF of the serving PLMN can then use such EDI to find the EAS hosted in the other PLMN. Similar to Solution #22, this solution has no UE impacts.

Solution #40 has a similar concept to Option 2 of Solution #22, in that it forwards the DNS query from the EASDF of the PLMN serving the UE to the EASDF of the PLMN hosting the EHE, if the former cannot find it. Differently from Solution #22 though, the inter-EASDF communication takes place via SMF and NEF. In addition, Solution #40 proposes the use of additional information (S-NSSAI, DNN, UE location and External Group ID) to allow the SMF in the PLMN hosting the EHE to find the proper EASDF.

Solution #21 ensures that an EAS deployed by a certain operator in a VPLMN's EHE can only be discovered and accessed by the UE of that operator while roaming in local breakout to the VPLMN. To do this, the solution the AF to provide, together with the EAS Deployment Information, the PLMN ID of the EAS owner so that the SMF can match it with the UE's HPLMN ID when the EAS discovery is triggered. This solution, however, does not seem to address the ENS (Edge Node Sharing) deployment which allows OP B to deploy Edge Computing applications in the Operator A's EHE, without restricting the access to it to only OP B's UEs.

Solution #55 ensures that an EAS deployed by a certain operator (PLMN B) can only be discovered by UE with the support of EDI from this operator. The AF in PLMN B provides the EDI to PLMN A (serving PLMN) and the SMF can decide the DNS message handling rule of the DNS query which contains the FQDN in EDI from PLMN B and how to resolve the FQDN can only be known by PLMN B according to the EDI. Also, after receiving the DNS response from DNS server, the SMF in PLMN A recovers the mapping table between target DNAI and IP address/range and decides the target DNAI according to the EAS IP address in DNS response. The target DNAI can be used to select PSA/UL CL.

Solutions to enable the EAS relocation for shared EHE

Solution #23 proposes to support the scenario of EAS relocation between EHE deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers. It needs the NRF or NEF maintains parameters for each AF, e.g. EHE domain it supports, service area, etc. Then SMF selects suitable target AF. The impact is introduced on 5GC and no impact on UE.

Solution #39 proposes to support the scenario EAS relocation between EHEs deployed in different PLMNs. The AF sends corresponding PLMN ID to network to indicate the EAS should be relocated to the target PLMN, and based on retrieval of PLMN ID, SMF selects UPF to access target EAS via controlled IP network. The impact is introduced on 5GC and no impact on UE.

# >>>>END OF CHANGES<<<<