SA WG2 Meeting #153-e	S2-2209169r01

10-17 October 2022, Electronic Meeting

# Title:	[DRAFT] LS on Time Synchronization Status notification towards UE(s)

# Release:	Rel-18

# Work Item:	FS_5TRS_URLLC

Source:	3GPP SA WG2

To:	3GPP RAN WG2, RAN WG3, SA WG3

Cc:	3GPP RAN WG1

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

3GPP SA WG2 is working on the study item “Study on 5G Timing Resiliency and TSC & URLLC enhancements (FS_5TRS_URLLC)”, documented in TR 23.700-25. SA2 would like to get your feedback regarding 5GS time synchronization status report towards the UE(s) (KI #1).

SA2 has agreed that 5GS shall support informing UE and AF about the 5GS clock quality, including informing the UE and AF about 5GS clock quality degradation/improvement at different levels of degradation and improvement.

Editor's note: 	How the clock quality of a given cell is indicated to UE and AF, e.g., by providing a set of individual metrics (for example UTC traceability, accuracy, frequency stability, etc.) or by providing a clock quality index or whether to support both options is FFS.

In this regard, SA2 has discussed different methods on how the time synchronization status can be provided to connected UE(s). These methods are documented in TR 23.700-25 Annex A. and SA2 has the following questions:

- Which would be the preferred method of providing a time synchronization status (or an indication about a change of time synchronization status) to the UE(s) in RRC_IDLE or RRC_INACTIVE state using SIB (& more specifically SIB9)?

- On various solutions described in Annex A, one alternative proposes to use the Ciphered SIB approach used for positioning. SA2 would like to get feedback on this from SA3, RAN2 as well as RAN3, since this method will require SIB information to be sent to CN before broadcast to cipher.

- Is the use of the dedicated RRC signalling to deliver a time synchronization status report to UE(s) in RRC_CONNECTED state?

2. Actions:

To 3GPP RAN WG2, SA3, RAN3

ACTION:

SA WG2 kindly requests 3GPP RAN WG2, RAN3 and SA3 to provide feedback on the relevant areas of their expertise.

3. Date of Next TSG SA WG2 Meetings:

3GPP TSG-SA2 Meeting #154	14 - 18 November 2022	Toulouse, France

3GPP TSG-SA2 Meeting Ad Hoc	16 - 20 January 2023	e-meeting