SA WG2 Meeting #153e	S2-2209196r02

# 17 - 26 August 2022, Electronic meeting         		            (revision of S2-220xxxx)

Source: 	Deutsche Telekom

Title: 	Evaluation and Conclusions for KI#2

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution: This paper provides evaluation text for KI#2 steering functionalities and proposes an initial conclusion.

# Discussion

The following changes below discuss several aspects of Solution #2.1 (MP-DCCP-LL), Solution #2.2 (MPQUIC-UDP) and  3) Solution #2.3 (aka MPQUIC-IP) and proposes initial conclusion.

First Change (all text new)

## 8.1	Conclusions for KI #2: New steering functionalities for non-TCP traffic

In addition to support plain IP and Ethernet traffic it is concluded that:

- For supporting non-TCP traffic splitting (Ethernet, IP, UDP)  the MP-DCCP-LL solution will be specified during the normative phase, which will be based on solution #2.1 “MP-DCCP based Steering Functionality”

  - The specified lower-layer steering functionality shall be able to support all ATSSS steering modes (i.e., the redundant steering mode and all other steering modes defined in Rel-17).

End of Changes