SA WG2 Meeting #153E	S2-2209304

October 10 – 17, 2022, E-meeting

Source:	vivo

Title:	Update to Sol#43

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract of the contribution: This contribution proposes to update solution 43.

Discussion

It is proposed to update solution 43 for KI#1 in TR 23.700-41. And the main aspect of the updated information are:

- Explain the detail of secondary NSSAI determination by query NSSF, clarify that AMF do not need to connect to NSSF every time that secondary NSSAI is needed.

- Update the USRP to explain how UE can perform related processing after PDU session transfer is determined.

Proposal

It is proposed to update TR 23.700-41 as follows:

**** Start of Change ****

## 6.43	Solution #43: Allowed NSSAI Determination in Initial Registration to Support Network Slice Service Continuity

### 6.43.1	Introduction

The solution aims to address Key Issue #1: Support of network slice service continuity.

1)	No mobility scenario:

Scenario 1b): network slice or network slice instance is overloaded or undergoing planned maintenance in CN (e.g. network slice termination).

Scenario 1c): network performance of the network slice cannot meet the SLA.

2)	Inter RA Mobility scenario:

Scenario 2d): network slice or network slice instance is overloaded in the target CN.

### 6.43.2	Functional Description

This solution allows AMF to obtain a secondary NSSAI related to each S-NSSAI in Allowed NSSAI during registration procedure. The value of secondary NSSAI is decided by the AMF through querying NSSF. NSSF determines secondary NSSAI based on the slice load, UE subscription, etc. The AMF then stores the mapping between the secondary NSSAI and Allowed NSSAI that UE can access in current registration area, and UE will not need to re-visit the NSSF every time the value of secondary NSSAI needed unless the stored information in AMF cannot determine the secondary NSSAI in the current situation, e.g. the access type changes, the mapping information stored in the AMF is missing/expired, UE subscription changes or UE has moved out of current registration area, etc. The secondary NSSAI is also part of Allowed S-NSSAI, so the Allowed S-NSSAI consists two parts: S-NSSAIs of network slices which UE can access in current registration area and S-NSSAIs (i.e. secondary NSSAI) which network operator deploy to support slice continuity. USRP provided to UE contains only first part.

In no mobility scenario, AMF will trigger network slice PDU session change to the new secondary NSSAI.

In mobility scenario, the mapping information is provided from the source AMF to the target AMF during handover procedure, if the target AMF cannot support all the allowed NSSAI and secondary NSSAI, it will request to the NSSF in the registration procedure to ask for a new mapping information in current tracking area, make sure the new target AMF can also support the service continuity.

### 6.43.3	Procedures

#### 6.43.3.1	Secondary NSSAI Determination

Figure 6.43.3.1-1: Secondary NSSAI information obtained during initial registration procedure

1:	UE carries the Requested NSSAI to initiate the initial registration process.

2:	If the current Initial AMF needs to obtain the UE subscription information to perform the redirection/reroute process, the AMF initiates a Nudm_SDM_Get request to obtain the UE subscription information.

3:	UDM returns the subscription information of the UE.

4:	Initial AMF carries the Requested NSSAI, Subscribed NSSAI, TAI and other information to initiate a Nnssf_NSSelection_Get request to obtain relevant network slice selection information.

5:	NSSF returns Initial AMF related information, including: Allowed NSSAI, Secondary NSSAI mapping with Allowed NSSAI UE can access in current registration area, AMF Set or AMF address list, NSI-ID. NSSF can determine a secondary NSSAI for each Subscribed NSSAI by considering UE location, subscription and some other information.

6:	Initial AMF performs AMF redirection, and transmits the network slice selection information to the target AMF, including: Allowed NSSAI, Secondary NSSAI mapping corresponding to Allowed NSSAI UE can access in current registration area, AMF Set or AMF address list, NSI-ID. In this step, Target AMF stores the mapping messages of Allowed NSSAI UE can access in current registration area and Secondary NSSAI returned by NSSF, avoiding query NSSF every time UE registered into the network.

7,8:	The rest of the registration procedure.

#### 6.43.3.2	PDU Session Transferred to a New Network Slice in No Mobility Scenario

Figure 6.43.3.2-1: PDU Session transferred to a new network slice in no mobility scenario

1:	AMF decides to trigger PDU session handover to secondary NSSAI when the SLA cannot meet or the CN in overloaded. The determination of Secondary NSSAI is according to the relationship stored inside AMF during registration procedure.

2:	AMF triggers the PDU session Modification process, the AMF send secondary NSSAI to S-SMF in smf_PDUSession_UpdateSMContext request.

4-10.	The rest of PDU session management procedures in different SSC mode in clause 4.3.5 of TS 23.502 [5].

#### 6.43.3.3	PDU session transferred to a new network slice in mobility scenario

Figure 6.43.3.3-1: PDU Session transferred to a new network slice in no mobility scenario

1:	S-RAN determines to execute the N2 relocation process

2:	S-RAN sends a handover request to S-AMF

3:	During the T-AMF selection process, the corresponding relationship between the Secondary NSSAI and the Allowed NSSAI originally stored in the S-AMF is sent to the T-AMF.

After the information is received by T-AMF, it compares with the corresponding relationship stored by itself. If it is consistent with the corresponding relationship stored in the current T-AMF, then continue the rest of handover procedure; if it is inconsistent with the corresponding relationship stored in the current T-AMF, there are following possibilities:

Existed relationship inside T-AMF includes all the Allowed NSSAI in S-AMF but the value of Secondary NSSAI is different, the T-AMF will perform following service continuity process according to the existed relationship and delete the received information from S-AMF;

Existed relationship in T-AMF does not include all the Allowed NSSAI in S-AMF, T-AMF will query NSSF in the following registration procedure after handover complete as described in clause 4.9.1.3.3 of TS 23.502 [5], even if AMF re-allocation is not needed any more, to obtain the latest Secondary NSSAI and Allowed NSSAI related information at UE's current location area.

4:	Rest of the RAN handover procedures.

5, 6:	Similar to the No mobility scenario, AMF triggers the migration of network slices when it finds that the current SLA is not satisfied or the CN load is insufficient as described in clause 6.43.3.2.

### 6.43.4	Impacts on services, entities and interfaces

The following impacts have been identified:

AMF:

-	Support storage of secondary NSSAI information obtained in initial registration procedure;

-	Send original secondary NSSAI information from S-AMF to T-AMF during RAN handover and related processing.

NSSF:

-	Decide value of secondary NSSAI during initial registration procedure.

SMF:

-	Trigger PDU session modification when receiving secondary NSSAI from AMF.

UE, NG-RAN:

-	Handling PDU session modification which related to the secondary NSSAI.

**** End of Change ****