SA WG2 Meeting #153E (e-meeting)	S2-2209324

October 10 – 17, 2022, Elbonia	(revision of S2-2208598r01)

Source:	LG Electronics, Ericsson, Huawei, Intel, vivo, Qualcomm Incorporated

Title:	KI#1, Conclusion update: EN resolution

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper updates conclusion for KI#1 "URSP in VPLMN".

Discussion

Two ENs exist under clause 8.1.3. This paper focuses on resolving the second EN and the related bullet below.

- The H-PCF generate VPLMN specific URSP rules by taking Service Parameters either from V-PCF or the V-AF in. Editor's note: It is FFS which solution is taken for normative specification, H-PCF generating VPLMN specific URSP rules based on service parameters provided by V-PCF or based on URSP guidance provided by the AF from VPLMN for the case when the H-PLMN generates the VPLMN specific URSP rules.

This is proposed by Solution#5 while this is proposed by Solution#1 and Solution#2.

Proposal

It is proposed to agree the following changes into TR 23.700-85v1.0.0.

* * * * Start of 1st Change * * * *

8.1.3	Conclusions on which PLMN determines the VPLMN specific URSP Rules

The scenario where the network provides the UE with URSP rules applicable in the VPLMN It is proposed to adopt the following interim conclusion principles:

-	The H-PCF provides VPLMN specific URSP Rules to the UE.

-	The H-PCF generates VPLMN specific URSP rules by taking Service Parameters from V-PCF or the V-AF into account.

-	The Service Parameters provided by the V-AF are either provided to:

-	V-NEF and stored in V-UDR so that the V-PCF can obtain the Service Parameters from V-UDR.

-	H-NEF and stored in the H-UDR. H-PCF retrieves the service parameters directly from the H-UDR

-	The Service Parameters are AF guidance for URSP Rule determination for subscribers of a certain PLMN.

Editor's note:	It is FFS whether a new re-evaluation trigger (e.g., change of PLMN/Route Selection Validation Criteria or an indication in the URSP Rule) is needed when VPLMN specific URSP Rules or VPLMN specific RSDs are used.

* * * * End of Changes * * * *