SA WG2 Meeting #153E e-meeting	S2-2209563

# 10 - 17 October 2022, Elbonia	(was S2-2209058r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3747 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on conditions for using SPI for UE derived QoS rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When Reflective QoS is enabled, the UE dervies QoS rules following the principles defined in clasue 5.7.5.2. For PDU Session of IP type, the UL Packet Filter may be derived use the SPI of the IPSec SA as described below: In this case, if an uplink IPSec SA corresponding to a downlink IPSec SA of the SPI in the DL packet exists, then the UL Packet Filter contains an SPI of the uplink IPSec SA. However, this condition may not be valid in certain cases, i.e. the uplink IPSec SA may exist, but may not be available to the entity deriving the QoS rules. Normally, UL and DL IPSec SA pairing is handled by the HLOS layer or application layer that creates the IPSec SAs, and is not exposed to the layer managing the QoS rules. According to TS 24.501, the QoS rules are derived at NAS layer. For example, in the following cases, the entity derving the QoS rules cannot obtain the SPI of the uplink IPSec SA: - In the case when the MT and TE are separated, the TE handles the IPSec SAs. The MT that handles the QoS rules does not have the information regarding IPSec SAs and the uplink SPIs. - Even in case of normal UEs, the HLOS layer does not have interface to pass the IPSec SA and SPI informaiton to the NAS layer. Therefore, the condition for using the SPI for the UL Packet Filter needs to be corrected to “when available to NAS layer”. Otherwise, the UE would attempt to create UL Packet Filters with the SPIs, which would cause error and might result in incorrect QoS treatment of the traffic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the clause 5.7.5.2 on conditions for using SPI to derive the UL Packet Fitler to “is known to NAS layer”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Causing error in derived QoS rule generation and interrupt Reflective QoS operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.7.5.2	UE Derived QoS Rule

The UE derived QoS rule contains following parameters:

-	One UL Packet Filter (in the Packet Filter Set as defined in clause 5.7.6);

-	QFI;

-	Precedence value (see clause 5.7.1.9).

Upon receiving DL packet, one UL Packet Filter derived from the received DL packet as described in this clause is used to identify a UE derived QoS rule within a PDU Session.

For PDU Session of IP type, the UL Packet Filter is derived based on the received DL packet as follows:

-	When Protocol ID / Next Header is set to TCP or UDP, by using the source and destination IP addresses, source and destination port numbers, and the Protocol ID / Next Header field itself.

-	When Protocol ID / Next Header is set to UDP, if the received DL packet is UDP-encapsulated IPSec protected packet, by using the source and destination IP addresses, source and destination port numbers, the Security Parameter Index, and the Protocol ID / Next Header field itself. In this case, if an uplink IPSec SA corresponding to a downlink IPSec SA of the SPI in the DL packet exists and the SPI of the uplink IPSec SA is known to the NAS layer , then the UL Packet Filter contains an SPI of the uplink IPSec SA.

-	When Protocol ID / Next Header is set to ESP, by using the source and destination IP addresses, the Security Parameter Index, and the Protocol ID / Next Header field itself. If the received DL packet is an IPSec protected packet, and an uplink IPSec SA corresponding to a downlink IPSec SA of the SPI in the DL packet exists and the SPI of the uplink IPSec SA is known to the NAS layer , then the UL Packet Filter contains an SPI of the uplink IPSec SA.

NOTE 1:	In this Release of the specification for PDU Sessions of IP type the use of Reflective QoS is restricted to service data flows for which Protocol ID / Next Header is set to TCP, UDP or ESP.

NOTE 2:	The UE does not verify whether the downlink packets with RQI indication match the restrictions on Reflective QoS.

NOTE 3:	How to determine the received DL packet is UDP-encapsulated IPSec protected packet is defined in RFC 3948 [138]. UDP encapsulation for ESP is used when a NAT is detected, and there can be different Security Parameter Indexes within the same IP-tuples.

For PDU Session of Ethernet type the UL Packet Filter is derived based on the received DL packet by using the source and destination MAC addresses, the Ethertype on received DL packet is used as Ethertype for UL packet. In the case of presence of 802.1Q [98], the VID and PCP in IEEE Std 802.1Q [98] header(s) of the received DL packet is also used as the VID and PCP field for the UL Packet Filter. When double 802.1Q [98] tagging is used, only the outer (S-TAG) is taken into account for the UL Packet Filter derivation.

NOTE 4:	In this Release of the specification for PDU Sessions of Ethernet type the use of Reflective QoS is restricted to service data flows for which 802.1Q [98] tagging is used.

The QFI of the UE derived QoS rule is set to the value received in the DL packet.

When Reflective QoS is activated the precedence value for all UE derived QoS rules is set to a standardised value.

# >>>>END OF CHANGES<<<<