3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209593

Elbonia, October 10 – 17, 2022	(revision of S2-2208749r01)

Source:	Huawei, HiSilicon

Title:	KI#9, update solution#34 to clarify impacts on service, entities and interfaces

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_ph3/ Rel-18

Abstract: Update solution#34 to clarify impacts on service, entities and interfaces.

1. Introduction/Discussion

This contribution clarify impacts on service, entities and interfaces of solution#34.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

6.34	Solution #34: UE location determination for Mobility Restriction enforcement

6.34.1	Introduction

Using UE-generated location information (e.g. GNSS/A-GNSS) to determine the TAI where the UE is geographically located can be accurate but may be unreliable as has been evaluated by SA WG3.

To perform precise Mobility Restriction enforcement, this solution proposes to use NI-LR procedure to determine the TAI where the UE is geographically located, when NG-RAN reports multiple TAIs to AMF and any but not all of them are Forbidden Area or Non-allowed area.

6.34.2	Functional Description

This solution proposes to utilize NI-LR procedure to determine the TAI where the UE (with satellite access) is geographically located, NG-RAN reports multiple TAIs to AMF (not indicating the TAI in which the UE is located) and any but not all of the reported TAIs are Forbidden Area or Non-allowed area.

6.34.3	Procedure

Figure 6.34.2-1: NI-LR based UE location determination for Mobility Restriction enforcement

1.	AMF receives a NGAP message containing User Location Information for a UE using NR satellite access during a Mobility Management or Session Management procedure. The ULI information contains multiple TAIs.

NOTE 1:	In step 1, NG-RAN does not indicate the TAI in which the UE is located as part of the ULI.

2.	AMF determines to initiate NI-LR procedure if any but not all of the received TAIs in step 1 is Forbidden or Not-Allowed.

3.	AMF sends Nlmf_location_DetermineLocation request to LMF including a UE TAI determination indication.

4.	Based on the indication of step 3, LMF may do not select UE standalone or UE-based positioning mode for UE positioning, or also may restrict the usage of UE-assisted method, which can ensure that network verification of UE location is performed with a reliable method.

NOTE 2:	The positioning method mode selection needs coordination with RAN TSG WG and depends on RAN supported methods.

5.	LMF sends the UE location estimate to AMF via Nlmf_location_DetermineLocation response.

6.	AMF maps the UE location into TAI and performs mobility restriction based on clause 5.3.4.1 of TS 23.501 [2].

NOTE 3:	AMF is configured with TAI geographic information.

# * * * * Second change * * * *

6.34.4	Impacts on services, entities, and interfaces

AMF:

- Triggers NI-LR procedure when NG-RAN reports multiple TAIs (not indicating the TAI in which the UE is located) and any but not all of the reported TAIs are Forbidden Area or Non-allowed area) and any but not all of them are Forbidden Area or Non-allowed area.

- Indicates to LMF in the NI-LR request that the request is for UE TAI determination.

- AMF is configured with TAI geographic information.

LMF:

- May select a reliable positioning method supported by RAN based on the UE TAI determination indication.

RAN:

- NTN-based NG-RAN needs to support reliable positioning methods.

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *