3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209863

Elbonia, October 10 – 17, 2022	(revision of S2-2208316r02)

Source:	Huawei, HiSilicon

Title:	KI#3, Update conclusion on KI#3

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: The conclusion on KI#3 of FS_eNPN_Ph2 about how to enable NPN as hosting network for providing access to localized services.

1. Introduction/Discussion

The contribution would like to further update the conclusion on KI#3 about how to enable NPN as hosting network for providing access to localized services.

Firstly, the negotiation or interaction between the hosting network and the localized service provider is needed for hosting network to provide connectivity with UE to access to the localized services. The information for the negotiation shall be localized service level to enable the hosting network provide corresponding SLA for the localized services.

Secondly, in order to enable the UE to obtain the information for selecting the appropriate hosting network, the interaction between the localized service provider and home network is needed. The information for the negotiation is also localized service level, which shall further include the subscribing UEs so that the home network can be aware of which UEs need to obtain the hosting network information.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

8.3	Key Issue #3: Enabling NPN as hosting network for providing access to localized services

When describing the feature description in the normative phase, the definitions for localized service and localized service provider in clause 3 can be used as basis.

There are different types of interactions between hosting network operator and localized service provider. Depending on the types of the interactions, following principles are proposed to address this key issue:

For interactions between localized service provider and hosting network operator:

The service agreements between hosting network operator and localized service provider can include:

-	list of the identification of the localized services;

-	validity restriction for each localized service, e.g. the validity of time or location;

-	service parameters for each localized service, e.g. DNN, S-NSSAI and QoS requirements;

-	service authorization methods, e.g. NSSAA or PDU Session SAA;

# * * * * End of changes * * * *