SA WG2 Meeting #S2-153E	S2-2209864

10 - 17 Oct, 2022, E-Meeting

Source: 	Ericsson, Qualcomm

Title: 	KI#3: Evaluation and conclusion for Key Issue #3

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an update for evaluation and conclusion of Key Issue #3.

# Discussion

The “KI#3: Enabling NPN as hosting network for providing access to localized services” includes the following description (in italics).

This key issue aims at addressing how to enable a NPN (i.e. a SNPN or a PNI-NPN) as a hosting network for providing access to localized services with the following aspects:

-	Define hosting NPN and identify the difference(s) between hosting NPN and NPN defined in both Rel-16 and Rel-17.

-	Define localized services and identify the difference(s) between localized services and regular services.

-	Define where and when localized services are available based on localized service agreements (i.e. a service agreement for a localized service).

-	What is required to enable communication between a network operator deploying a hosting network and a localized services provider:

-	Investigate which type of interaction (e.g. configuration of the hosting network, information reporting) is needed, in such relation to enable the localized services provider for making the best use of the hosting network; and

This contribution provides updates to the evaluation and conclusion clauses for KI #3 in TR 23.700-08, primarily focusing the evaluation on the interaction between the localized service provider and the hosting network.

R01 removes evaluation as considered merged into 08313.

# Proposal

Add the following evaluation and conclusion to TR 23.700-08.

*** BEGIN FIRST CHANGE ***

## 8.3	Key Issue #3: Enabling NPN as hosting network for providing access to localized services

When describing the feature description in the normative phase, the definitions for localized service and localized service provider in clause 3 can be used as basis.

There are different types of interactions between hosting network operator and localized service provider. Depending on the types of the interactions, following principles are proposed to address this key issue:

Depending on the types of the interactions, following principles are proposed:

-	If the purpose is to configure the hosting network (e.g., creation of network slice/DNN for carrying localized service traffic), existing OAM mechanisms can be re-used as per 3GPP TS 28.557 [x] clause 6.3.1, that provides a solution for NPN provisioning by a network slice of a PLMN and for exposure of management capability of PNI-NPN (clause 6.3.2). The attributes to support this management is further documented in 3GPP TS 28.541 [y].

-	For other types of Session Management level interactions (e.g., monitoring hosting network performance, enabling special QoS for UE in hosting network for localized service, etc.), the following options can be considered:

-	Covered by the SLA.

-	Reuse the existing network exposure procedures (see TS 23.502 [4] clause 4.15).

-	Enable NEF/PCF in the hosting network (via AF of the localized service provider) to receive and forward the validity conditions and QoS requirements of the localized services to the AMF/SMF by reusing the existing PCF initiated AMF/SM policy association procedure.

Conclusions in Key issue #4 and Key issue #5 are sufficient to cover all necessary interactions between home network and hosting network.

It is suggested to re-use existing mechanisms that does not require SA2 normative work.

*** END FIRST CHANGE ***

*** BEGIN SECOND CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[6]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[7]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[8]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[9]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[10]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[11]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[12]	3GPP TR 22.844: "Study on 5G Networks Providing Access to Localized Services".

[13]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[15]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

[16]	3GPP TR 23.700-17: "Study on support for 5WWC; Phase 2".

[17]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[18]	3GPP TR 23.700-85: "Study on enhancement of 5G User Equipment (UE) policy".

[19]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS)".

[x]	3GPP TS 28.557: "Management and orchestration; Management of Non-Public Networks (NPN)".

[y]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM)".

*** END SECOND CHANGE ***