3GPP TSG-SA WG2#153E e-meeting	S2-2209894

Elbonia, 10 – 17 October 2022	(was S2-2209154r13)

Source:	Qualcomm Incorporated, Charter Communications, AT&T

Title:	KI#5: Conclusion update to address EAS discovery

Document for:	Discussion/Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: This contribution finalizes the description of Sol. 22 option 2.

# 1.	Discussion

This document addresses the EN in the conclusion of KI#5 related to EAS discovery

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-48:

# >>>>BEGINNING OF CHANGES<<<<

## 8.5	KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed by the serving Operator’s OP on cloud resources provided by another PLMN or partner (i.e., the Edge Node Sharing Scenario as described in GSMA OPG.02 [X]  the following is recommended as baseline for normative work.

EAS Deployment Provision

The serving PLMN is provided with the information needed to support UEs to discover and connect to the shared EHE deployed in the hosting network. In particular, the serving SMF may either store:

-	the FQDN(s), DNS Server Information, and/or DNAI(s) of the EAS Deployment Information (EDI, see TS 23.548 [x] clause 6.2.3.4-1). Depending on GSMA OPG feedback a Partner ID (e.g., PLMN ID) to identify to which partner that EDI corresponds to may also be included. Such EDI information can be preconfigured in the serving SMF provided or provided by the AF.

-	The serving SMF configures EASDF according to EDI based on Rel-17 methods (see TS 23.548 [3]).

-	The DNS server can be selected by SMF by considering the UE location as described in Rel-17 TS 23.548 [3].

EAS Discovery Procedure

-	The EAS discovery procedure defined in TS 23.548 [3] will be reused.

Traffic transmission between two PLMNs

-	When the traffic is transmitted between two PLMNs, the UPF in the serving PLMN should support mechanisms to facilitate low latency transmission. N6 routing information (as defined in TS23.501 clause 5.6.7) could be included in EDI for routing specific edge.

NOTE:	How to support charging in EHE sharing scenario will be specified by considering GSMA OPG's feedback. Some cooperation with SA5 may be needed.

Edge Relocation

Editor's note:	It is FFS how to support the Edge Relocation in GSMA OPG scenario.

Editor's note:	It is FFS how to support edge relocation between an EAS deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers.

Editor's note:	It is FFS how to support edge relocation between PLMNs.

# >>>>END OF CHANGES<<<<