3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209904

Elbonia, October 10th – 14th, 2022	(merge of S2-2209011 and S2-2208629)

Source:	vivo

Title:	Editorial modification on interim conclusions of KI#2

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes editorial modification on interim conclusions on KI#2.

1. Introduction

This paper proposes editorial modification on interim conclusions on KI#2. The conclusions indicate no 5GC impact.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

## 8.2	Conclusion on Key Issue #2

The following conclusions are agreed, which does not have normative work:

1)	The interfaces for PIN discovery between PINE and PEGC, PINE and PEMC, PEGC and PEMC are divided into two layers: transport layer and application layer.

a)	The transport layer is based on non-3GPP communication or 3GPP PC5 between PEGC and PEMC (which includes direct communication and direct discovery). Transport layer functionality based on non-3GPP communication specification is outside the 3GPP scope. When transport layer is based on 3GPP PC5, the existing procedures defined for 5G ProSe Direct Communication are re-used.

b)	The application layer for PIN and PIN Element discovery and selection is not specified by SA WG2. No normative work is required for this aspect.

2)	The mechanism for discovering of PIN, and PIN elements is based on non-3GPP access discovery method and is not specified by 3GPP.

3)	The procedure of selection of PEMC and PEGC by PINE is implementation specific and therefore it is not specified by 3GPP.

# * * * * End of changes * * * *