SA WG2 Meeting #SA2-153E 	S2-2209908

Oct 10 – 17, 2022, Electronic meeting

Source:	China Mobile, Huawei, HiSilicon, China Telecom, InterDigital, Nokia, OPPO,Samsung, vivo, Ericsson, ZTE

Title:	Evaluation and conclusion for KI1

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes Evaluation and conclusion for KI1

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

8.X	Conclusions for Key issue#1

The following aspects are concluded as principles for the normative work:

For Key issue#1, single UE case,

The following aspects are concluded as principles for the normative work:

- Those data streams that are closely related and require strong application coordination are transmitted in a single PDU session by a single UE. However, those data streams that contribute to the immersive experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs. In order to ensure that the UE selects the correct DNN/S-NSSAI combination for the XRM traffic, the existing URSP Rule evaluation framework can be reused. A traffic descriptor (e.g. an FQDN) for the XRM session will be used during URSP rule.

- The procedure for AF session setup with required QoS, is reused for XRM applications (untrusted AFs) interacting with NEF. However, current Nnef_AFsessionWithQoS service shall be extended to allow the AF to provide information for multiple medias.

  - Normative impact to AF and NEF/PCF: extend the existing Nnef_AFsessionWithQoS service to allow the AF to provide, at the same time, service requirements, alternative service requirements, a common ID and any additional requirements for multiple IP data flows associated to a multi-modal (XRM) application.

Editor’s Note:

Additional impacts are FFS.

Editor’s Note: whether the AF can provide maximum 5GS delay difference threshold to 5GS to guarantee the flows delay difference is FFS.

-	PCF generates policies to support the following:

-	PCF performs the flow authorization.

-	PCF provisions  QoS information considering the requirements  provided by the AF for all data flows associated to a multi-modal (XRM) application..

-	PCF enforces the policy for the use of Alternative QoS parameters.

-	These policies above are enforced only according to the AF provided explicit requirements.

Editor’s Note: 	The details on how the PCF enforces the flow admission, QoS fulfilment and alternative QoS profiles are FFS.

Editor’s Note: 	Whether the PCF sends the policy information to SMF/NG-RAN, and Whether the NG-RAN should support the additional policies and how NG-RAN uses them is FFS.

# * * * *end of change * * * *