SA WG2 Meeting #153e	S2-2209954

# October 10th – 17th, 2022; Elbonia              	(revision of S2-2208222r08)

Revision of S2-2208222r08

+

Changes made between “start of changes” and “second change” are excluded.

Add this sentence to the conclusion.

“Normative work will be based on NEF contacting NRF and UPF before invoking BSF”

Source: 	Nokia, Nokia Shanghai Bell, DISH Network

Title: 	Update to the solution 20 (UE IP address mapping information exposure by UPF)

Document for: 	Approval

Agenda Item: 	9.25.1

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Update to the solution 20 (UE IP address mapping information exposure by UPF)

# 1	Discussion

The EN on exposing DNN and/or S-NSSAI OR IP domain is removed as this information is not exposed more than what is defined in R17 specifications, taking e.g. 23.502 Figure 4.15.10-1: AF specific UE ID retrieval which is by the way the baseline of this solution

1.	AF requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message shall include UE address (IP address or MAC address) and AF Identifier, it may include, MTC Provider Information, Application Port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application. If available the AF may also provide the corresponding DNN and/or S-NSSAI.

Following EN is removed as the AF is not meant to keep IP address binding information

Editor's note:	It is FFS how long the binding information can be kept at the AF.

##### Please Note that 29.510 Table 6.1.6.2.21-1: “Definition of type BsfInfo” defines the following

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of IPv4 addresses handled by BSF. If not provided, the BSF can serve any IPv4 address. |
| dnnList | array(Dnn) | O | 1..N | List of DNNs handled by the BSF. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the BSF can serve any DNN. |
| ipDomainList | array(string) | O | 1..N | List of IPv4 address domains, as described in clause 6.2 of 3GPP TS 29.513 [28], handled by the BSF. If not provided, the BSF can serve any IP domain. |

# 2 Proposal

It is proposed to update TR 23700-62 as follows

change

## 8.2	Conclusions for KI#2

The following interim conclusions are proposed for KI#2:

1.	Subscription to UPF events via SMF is the rule except for the cases listed in bullet 2; Subscription via SMF means the final consumer of UPF event notifications sends the subscription request to the SMF and then the SMF is doing a third-party subscription onto UPF on behalf of this final consumer. Conversely the notifications are directly sent by the UPF to the final consumer of UPF notifications.

NOTE 1:	Optimizing notifications is more important than optimizing subscriptions.

NOTE 2:	Subscriptions related with AoI are handled by SMF that subscribe/unsubscribe to the relevant UPF(s) on behalf of the final consumer based on whether the UE is in the target AoI. This allows the UPF not having to determine the AMF where to subscribe for UE presence in the AoI.

NOTE 3:	For event subscriptions requiring interactions with 5G AN, a solution where the UPF event consumer would directly subscribe to UPF and then UPF would ask SMF to send N2 SM signalling to 5G AN would be more complex and not bring advantage.

2.	Direct subscription to UPF (i.e. not requiring third party subscription to UPF via SMF) shall be possible for data collection where UPF is the source as defined in TS 23.288 [5], i.e. the following cases:

A.	TS 23.288 [5] Table 6.5.2-2: Data collected by NWDAF for UPF load analytics recalled in item 2 of Annex A of the TR.

B.	For analytics targeting "any UE" (possibly for specific DNN and or slices) and not related with an AoI or with a specific data flow.

C.	The case described in solution 20 (UE IP address mapping information exposure by UPF) where the 5GC NF (e.g., NEF, BSF) can directly discover the UPF that performs NAT (based on the NATed IP address) and then invoke directly the NAT related API at the UPF. This is an optional feature.

Normative work will be based on NEF contacting NRF and UPF before invoking BSF

NOTE X:	Which 5GC NF can directly discover the UPF and invoke the NAT related API will be confirmed during normative phase.

NOTE 4:	This can relate to use cases such as Data collected by NWDAF for UPF load analytics, User Data Congestion Analytics, Data Volume dispersion analytics, WLAN performance analytics.

Editor's note:	Whether other direct subscriptions are possible is FFS.

3.

Editor's note:	When a SMF is doing a third-party subscription on behalf of the final consumers of UPF notifications, it is FFS whether SBI and/or PFCP (TS 29.244 [8]) is used for interaction with UPF.

4.	In Rel18:

A.	the only defined consumers of UPF event SUBSCRIBE are SMF, and NWDAF.

B.	the only defined consumers of UPF event notifications are AF/NEF, TSNAF/TSCTSF and NWDAF.

Editor's note:	Whether AF/NEF can directly subscribe to UPF is FFS.

5.	UPF event exposure Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on UPFs.

The following events may be subscribed by a NF consumer:

-	Event: QoS monitoring. This event provides QoS Flow level performance information (information listed in Solution #8, clause 6.8.2).

-	Event: UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session (information listed in Solution #7, clause 6.7.2).

-	Event: UserDataUsageTrends. This event provides statistical measurements (information listed in Solution #7, clause 6.7.2).

Editor's note:	The following part is FFS "Both UserDataUsageMeasures and serDataUsageTrends events provide measurement context (for example, time stamps for the packets and the measures) and information of the PDU Session. When the information refers to an application, the Application Id or Packet Filter Set is included".

6.	To determine which SMF to contact the final consumer of UPF events proceeds as follows:

-	If the event targets any UE, the final consumer of UPF events looks up the NRF to discover all suitable SMF(s) (e.g. SMF(s) that serve the target combination of DNN and S-NSSAI).

-	If the event targets a unique UE identified by its SUPI, the final consumer of UPF events sends Nudm_UECM_Get_Request (SUPI, type of requested information set to SMF Registration Info and the S-NSSAI and DNN) to UDM to get the SMF ID serving the target UE.

Editor's note:	If the consumer of NWDAF service doesn't provide the necessary parameters to NWDAF, for example, the DNN, S-NSSAI (these parameters are optional for NWDAF service defined in TS 23.288 [5]), how to discover the SMF in any UE situation without these parameters is FFS.

7.	For the UPF data collection, the event subscription includes Reporting suggestion information as described in Sol#14, which is used to assist the UPF event notification. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.