SA WG2 Meeting #153E	S2-2209983

10 - 17 October 2022, Elbonia	Revision of S2-2208993

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution #38 evaluation and conclusions for key issue #6

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution provides proposal for solution #38 evaluation as well as related conclusions.

# 1		Discussion

This contribution provides proposal for solution #38 evaluation as well as related conclusions.

It is proposed to update FS_AIMLsys TR 23.700-80 as below.

*** 1st Change ***

### 7.6.3	Other topics

Solution #38 proposes that AF provides time dependency information to the NEF in addition to existing QoS requirements when requesting to setup an AF session. Alternatively, the AF provides traffic pattern information which are converted into QoS profiles and time information by a dedicated FL assistance function (as proposed also in solution #23). Then PCF derives PCC rules with QoS applicability times, based on different QoS profiles with different time information received either from NEF or from FL assistance function, and provides these PCC rules to the SMF. The SMF then determines QoS profiles and provisions QoS profiles with QoS applicability time to the NG-RAN and time dependent PDRs to the UPF, and NG-RAN then schedules time dependent QoSs.

Editor's note:	Evaluation would need updates after SA2#152E progress, potential updates to the related solutions.

*** End of Changes ***