3GPP TSG SA WG2 Meeting #154-AHe                                                        	S2-2301018

e-Meeting, 16th – 20th January, 2023

Source:	Samsung

Title:	Discussion on not allowed to act as MBSR handling.

Document for:	Discussion

Agenda item:	VMR

# 1. Introduction

In last couple of meetings, the SA2 WG could not conclude on how to indicate MBSR that it is not allowed to act as MBSR. This paper discusses different possibilities and proposes a way forward.

# 2. Discussion

When registration is triggered by the UE, the MBSR support indication is provided by UE to the NG-RAN and it is propagated to AMF. The AMF uses this information to decide, based on subscription information and local policy, whether UE should act as MBSR. There are two options with the AMF if it deems UE cannot act as MBSR, it can send:

a)	Registration accept to the UE; or

b)	Registration reject to the UE.

In our view, in this situation, it is ok to have registration reject as an option for network to keep UE in unregistered state. We believe it is not reasonable to have no option for the network to keep the UE in registered state due to below reasons:

a)	The UE should cross all the subscription/authentication checks and should be capable of acting as a normal UE otherwise existing reject causes would trigger to keep the UE unregistered anyways. Thus there is no need to do an extra check of “allowed to act as MBSR” to keep the UE in unregistered state by providing registration reject message.

b)	If subscription status change for the UE, UE can be notified if it is in the registered state, so that it can start acting as MBSR otherwise it would need a manual intervention (which is not desirable).

c)	The aim of the IAB-node is to mount base-station like operation on the normal UE, thus it looks reasonable for UE to receive normal services based on its available subscriptions irrespective of whether it has MBSR subscription or not. In our view there is no need to create strong coupling between MBSR subscription and other subscriptions of the UE.

To keep the UE in registered state there are two options with AMF:

1)	the AMF provides registration accept message to the UE(based on current protocol – no changes required), additionally indicate to NG-RAN that MBSR-UE is not allowed to act as MBSR and NG-RAN propagates this information to the UE.

2)	AMF indicates to the UE in registration accept message that MBSR-UE is not allowed to act as MBSR.

In our understanding, “1” is rel-17 behaviour already available for IAB-node. Thus, “1” is proposed as part of the CR  S2-2301015

# 3. Conclusion & proposal

In our understanding, “1” is rel-17 behaviour already available for IAB-node. Thus, option “1” is proposed as part of the CR S2-2301015.

# 4. Reference

NA.