SA WG2 Meeting ##154AH-e	S2-2300030

16 - 20 January, 2023, Electronic Meeting

3GPP TSG-CT3 Meeting #125	C3-225374

# Toulouse, France, 14th - 18th November 2022

# Title:	LS on dedicated DNN for 5G ProSe L3 UE-to-Network Relay connectivity

# Response to:	-

# Release:	Rel-17

# Work Item:	5G_ProSe

Source:	CT3

To:	SA2

Cc:

Contact Person:

Name:	Fuencisla Garcia

E-mail Address:	fuencisla.garcia@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Per TS 23.304 clause 5.4.1.3 (see excerpt below), for 5G ProSe Layer-3 UE-to-Network Relay connectivity, the UE identity provided by the AF and the UE identity provided by the SMF can be different, therefore the UE identity shall be ignored for session binding.

NOTE 2:    For 5G ProSe Layer-3 UE-to-Network Relay connectivity, the UE identity that the SMF has provided (i.e. 5G ProSe Layer-3 UE-to-Network Relay Identity) and a UE identity provided by the AF (i.e. 5G ProSe Layer-3 Remote UE Identity) can be different, while the Session binding with the PDU Session is valid.

-	For a PDU Session to the dedicated DNN for 5G ProSe Layer-3 UE-to-Network Relay connectivity and using IPv6 prefix delegation (i.e. the assigned IPv6 network prefix is shorter than 64), the PCF shall perform session binding based on the IPv6 network prefix only. A successful session binding occurs whenever a longer prefix received from an AF matches the prefix value of the PDU Session. PCF shall not use the UE Identity for session binding for this PDU Session.

TS 23.304 clause 5.4.1.3 also states that the SMF and PCF may be configured with a dedicated DNN for 5G ProSe L3 UE-to-Network Relay connectivity, see below:

To enable support for policy control for 5G ProSe Layer-3 Remote UEs accessing 5GC via a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the policy control functionality specified in TS 23.503 [9] is applied with the following functionalities:

-	The 5G ProSe Layer-3 UE-to-Network Relay, the SMF and PCF may be configured with a dedicated DNN for UE-to-Network Relay connectivity.

The above implies that dedicated DNN may or may not be configured for the 5G ProSe L3 UE-to-Network Relay connectivity:

-	For the case that dedicated DNN is configured, TS 23.304 specified that the PCF shall ignore UE Identity in session binding.

-	However, for the case that dedicated DNN is not configured, it is not clear how the PCF can determine to ignore the UE Identity in session binding.

Therefore, CT3 would like to ask SA2 the following questions:

Q1: Would it be possible to mandate a dedicated DNN for 5G ProSe L3 UE-to-Network Relay connectivity so that the PCF can determine to ignore UE identity in session binding based on the configured DNN?

Q2: If the answer to Q1 is NO, how can PCF determine to ignore the UE Identity in session binding?

2. Actions:

To SA2:

ACTION: 	CT3 kindly asks SA2 to answer the questions.

3. Date of Next CT3 Meetings:

3GPP TSG CT3#126	2023-02-27 – 2023-03-03	Athens, GR

3GPP TSG CT3#127e	2023-04-17 – 2023-04-21	Electronic