3GPP TSG-SA WG2#154-AH-E	S2-2300125

# Electronic, 16 – 20 January, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0797 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Experience Analytics ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 01/09/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Analytics ID on “Service Experience” may provide different values depending on the PDU Session that transfers the service data, However the existing description refers to Route Selection Components instead. The NWDAF does not collect input data per RSC companent but collects input data for the PDU Session instead. See Table 6.4.2-2 that lists the PDU Session parameters reported by SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remoalof Editor's note: It is FFS how to extend Service Experience Analytics to request analytics for the different combinations of RSC values and which RSC combinations are applicable. Clarification on the Analytics request and the different parameter combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of the Service Experience Analytics ID |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# First change

#### 6.1.1.3	Policy decisions based on network analytics

Policy decisions based on network analytics allow PCF to perform policy decisions taking into account analytics information defined for Analytics IDs listed in TS 23.288 [24]. The analytics information may be provided by NWDAF directly or via DCCF, depending on the deployment of NWDAF or DCCF. Local configuration in the PCF indicates if one or multiple or all Analytics ID(s) are retrieved either from NWDAF directly or using DCCF. The PCF uses the DCCF services and DCCF service operations to fetch, subscribe and unsubscribe to the Analytics IDs as described in clause 6.1.4 and clause 8 in TS 23.288 [24].

The PCF performs discovery and selection of NWDAF and DCCF as defined in TS 23.501 [2] and subscribes/unsubscribes to Analytics information as defined in TS 23.288 [24]. In addition, the AMF and/or SMF may include, in the AM/SM Policy Association establishment or modification procedures, the list of NWDAF instance IDs used for the UE or the PDU Session and their associated Analytics ID(s) consumed by the AMF or SMF respectively. The PCF may select those NWDAF instances as the ones to subscribe for their associated Analytics ID(s) for the UE for which those AM/SM Policy Associations are related to or may perform NWDAF discovery if the NWDAF for an Analytics ID not provided by the AMF or SMF is needed.

The following Analytics IDs are relevant for Policy decisions: "Load level information", "Service Experience", "Network Performance", "Abnormal behaviour", "UE Mobility", "UE Communication", "User Data Congestion", "Data Dispersion", "Session Management Congestion Control Experience", "DN Performance" and "WLAN performance". The PCF may subscribe to NWDAF as described below or alternatively, the PCF may use Ndccf_DataManagement_Subscribe including the "Analytics Specification" with the same information as provided in the Nnwdaf_AnalyticsSubscription_Subscribe, and optionally the PCF may include the NWDAF ID, e.g. if provided by AMF or SMF:

-	The PCF may subscribe to notifications of network analytics related to "Load Level Information" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Load level information", the Analytics Filter "S-NSSAI" and the Analytics Reporting Information set to a load level threshold value. The PCF is notified when the load level of the Network Slice Instance reaches the threshold.

The NWDAF service to retrieve the Load Level Information is described in clause 6.3 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "Service Experience" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Service Experience", the Target of Analytics Reporting "SUPI", "Internal Group Id" or "any UE", the Analytics Filter including one or more Application Identifier(s), one or more or "any" RAT Type(s) or Frequency value(s), and the Analytics Reporting Information set to service experience threshold value(s) for the RAT Type(s) and/or Frequency value(s). The PCF is notified on the Service Experience statistics or predictions including, for each Application Identifier, the list of SUPIs for which Service Experience is provided and the list of RAT Types and/or Frequency values for which the Service Experience applies. In addition, both spatial and time validity may be provided..

-	The PCF may subscribe to notifications of network analytics related to "Service Experience" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Service Experience", the Target of Analytics Reporting "SUPI", "Internal Group Id" or "any UE", the Analytics Filter including one or more Application Identifier(s), one or more list of combination of (S-NSSAI, DNN, PDU Session type and SSC Mode) optionally per Access Type. The PCF is notified on the Service Experience statistics or predictions for each Application Identifier when routing traffic over PDU Session with those lists of combinations possibility over different Access Types. In addition, the list of SUPIs for which Service Experience is provided is also added when the Target of Analytics Reporting is "Internal Group Id" or "any UE". Both spatial and time validity conditions for the “Service Experience” may be provided.

The NWDAF service to retrieve the service experience (i.e. the average observed Service MoS) is described in clause 6.4 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "Network Performance" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Network Performance", the Target of Analytics Reporting "Internal Group Id" and the Analytics Filter including the Area of Interest. The PCF is notified on the Network Performance statistics or predictions including the Area of Interest. In addition, the confidence of the prediction may be provided.

The NWDAF services to retrieve "Network Performance" as described in clause 6.6 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "Abnormal behaviour" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Abnormal behaviour", the Target of Analytics Reporting "SUPI", "Internal Group Id" or "any UE" and the Analytics Filter including the expected analytics type or the list of Exceptions IDs and per each Exception Id a possible threshold and other Analytics Filter Information if needed. The list of Exception IDs is specified in TS 23.288 [24].

The NWDAF services to retrieve "Abnormal behaviour" analytics are described in clause 6.7.5 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "UE Mobility" using NWDAF_AnalyticsSubscription_Subscribe service operation including the Analytics ID "UE Mobility", the Target of Analytics Reporting "SUPI", "Internal Group Id" and the Analytics Filter may include one or more "Area(s) of Interest". The PCF is notified on the UE Mobility statistics or predictions as defined clause 6.7.2 of TS 23.288 [24].

The NWDAF services to retrieve "UE Mobility" analytics are described in clause 6.7.2 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "UE Communication" using NWDAF_AnalyticsSubscription_Subscribe service operation including the Analytics ID "UE communication", the Target of Analytics Reporting "SUPI", "Internal Group Id" and the Analytics Filter may include one or more "Application Identifier(s)". The PCF is notified on the UE communication statistics or predictions including list of application(s) in use and corresponding characteristics, e.g. start time and duration time. In addition, the confidence of the prediction may be provided.

The NWDAF services to retrieve "UE Communication" analytics are described in clause 6.7.3 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "User Data Congestion" using NWDAF_AnalyticsSubscription_Subscribe service operation including the Analytics ID "User Data Congestion", the Target of Analytics Reporting containing a SUPI, indication requesting the identifiers of the applications that contribute the most to the traffic and the Analytics Filter may include Area of Interest, reporting threshold and maximum number of applications to be reported. The PCF is notified when the congestion level reaches the threshold. The notification can include the identifiers of the applications that contribute the most to the traffic.

The NWDAF services to retrieve "User Data Congestion" analytics are described in clause 6.8 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "Data Dispersion" using NWDAF_AnalyticsSubscription_Subscribe service operation including the Analytics ID "UE Dispersion Analytics" and the dispersion analytic (DA) type, i.e. Data or Transactions. The Target of Analytics Reporting containing "SUPI", "Internal Group Id" or "any UE", and the Analytics Filter may include a list of TA(s) or an Area of Interest, or a list of Cells, or an S-NSSAI or top heavy users. With the Data Volume Dispersion Analytics type, the PCF may calculate the average data rate in the network slice by subscribing to notifications of network analytics related to Data Volume Dispersion in the network slice for a duration of interest when it sets the Target of Analytics Reporting as "any UE" and the Analytics Filter as the S-NSSAI.

The NWDAF services to retrieve "Data Dispersion" analytics are described in clause 6.10 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "WLAN performance" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "WLAN performance", the Target of Analytics Reporting "SUPI", "Internal Group Id" or "any UE" and the Analytics Filter including the Area of Interest, SSID(s), or BSSID(s). The PCF is notified on the WLAN performance statistics or predictions. In addition, the confidence of the prediction may be provided.

The NWDAF services to retrieve "WLAN performance" analytics are described in clause 6.11 of TS 23.288 [24].

Editor's note:	Whether the PCF needs to subscribe to any new AnalyticsID for the purpose of negotiation of the planned data transfer with QoS request is FFS.

-	The PCF may subscribe to notifications of network analytics related to "Session Management Congestion Control Experience" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "Session Management Congestion Control Experience", the Target of Analytics Reporting containing "SUPI" and the Analytics Filter may include DNN and/or S-NSSAI. The PCF is notified on the Session Management Congestion Control Experience statistics including the DNN and/or S-NSSAI.

The NWDAF services to retrieve "Session Management Congestion Control Experience" analytics are described in clause 6.12 of TS 23.288 [24].

-	The PCF may subscribe to notifications of network analytics related to "DN Performance" using the Nnwdaf_AnalyticsSubscription_Subscribe service operation including the Analytics ID "DN Performance", the Target of Analytics Reporting containing "SUPI", "Internal Group Id" or "any UE", and the Analytics Filter may include Application ID(s). The PCF is notified on the DN Performance statistics or predictions including Application ID(s). In addition, the confidence of the prediction may be provided.

The NWDAF services to retrieve "DN Performance" analytics are described in clause 6.14 of TS 23.288 [24].

Editor's note:	How to PCF consume analytic of Redundant Transmission Experience from NWDAF is FFS.

NOTE:	Care needs to be taken with regards to signalling and processing load caused when requesting analytics targeting "Any UE". A PCF preferably limits the analytics requests to a smaller UE set to reduce the load.

Possible triggers for the PCF to subscribe to analytics information from the NWDAF may include:

-	Requests from AF/NEF;

-	AM Policy Association establishment or modification request from the AMF;

-	UE Policy Association establishment or modification;

-	SM Policy Association establishment or modification request from the SMF;

-	Notifications received from UDR or CHF on UE subscription change;

-	Analytics information received.

The trigger conditions may depend on operator and implementation policy in the PCF. When a trigger condition happens, the PCF may check local configuration or evaluate operator policy to decide if any analytics information is needed and if so, initiate a subscription to the analytics information from the NWDAF directly or via DCCF, if deployed.

The PCF may, upon a request from AF/NEF for negotiation for future background data transfer, subscribe to the network analytics related to "Network Performance" from the NWDAF directly or via DCCF, if deployed to assist in determination of BDT policies as described in clause 6.1.2.4.

The PCF may, upon AM or UE or SM Policy Association establishment or modification request from the AMF or SMF, or based on notifications received from UDR or CHF on UE subscription change, decide that network analytics related to "Abnormal behaviour", "UE Mobility" or "UE Communication" of the UE, "Session Management Congestion Control Experience", "DN Performance", "User Data Congestion" is needed for policy decision and therefore subscribe to notifications of network analytics related to "Abnormal behaviour", "UE Mobility" or "UE Communication" of the UE from the NWDAF.

The PCF may, upon reception of analytics information, subscribe to other analytics information from the NWDAF directly or via DCCF, if deployed.

The PCF may use the network analytics information as input to its policy decision to apply operator defined actions for session management related policy control (as described in clauses 6.1.3), non-session management related policy control (as described in clause 6.1.2) and network slice related policy control (as described in clause 6.1.4.

Examples of operator policies including network analytics information as inputs for policy decisions included below:

-	Based on the notification of "Load Level Information" of the Network Slice Instance, the PCF may verify if the RFSP index value needs to be modified for a SUPI for which an AM Policy Association is created; this is based on operator policies in the PCF, as defined in clause 6.1.2.1.

-	Based on the "Service Experience" statistics or predictions, the PCF may check the 5QI values assigned to the Application, and may use this as input to calculate and update the authorized QoS for a service data flow template.

-	The PCF may use the network analytics related to "Network Performance" as input to calculate the background data transfer policies that are negotiated with the ASP, as defined in clause 6.1.2.4.

-	Based on the UE mobility statistics or predictions, the PCF may adjust Service Area Restriction as defined in clause 6.1.2.1.

-	The PCF may use the network analytics related to "Unexpected UE location" as input to determine the Service Area Restrictions defined in clause 6.1.2.1, "Suspicion of DDoS attack" or "Too frequent Service Access" to request the SMF to terminate the PDU Session as defined in clause 6.1.3.6, "Wrong destination address" to perform gating of a service data flow as defined in clause 6.1.3.6 and "Unexpected long-live/large rate flows" to perform QoS related policies such as gating or policing as defined in clause 6.2.1.1.

-	Based on the WLAN performance statistics or predictions, the PCF may update WLANSP as defined in clause 6.1.2.2.1.

-	Based on the "User Data Congestion" statistics or predictions including the list of applications contributing the most to the traffic the PCF may perform SM Policy Association modifications to update policies in the SMF for the PDU sessions handling traffic from those applications.

-	The PCF may use the network analytics on "Service Experience" for an Application Identifier, "any RAT type" and/or "any Frequency value" to determine the RFSP Index value for running this application, as described in clause 6.1.2.1.

-	The PCF may use the network analytics on "Service Experience" for an Application Identifier running over a PDU Session with PDU Session parameters set to a combination of (DNN, S-NSSAI, PDU Session Type and SSC mode alues) optionally per Access Type, to determine what values of the RSC parameters will be included in the RSD of the URSP Rules and to determine the precedence of the RSD components in the URSP Rule with the Traffic Descriptor that contains the Application Identifier.

-	The PCF may also use the network analytics as input to its policy decision to apply operator defined actions for example for the UE context(s) or PDU Session(s).

Editor's note:	Which AnalyticsID the PCF subscribes to for the purpose to determine the recommended time window for the planned data transfer with QoS request is FFS.

-	Based on the notification of "Load Level Information" for network slice(s), the PCF may give priority to consider the one with lowest load for an application, and the PCF may update URSP on Network Slice Selection Policy.

-	Based on the "UE communication" statistics or predictions, the PCF may consider the DNN in Traffic characterization and associated Traffic Volume, Spatial validity and inactivity time, and the PCF may update URSP on DNN Selection Policy for associated UEs.

-	Based on the "Dispersion Analytics" statistics or predictions for Data Volume Dispersion in network slice(s), the PCF may calculate the average data rate in the network slice, and the PCF may update URSP on Network Slice Selection Policy for the associated UEs.

-	Based on the "Session Management Congestion Control Experience" statistics or predictions for PDU Session(s) associated with corresponding S-NSSAI(s) or DNN for a UE, PCF may give priority to consider the S-NSSAI or the DNN that provides the lowest experience level of Session Management Congestion Control, and then the PCF may update URSP on Network Slice Selection Policy and/or DNN Selection Policy for the UE.

-	Based on the "Network Performance" statistics or predictions on gNB status information, gNB resource usage, communication performance and mobility performance in an Area of Interest, the PCF may consider to offload the traffic to non-3GPP access for the number of UEs that are located in that Area of Interest, and the PCF may update URSP on Non-Seamless Offload Policy for associated UEs.

-	Based on the "User Data Congestion" statistics or predictions including the list of applications contributing the most to the traffic, the PCF may consider to offload the traffic to non-3GPP access for those applications, and the PCF may update URSP on Non-Seamless Offload Policy for those applications.

-	Based on the "DN Performance" statistics or predictions for user plane performance for an application, the PCF may consider the DNN with higher performance, and the PCF may update URSP on DNN Selection Policy for the application.

Examples of operator policies including combination of multiple network analytics as inputs for policy decisions are included below:

-	Based on the notification of application(s) in use, provided by "UE Communication" analytics, the PCF may request the "Service Experience" analytics (optionally per RAT Type and/or per Frequency) for each application in use as defined in the list of examples of operator policies that may include network analytics as input for a policy decision.

-	Based on the notification of "User Data Congestion", the PCF may further request the NWDAF directly or via DCCF, if deployed, to report the "Data Dispersion Analytics" of either a UE or just the Top Heavy UEs located at the congested area of interest. To mitigate the reported or predicted congestion at the area of interest, the PCF may perform:

-	AM Policy Association modification to update UE-AMBR, RFSP index and/or service area restriction, for those UEs reported as heavy users.

-	SM Policy Association modification to update the policies in the SMF for those UEs reported as heavy users.

-	UE Policy Association establishment/modification: Based on the Load level information", "Service Experience", "Network Performance", "Abnormal behaviour", "UE Mobility", "UE Communication", "User Data Congestion", "Data Dispersion", "Session Management Congestion Control Experience", "DN Performance", "User Data Congestion" and "WLAN performance" statistics or predictions, the PCF uses analytics results from NWDAF to select proper values of URSP rules provided to UEs.

-	Based on the notification of Spatial validity for application(s) in use, provided by "UE Communication" analytics, the PCF may request the "WLAN performance" analytics for the Area of Interest derived from the Spatial validity of "UE Communication" analytics, and the PCF may update URSP on Non-Seamless Offload Policy for associated UEs.

# * * * * End of changes * * * *