3GPP TSG-SA2  Meeting #154AHe  	S2-2300129

# January 16 – 20, 2023	(revision of S2-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3664 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Support for HPLMN admission mode while Roaming. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfill the GSMA requirement to enable an HPLMN to handle admission related to number of registered UEs and the number of LBO PDU sessions for its outbound roamers in any VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support two modes to fulfill GSMA requirement: First mode, HPLMN delegated model where HPLMN delegates to VPMLN admission with the actual quota information fetched by the VPLMN from the HPLMN. HPLMN can update the quota information real-time. Additionally VPLMN sends any additional requests beyond the quota for the HPLMN to decide on their handling, with the result conveyed to the VPLMN. In the second mode, all admission requests are sent to the HPLMN for enforcement and handling. Both modes apply to centralized and hierarchal NSACF architectures. In addition to the above, the Release 17 VPLMN admission mode is enhanced to enable the VPLMN to fetch the quota to be administered by the VPLMN to avoid configuring the VPLMN with the information. This also enables the HPLMN to update this information real-time. Real-time update of quota information by HPLMN is achieved through the use of the Subscribe/Notify NSACF framework extended with a new event. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Support for roaming as required by eNS-PH3 for KI#4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.5.1, 4.2.11.5.2, 5.2.21.4, 4.2.11.5.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 4.2.11.5.1	Network Slice Admission Control Support for Roaming by VPLMN

For NSAC for roaming UEs, a maximum number of allowed UEs per mapped S-NSSAI in HPLMN and/or a maximum number of allowed PDU Sessions in LBO mode per mapped S-NSSAI in HPLMN is allocated to the VPLMN for each S-NSSAI in HPLMN and stored in one NSCAF in the VPLMN responsible for NSAC for the S-NSSAI in the HPLMN, subject to NSAC.

The maximum number of UEs registered with a network slice monitoring and enforcement is done in the VPLMN by the NSACF in the VPLMN as per the description in Figure 4.2.11.2-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfUEsUpdate_Request service operation the AMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the VPLMN.

-	Step 3, the NSACF in the VPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN based on the SLA between VPLMN and HPLMN.

For LBO, enforcement of the maximum number of PDU Sessions established for an S-NSSAI is performed in the VPLMN by the NSACF in the VPLMN as per the description in Figure 4.2.11.4-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfPDUsUpdate_Request service operation the V-SMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the VPLMN.

-	Step 3, the NSACF in the VPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN based on the SLA between VPLMN and HPLMN.

NSACF can optionally subscribe to HPLMN to fetch the maximum number of registered UEs to be enforced, and the maximum number of LBO PDU sessions to be enforced rather than have them pre-configured. The NSACF in the VPLMN also receives the applicable admission mode, VPLMN, VPLMN assisted by HPMLN, or HPLMN, during the subscription by NSACF in VPLMN to acquire the quota to be enforced. In this case, the following is performed:

-	For a centralized architecture in the VPLMN, the NSACF, being a centralized NSACF subscribes to the home central or primary NSACF to be notified of the maximum number of registered UEs, and the maximum number of LBO PDU sessions to be enforced.

-	For a hierarchal architecture in the VPLMN, the NSACF subscribes to the primary VPLMN NCAF to be notified of the maximum number of registered UEs, and the maximum number of LBO PDU sessions to be enforced. The VPLMN primary NSACF in turn, subscribes to the home central or primary NSACF to be notified of the maximum number of registered UEs, and the maximum number of LBO PDU sessions to be enforced; this information in turn is sent to the VPLMN NSACF.

The VPLMN NSACF discovers the HPLMN primary or central NSACF or optionally be configured with the needed information.

For PDU sessions in the home-routed roaming case, the SMF in HPLMN performs NSAC for the S-NSSAI(s) subject to NSAC.

##### 4.2.11.5.2	Network Slice Admission Control Support for Roaming by HPLMN

To enable an NSACF in the VPLMN to distinguish VPLMN admission mode vs HPLMN delegation to VPLMN mode, NSACF in VPLMN receives the applicable mode during the subscription by NSACF in VPLMN to acquire the quota to be enforced.

4.2.11.5.2.1 VPLMN with HPLMN assistance NSAC admission

For NSAC for roaming UEs, a maximum number of allowed UEs per mapped S-NSSAI in HPLMN and/or a maximum number of allowed PDU Sessions in LBO mode per mapped S-NSSAI in HPLMN is allocated and delegated to the VPLMN for each S-NSSAI in HPLMN subject to NSAC. The information is stored in one NSCAF in the VPLMN responsible for NSAC for the S-NSSAI in the HPLMN, subject to NSAC.

Monitoring and enforcement for the maximum number of UEs registered with a network slice monitoring is done by the NSACF in the VPLMN as per the description in Figure 4.2.11.2-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfUEsUpdate_Request service operation the V-AMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the VPLMN.

-	Step 3, the NSACF in the VPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN.  If the maximum number of registered UEs to be enforced is not available in the NSACF in the VPLMN, the following is performed:

-	For a centralized architecture in the VPLMN, the NSACF, being a centralized NSACF subscribes to the home central or primary NSACF to be notified of the maximum number of registered UEs to be enforced.

-	For a hierarchal architecture in the VPLMN, the NSACF subscribes to the primary VPLMN NCAF to be notified of the maximum number of registered UEs to be enforced. The VPLMN primary NSACF in turn, subscribes to the home central or primary NSACF to be notified of the maximum number of registered UEs to be enforced; this information in turn is sent to the VPLMN NSACF.

If the maximum number of Registered UEs have been reached, the NSACF in VPLMN forwards the request to the HPLMN for a decision. The NSACF contacts the same node as described above for acquiring the quota information. The appropriate NSCAF in HPLMN provides the final decision, which is conveyed back to the NSACF in the VPLMN. If admission is allowed or not granted normal processing is followed as described in the step.

For LBO PDU sessions, enforcement of the maximum number of PDU Sessions established for an S-NSSAI is performed by the NSACF in the VPLMN as per the description in Figure 4.2.11.4-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfPDUsUpdate_Request service operation the V-SMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the VPLMN.

-	Step 3, the NSACF in the HPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN based on the SLA between VPLMN and HPLMN. If the maximum number of LBO PDU sessions is not available in the NSACF in the VPLMN, the following is performed:

-	For a centralized architecture in the VPLMN, the NSACF, being a centralized NSACF subscribes to the home central or primary NSACF to be notified of the maximum number of LBO PDU sessions to be enforced.

-	For a hierarchal architecture in the VPLMN, the NSACF subscribes to the primary VPLMN NCAF to be notified of the maximum number LBO PDU sessions to be enforced. The VPLMN primary NSACF in turn, subscribes to the home central or primary NSACF to be notified of the maximum number of LBO PDU sessions to be enforced; this information in turn is sent to the VPLMN NSACF.

If the maximum number of LBO PDU sessions have been reached, the NSACF in VPLMN forwards the request to the HPLMN for a decision. The NSACF contacts the same node as described above for acquiring the quota information. The appropriate NSCAF in HPLMN provides the final decision, which is conveyed back to the NSACF in the VPLMN. If admission is allowed or not granted normal processing is followed as described in the step.

NSACF nodes to be contacted in all the above are either configured or discovered.

For PDU sessions in the home-routed roaming case, the SMF in HPLMN performs NSAC for the S-NSSAI(s) subject to NSAC.

4.2.11.5.2.2 HPLMN NSAC Admission

Monitoring and enforcement for the maximum number of UEs registered with a network slice monitoring is done by the NSACF in the HPLMN as per the description in Figure 4.2.11.2-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfUEsUpdate_Request service operation the V-AMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the HPLMN.

-	Step 3, the NSAsCF in the HPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN.

For LBO enforcement of the maximum number of LBO PDU Sessions established for an S-NSSAI is performed by the NSACF in the HPLMN as per the description in Figure 4.2.11.4-1 with the following differences:

-	Step 2, in the Nnsacf_NSAC_NumOfPDUsUpdate_Request service operation the V-SMF provides both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN to the NSACF in the VPLMN.

-	Step 3, the NSACF in the HPLMN performs NSAC for both the S-NSSAI in VPLMN and the corresponding mapped S-NSSAI in HPLMN based on the SLA between VPLMN and HPLMN.

NSACF nodes to be contacted in all the above are either configured or discovered

# * * * * Next change * * * *

Table 4.15.3.1-1: List of events for monitoring capability

| Event | Detection criteria | Which NF detects the event |  |
| --- | --- | --- | --- |
| Loss of Connectivity | Network detects that the UE is no longer reachable for either signalling or user plane communication (see NOTE 4). The AF may provide a Maximum Detection Time, which indicates the maximum period of time without any communication with the UE after which the AF is to be informed that the UE is considered to be unreachable (see NOTE 7). | AMF |  |
| UE reachability | Detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g. Periodic Registration Update timer. It indicates when the UE becomes reachable for sending downlink data to the UE. The AF may provide the following parameters: 1) Maximum Latency; 2) Maximum Response Time; 3) Suggested number of downlink packets. (see NOTE 5 and NOTE 7). This event requires the Reachability Filter set to UE reachable for DL traffic" (see clause 5.2.2.3.1-1). For the usage of this event, see clauses 4.2.5.2 and 4.2.5.3. When requesting UE reachability monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF, UDM |  |
| Location Reporting | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (one-time reporting, maximum number of reports, maximum duration of reporting, periodicity, etc. as specified in clause 4.15.1). It reports either the Current Location or the Last Known Location of a UE. When AMF is the detecting NF: One-time and Continuous Location Reporting are supported. For Continuous Location Reporting the serving node(s) sends a notification every time it becomes aware of a location change, with the granularity depending on the accepted accuracy of location (see NOTE 1). For One-time Reporting with immediate reporting flag set, AMF reports the Last Known Location immediately. When AMF is the detecting NF: If the immediate reporting flag is not set, the AMF reports the UE Current Location (In case the AMF does not have the UE current location in the granularity as requested by the location report, the AMF retrieves the information via NG-RAN Location reporting procedure as defined in clause 4.10). When GMLC is the detecting NF: Immediate and Deferred Location Reporting is supported. For Deferred Location Reporting the event types UE availability, Area, Periodic Location and Motion are supported. | AMF, GMLC |  |
| Change of SUPI-PEI association | This event is detected when the association between PEI and subscription (SUPI) changes (USIM change). | UDM |  |
| Roaming status | This event is detected based on the UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) and notification is sent when that status changes. (see NOTE 2). If the UE is registered via both 3GPP and N3GPP Access Type, then both instances of Roaming status are included. | UDM |  |
| Communication failure | This event is detected when RAN or NAS level failure is detected based on connection release and it identifies RAN/NAS release code. | AMF |  |
| Availability after Downlink Data Notification failure | This event is detected when the UE becomes reachable again after downlink data delivery failure. When requesting Availability after Downlink Data Notification failure monitoring, the AF may in addition request Idle Status Indication to be included in the UE reachability event reporting. | AMF |  |
| PDU Session Status | This event is detected when PDU session is established or released. (see NOTE 6) | SMF |  |
| Number of UEs present in a geographical area | This event is detected based on the Event Reporting Information Parameters that were received in the Monitoring Request (Level of aggregation, Sampling ratio, see clause 4.15.1). It indicates the number of UEs that are in the geographical area described by the AF. The AF may ask for the UEs that the system knows by its normal operation to be within the area (Last Known Location) or the AF may request the system to also actively look for the UEs within the area (Current Location). | AMF |  |
| CN Type change | The event is detected when the UE moves between EPC and 5GC. It indicates the current CN type for a UE or a group of UEs when detecting that the UE switches between being served by a MME and an AMF or when accepting the event subscription. (see NOTE 3) | UDM |  |
| Downlink data delivery status | It indicates the downlink data delivery status in the core network. Events are reported at the first occurrence of packets being buffered, transmitted or discarded, including: - Downlink data in extended buffering, including: - First data packet buffered event - Estimated buffering time, as per clause 4.2.3.3 - First downlink data transmitted event - First downlink data discarded event | SMF |  |
| UE reachability for SMS delivery | For SMS over NAS, this event is detected when an SMSF is registered for a UE and the UE is reachable as determined by the AMF and the UDM. For SMS over IP, the event is detected when the UE is reachable as determined by the AMF and the UDM regardless of an SMSF being registered. This enables the UE to receive an SMS. See clauses 4.2.5.2 and 4.2.5.3 (see NOTE 8). | UDM |  |
| UE memory available for SMS | This event is detected when the UE memory is available for SMS as determined by the AMF and the UDM. This enables to trigger Alert for SMS. See clause 5.1.8 of TS 23.540 [84] | UDM |  |
| Number of registered UEs or established PDU Sessions | It indicates the current number of registered UEs or established PDU Sessions for a network slice that is subject to NSAC. For One-time Reporting with Immediate Reporting Flag set, NSACF reports the number of registered UEs or established PDU Sessions immediately. | NSACF |  |
| Number of registered UEs, and number of PDU Sessions to be enforced. | It indicates the number of registered UEs to be enforced by an AMF, number of LBO PDU Sessions to be enforced by an SMF for slice subject to NSAC. | NSACF |
| Area Of Interest | It indicates change of the UE presence in the Area Of Interest. | AMF, GMLC |  |
| NOTE 1: Location granularity for event request, or event report, or both could be at cell level (Cell ID) or TA level. The granularity can also be expressed by other formats such as geodetic uncertainty shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.) which can be mapped by NEF to AMF specific granularity levels. NOTE 2: Roaming status means whether the UE is in HPLMN or VPLMN based on the most recently received registration state in the UDM. NOTE 3: CN type of CN Type change event is defined in clause 5.17.5.1 of TS 23.501 [2]. NOTE 4: In the case of UDM service operation information flow, the UDM should set the subscribed periodic registration timer to a smaller value than the value of Maximum Detection Time, since the value of the mobile reachable timer is larger than the value of the periodic registration timer. NOTE 5: Maximum Latency, Maximum Response Time and Suggested number of downlink packets are defined in clause 4.15.6.3a. NOTE 6: The NEF makes a mapping between the 5GS internal event "PDU Session Status" and the T8 API event "PDN Connectivity Status". NOTE 7: The preferred method for provisioning Network Configuration Parameters is External Parameter Provisioning specified in clause 4.15.6.3a. Provisioning event specific parameters as part of Monitoring Request is expected to be used only by the AF that does not support Parameter Provisioning procedure specified in clause 4.15.6.3a. NOTE 8: The NEF maps between the T8 API event "UE reachability" with reachability type SMS and the 5GS internal event "UE reachability for SMS delivery" for SMS over NAS. The event "UE reachability for SMS delivery" for SMS over IP is used by HSS as described in clause 5.5.6.3 of TS 23.632 [68]. |  |  |  |

# * * * * Next change * * * *

#### 5.2.21.4	Nnsacf_SliceEventExposure services

##### 5.2.21.4.1	General

Service Description: The Nnsacf_SliceEventExposure services provide event based notifications to the consumer NF related to the following :

- 	Current number of UEs registered for a network slice or/and the current number of PDU Sessions established on a network slice.

-	Number of registered UEs and number of LBO PDU sessions to be enforced in a VPLMN in roaming cases

##### 5.2.21.4.2	Nnsacf_SliceEventExposure_Subscribe service operation

Service operation name: Nnsacf_SliceEventExposureSubscribe

Description: This service operation is used by the consumer NF to subscribe or modify a subscription with the NSACF for event based notifications of the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice.

Inputs, Required: Event ID, Event Filter, Event Reporting information.

The Event ID parameter defines two events :

- Event 1: Whether to notify the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice.

- Event 2: Whether to request the number of registered UEs, the number of LBO PDU Sessions to be enforced for a network slice subject to admission.

The Event Filter parameter, applicable to Event 1 and Event 2  is the S-NSSAI for which the current number of UEs registered for a network slice or the current number of PDU Sessions established on a network slice or both are to be notified to the consumer NF, or maximum the number of UEs registered with a network slice and the maximum number of LBO PDU Sessions established on a network slice.

The Event Reporting information parameter, which is applicable for Event 1 only, defines whether the notification is threshold based (e.g. the notification is triggered when the current number of UEs or PDU Sessions with a network slice reaches a defined threshold value) or the notification is periodical (e.g. the notification is triggered at expiry of a periodic timer) and optionally the Immediate reporting flag.

Inputs, Optional: Notification threshold, Notification periodicity.

The Notification threshold parameter, applicable to Event 1 only is optional. It is provided when the Notification is threshold base. The notification threshold parameter may be a numeric value or a percentage of the maximum number of the UEs or PDU Sessions per network slice.

The Notification periodicity parameter is optional. It is provided when the Notification is periodical. The Notification periodicity parameter defines the time between the notification periodicity.

Outputs, Required: Operation execution result response, Subscription Correlation Id.

##### 5.2.21.4.4	Nnsacf_SliceEventExposure_Notify service operation

Service operation name: Nnsacf_SliceEventExposure_Notify

Description: This service operation is used by the NSACF to report the current number of UEs registered with a network slice or the current number of PDU Sessions established on a network slice in numbers or in percentage from the maximum allowed numbers, based on threshold or at expiry of periodic timer.

Inputs, Required: Event ID, Event Filter, Event Reporting information, Notification Correlation Information.

The Event ID parameter defines the type of the reported information, i.e. the number of UEs registered with a network slice or the number of PDUs Sessions established on a network slice, or the maximum number of registered UEs and the maximum number of LBO PDU Sessions to be enforced for a network slice in roaming cases.

The Event Filter parameter is the S-NSSAI for which the Notification applies

The Event Reporting information parameter provides for Event 1, the network slice status information in terms of the current number of UEs registered with a network slice or the current number of PDU Sessions established on a network slice. If the Notification is threshold based where the threshold is a certain number of UEs registered with a network slice or PDU Sessions established on a network slice or the threshold is a percentage of the maximum number of UEs registered with a network slice or the maximum number of PDU Sessions established on a network slice, the Event Reporting information parameter contains confirmation for reaching this threshold value. If the Notification is periodical, the Event Reporting information parameter provides information for the current number of UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of PDU Sessions established on a network slice (e.g. represented in percentage of the maximum number of the PDU Sessions established on the network slice) with periodicity provided during the subscription.

The Event Reporting information parameter provides for Event 2, the maximum number of registered UEs, the maximum number of LBO PDU Sessions to be enforced for a network slice in roaming cases.

Outputs, Required: None.

# * * * * End of changes * * * *