3GPP TSG-SA WG2 Meeting #154AH (e-meeting)	S2-2300250

# Elbonia, January 16th – 20th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0160 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to UE-to-UE Relay Model A Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Model A 5G ProSe UE-to-UE relay discovery, the relay UE includes its target UE list in its Announcement message. To keep an up-to-date target UE list, Sol#9 and Sol#10 propose that 5G ProSe End UEs that want to be discovered by other End UEs via this UE-to-UE Relay send the Announcement message triggered by the Announcement message from the UE-to-UE relay. The UE-to-UE Relay, by implementation, may determine whether to keep or remove specific User Info ID of the UE from the target UE list based on whether it receives the Announcement message from that End UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.3.2.4.2 is updated that UE-to-UE relay periodically sends its Annoucement message, 5G ProSe End UEs that want to be discovered by other End UEs via this UE-to-UE Relay send the Announcement message triggered by the Announcement message from the UE-to-UE relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay may not be able to keep an up-to-date target UE list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

##### 6.3.2.4.2	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

Depicted in Figure 6.3.2.4.2-1 is the procedure for 5G ProSe UE-to-UE Discovery with Model A.

Figure 6.3.2.4.2-1: 5G ProSe UE-to-UE Relay Discovery with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered other UEs in proximity (e.g. via a previous direct discovery or direct communication procedures). The 5G ProSe UE-to-UE Relay obtains the User Info ID of other UEs in proximity.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message periodically to its proximity, announcing its availability as a UE-to-UE relay. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, list of User Info ID of the 5G ProSe End UEs and RSCs. It is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE monitors announcement messages from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	When a 5G ProSe End UE receives a Relay Announcement message, it may decide whether 5G ProSe End UE  wants to be discovered by other End UEs via this relay, according to the RSC and the User Info ID of the relay. If the End UE wants to be discovered by other End UEs via this relay, it may send an Announcement message with its own User Info ID and the relay User Info ID. When the 5G ProSe UE-to-UE Relay receives an Announcement message from an 5G ProSe End UE and the message contains the User Info ID of the relay, the relay will add the User Info ID of that End UE into its target UE list, if the User Info ID is not in the list yet. Otherwise, the 5G ProSe UE-to-UE Relay may remove the User Info ID of that End UE from its target UE list according to its implementation, e.g. if a time out is reached.

* * * * End of Changes * * * *