SA WG2 Meeting #154AHe	S2-2300622

# January 16th – 20th, 2023; Elbonia               	  			  			       		 (revision of S2-230)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3758 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure service in TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2022-12-23 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 4.15.11 Exposure of Events from UPF and 23.501 contain overlapping content … that are not aligned |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ensure that clause 4.15.11 refers to 23.501 for content defined in 23.501: - The list of NF consumer which may receive UPF event notifications is defined in TS 23.501 - To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of mis-alignment between specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.501 CR 3949 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

### 4.15.11	Exposure of Events from UPF

#### 4.15.11.1	General

This clause contains the detailed description and the procedures for UPF event exposure service.

The list of NF consumer which may receive UPF event notifications is defined in TS 23.501 [2] clause 5.8.2,

To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. This is further defined in TS 23.501 [2] clause 5.8.2

#### 4.15.11.2	Information flow for subscription to UPF event exposure service for certain UE(s) via SMF

Figure 4.15.11.2-1: Subscription to UPF event exposure service for certain UE(s) via SMF

In the case of a group of UEs, the UPF event consumer (e.g. NWDAF) first issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the target Group ID and gets the NF profile of the UDM serves this group. Then, NWDAF obtains the list of SUPIs that correspond to the Group ID from UDM using Nudm_SDM_Get

NOTE 1:	It is assumed that all members of a Group ID belong to the same UDM.

Then, for each SUPI,

1.	The UPF event consumer (e.g. NWDAF) invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID, DNN, S-NSSAI and NF type = SMF,.

2.	The UDM provides a Nudm_UECM_Get response with the corresponding SMF.

3.	The UPF event consumer sends the request to the SMF to subscribe to UPF data, including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	Indication of UPF Event Exposure Service and UPF Event Id.

-	Event Filter Information: e.g.  S-NSSAI, Application Id, Area of Interest, SSID/BSSID.

-	Target of Event Reporting: e.g. a UE or a group of UE or Any UE.

4.	The SMF sends the request to the UPF including the UPF event consumer address, Notification Correlation Information, Event Filter Information, Target of Event Reporting, and Target Subscription Information.. Depending on the Event type and as defined in TS 23.501 [2] clause 5.8.2.17), this step may correspond to a Nupf_EventExposure_Subscribe (as defined in clause 5.2.26.2.3) or to a PFCP interaction (as defined in TS 23.501 [2] clause 5.8.2.11)

5.	The UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

#### 4.15.11.3	Information flow for UPF event exposure service to NWDAF for anyUE

When the NWDAF subscription request does not meets the criteria for direct subscription to UPF as described in 4.15.11.1, NWDAF shall subscribe via the SMF. NWDAF send a Nnrf_NFDiscovery_Request to NRF including the related discovery parameters and the NRF returns the SMF(s) which meet(s) the discovery request. NWDAF executes then steps 4-5 in figure 4.15.11.2-1 where the target of the event reporting is in this case anyUE.

When the NWDAF subscription request meets the criteria for direct subscription to UPF as described in 4.15.11.1, NWDAF queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request, and NWDAF sends the Nupf_EventExposure_Subscription directly to UPF.

## END of CHANGES