3GPP TSG-SA WG2 Meeting #154-AH-e		S2-2300632

# Elbonia, January 16 – 20, 2023															 (revison of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0644 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FL client NWDAF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principle 5 for KI#8 in TR 23.700-81 says that NWDAF containing MTLF as FL server may determine the final list of NWDAF containing MTLF as FL clients via initial FL request to FL clients to determine the availability and compatibility of the FL clients. However, in the procedure of Figure 6.2C.2.1-1, (i) it is not clear when the final list of FL client NWDAF is determined, and (ii) local model information reporting is performed before the determination of the final list, which may lead to unnecessary local ML model training and reporting of an NWDAF that would not be included in the final list of FL client NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Server NWDAF determines the final list of Client NWDAFs before the start of FL-based ML training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clarified when and how the list of FL client NWDAF is determined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.2C.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.1-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, including Analytic ID and ML model filter information as described in TS 23.288 [5], the NWDAF containing MTLF can be a FL server (Server NWDAF) with FL server capability or a MTLF without FL server capability.

Editor's note:	Procedure of MTLF registration and discovery with respect to FL is FFS.

Editor's note:	Definition of FL capability is FFS.

1.	Server NWDAF sends a FL request to the selected NWDAF containing MTLF (Client NWDAF) that potentially participates in the Federated learning to perform the local model training for Federated Learning.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	Each Client NWDAF responds with its availability for the FL to the Server NWDAF.

4.	The Server NWDAF determines the final FL list of Client NWDAFs and sends a FL request to the Client NWDAFs in the FL list to initiate ML model training via FL.

5.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF.

The ML model information are exchanged between the Client NWDAF(s) and the Server NWDAF during the FL training process.

Editor's note:	The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS.

Editor's note:	The services in step 1, 3, 5a and 6 to enable FL based ML model training is for FFS, and the Figure 6.2C.2.1-1 will be updated accordingly.

6.	The Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model.

7a.	Based on the consumer request, the Server NWDAF updates the training status (an accuracy level) to the consumer periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g. some accuracy level) is achieved.

7b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g. accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

Editor's note:	Further details on providing the accuracy of FL model process is for FFS.

7c.	According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

8.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

9.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 8.

NOTE:	The steps 5-9 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

* * * * End of Changes * * * *