SA WG2 Meeting #154AHe	S2-2300633

# January 16th – 20th, 2023; Elbonia               	  	       		   (revision of S2-230xxxx)

Title:	[Draft] Reply LS on Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge data network

Response to:	LS S2-2300083 / S6-223487 on Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge data network from SA6

Release:	3GPP Rel-18

Work Item:	FS_eEDGEAPP

Source:	SA2

To:	SA6

Cc:	SA3

Contact Person:

Name:	 Yan Han

E-mail Address:	 hanyan@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR and 23.502 CR

1. Overall Description:

SA2 thanks SA6 for their LS response (S2-2300083 / S6-223487) on FS_eEDGEAPP Solution for support of NAT deployed within the edge data network.

The questions in the incoming LS from SA6 in S6-223586 are as following

-	Q.1 Can SA2 provide support for the core network APIs (e.g., Nnef_UEId_Get) to resolve NATed IP address and Port number to UE ID as part of Rel-18 normative work?

-	Q.2 SA2 mentioned “The above DNN/S-NSSAI or ipDomain attribute is not expected to be exposed outside the 5GC network”. SA6 understands that DNN, S-NSSAI or ipDomain information can be exposed to AF in the PLMN trusted domain, and DNN/S-NSSAI can be exposed to UE (e.g., in URSP). SA6 would request SA2 to confirm this understanding.

-	Q.3 Can UE be provided with the ipDomain information together with UE IP address by the core network when UE IP address is allocated by the core network?

SA2 has some discussion the answers are below:

- For Q1, SA2 has agreed to the attached CR to provide UE IP address translation information exposure (e.g., UE private IP address for a UE public IP address) by the UPF if the UPF employs the NAT functionality. After obtaining the UE private IP address as an input, the AF can get the UE ID via Nnef_UEId_Get.

- For Q2, there is no existing specification in SA2 shows that DNN, S-NSSAI or ipDomain information can be exposed by NEF outside the 5GC network. AF may obtain these information from AS, but not by exposed from 5GC network. So it is not exact to say ‘DNN, S-NSSAI or ipDomain information can be exposed to AF’. Also, UE may get DNN/S-NSSAI through signal, but not through exposed by NEF. Using ‘exposed’ here is not correct.

- For Q3, there seems no requirement to provide ipDomain information together with UE IP address to UE in the current specifications.

.

2. Actions:

To SA6:

ACTION: 	SA2 kindly requests SA6 to take the above information into consideration.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #155	20-14 February 2023	Athens, GR

TSG-SA2 Meeting #156	  17-21 April 2023	              emeeting