SA WG2 Meeting #154-AH-e	S2-2300635

# January 16th – 20th, 2022; Elbonia              		(revision of SP-220799)

Source: 	China Telecom

Title: 	Revised WID: Dynamically Changing AM Policies in the 5GC Phase 2

Document for: 	Approval

Agenda Item: 	10.3

Work Item / Release:	TEI18_DCAMP_Ph2 / Rel-18

Abstract of the contribution: Revised WID: Dynamically Changing AM Policies in the 5GC Phase 2

Background:  updates the WID completion date in line with the work plan sheet that indicates that this WID needs to be completed in Q2.

TSG SA Meeting #SP-97-e	SP-220799

13 - 19 September 2022, Electronic meeting

# Source:	SA WG2

# Title:	New WID: Dynamically Changing AM Policies in the 5GC Phase 2

Agenda Item:	6.5

3GPP SA WG2 Meeting #152e 	S2-2207835

Electronic Meeting, 17 August – 26 August 2022	(revision of S2-2206740r03)

Source:	China Telecom, China Unicom, ZTE, vivo, CATT, Inspur, Huawei, Spreadtrum Communications, InterDigital

Title:	New WID on Dynamically Changing AM Policies in the 5GC Phase 2 (TEI18 _DCAMP_Ph2)

Document for:	Approval

Agenda Item:	10.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Dynamically Changing AM Policies in the 5GC Phase 2

Acronym:	TEI18_DCAMP_Ph2

Unique identifier:	970009

Potential target Release:	{Rel-18}

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 870005 | Dynamically Changing AM Policies in the 5GC | Rel-17 Work Item |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

3	Justification

The access and mobility policy control introduced in 5G encompasses the management of service area restrictions, the management of the RFSP functionalities, the management of the UE-AMBR, the management of the UE Slice-MBR and the management of the SMF selection by PCF.

Some potential enhancements on AM policy are identified based on the Rel-17 SA2 issues and requirements:

- As conclusion from DCAMP, the PCF serving a UE may receive notification from AF or from PCF(s) serving a PDU session that specific application traffic starts/stops, and then trigger AM policy check and update. More than one application is likely to be notified as “active” at the same time. However, the solution in R17 is only applicable to non-roaming scenario. To support AF’s requirement can be applied to roaming UEs, e.g. in LBO scenario where PCF for UE and the PCF for PDU session belong to VPLMN, enhancement on the current mechanism of dynamically changing AM policies for AF's request is needed.

4	Objective

The objective is to enhance the dynamically changing AM policies, as follows:

Objective 1: To define the mechanism of supporting DCAMP in roaming scenario. This objective will focus on providing dynamic AM policy for any roaming UEs in LBO roaming scenario.

Note: This work will require 0.5 TU.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Update the procedures to show how the AF influences AM policy decisions in LBO roaming scenario. | SA#99 (Jun. 2023) | This will be handled as “TEI18”. |

6	Work item Rapporteur(s)

Yubing Liu, liuyb9@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

China Unicom

ZTE

vivo

CATT

Inspur

Huawei

Spreadtrum Communications

InterDigital