3GPP TSG-SA WG2 Meeting #154-AH-e		S2-2300825

# Elbonia, January 16 – 20, 2023															 (revison of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3794 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NEF service used in correlating UE data collection and NWDAF request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Mirror CR for Rel-17 NEF service used in correlating UE data collection and NWDAF request. In clause 6.2.8.2.4.3 of TS 23.228, Nnef_UEId_Get service is used for AF to retrieve UE IP address for a given GPSI to correlate UE data collection and NWDAF request. However, the Nnef_UEId_Get service is designed to retrieve UE ID for a given UE IP address with its dedicated procedure in clause 4.15.10. To correct such misalignment, it is proposed to introduce a new NEF service to get UE IP address rather than revising the Nnef_UEId_Get service description in order to confusion and minimize the impact on the conventional NEF operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New NEF service description aligned to support that AF in untrusted domain correlates UE data collection and NWDAF request in clause 6.2.8.2.4.3 of TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between NEF service description and the procedure for correlating UE data collection and NWDAF request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.31 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0660 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution is Mirror CR of S2-2300743 (CR3783). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * * (All new text)

5.2.6.31	Nnef_UEAddress service

5.2.6.31.1	General

See clause 6.2.8.2.4.3 of TS 23.288 [50].

5.2.6.31.2	Nnef_UEAddress_Get operation

Service operation name: Nnef_UEAddress_Get

Description: Get the UE IP address (i.e., IPv4 address or IPv6 prefix).

Inputs, Required: GPSI, AF Identifier.

Inputs, Optional: None.

Outputs, Required: Result, UE address (i.e., IPv4 address or IPv6 prefix).

Outputs, Optional: None.

* * * * End of Changes * * * *