3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300853

January 16th – 20nd, 2023; Elbonia

Source:	Samsung

Title:	Discussion on NSSRG restriction across different access types over different PLMNs

Document for:	Discussion

Agenda Item:	8.4

Work Item / Release:	eNS_Ph2/ Rel-17

Abstract: Discussion on NSSRG restriction across different access types over different PLMNs.

# 1. Introduction

This discussion paper is about how to reply to the LS on NSSRG restriction across different access types over different PLMNs from CT1 (S2-2300023/ C1-227101).

Question #1: Shall the Requested NSSAI across different access types over different PLMNs share common NSSRG value(s)?

Question #2: If the answer of Question #1 is yes, how does the network (i.e. different AMFs in different PLMNs) enforce the common NSSRG value(s) restriction on the Requested NSSAI?

### 1.1  Discussion on Question#1

The incoming LS relates to 3GPP TS 23.501 clause 5.15.12 which specifies subscription-based restrictions to simultaneous registration of network slices:

“When the serving AMF provides the Configured NSSAI to the UE, and the UE has indicated it supports the subscription-based restrictions to simultaneous registration of network slices feature, the AMF also provides the UE with the NSSRG information related to the S-NSSAIs of the HPLMN which are in the mapping information of the Configured NSSAI. A UE which receives the NSSRG values in the network slicing configuration information shall only include in the Requested NSSAI S-NSSAIs that share a common NSSRG as per the received information.”

Observation 1: The yellow color text indicates that the UE receives the NSSRG information for those slices which are present in the Configured NSSAI.

TS 23.501 clause 5.15.3 describes how the UDM provides subscribed NSSAI-S and corresponding NSSRG information.

In roaming case, the UDM shall provide to the VPLMN only the S-NSSAIs from the Subscribed S-NSSAIs the HPLMN allows for the UE in the VPLMN. If the UE is subject to restrictions of simultaneous registration of network slices (i.e. if the Subscription Information for the S-NSSAIs contains NSSRG information), the UDM provides to the VPLMN a subscribed S-NSSAIs and, if applicable, NSSRG information, as described in clause 5.15.12.

One example is given to understand both the Subscribed NSSAI and NSSRG information during roaming.

In UDM:

Subscribed NSSAI : S1, S2, S3, S4, S5, S6, S7, S8

NSSRG Information: NSSRG1: (S1,S2), NSSRG2: (S3,S4), NSSRG3: (S5,S6), NSSRG4: (S7,S8)

In VPLMN1: (As per the SLA, HPLMN does not allow S5, S6, S7, S8)

During registration VAMF gets Subscribed NSSAI : S1, S2, S3, S4 and NSSRG Information: NSSRG1: (S1,S2), NSSRG2: (S3,S4) from UDM. UE gets Configured NSSAI: S1, S2, S3, S4 and NSSRG Information: NSSRG1: (S1,S2), NSSRG2: (S3, S4)

In VPLMN2: (As per the SLA, HPLMN does not allow S1, S2, S3, S4)

During registration VAMF gets Subscribed NSSAI : S5, S6, S7, S8 and NSSRG Information: NSSRG3: (S5,S6), NSSRG4: (S7,S8). UE gets Configured NSSAI: S5, S6, S7, S8 and NSSRG Information: NSSRG3: (S5,S6), NSSRG4: (S7, S8)

Observation 2: UE receives two different set of NSSRG information from two PLMN which is based on the Configured NSSAI.

Conclusion: By taking observation 1 and observation2 into consideration it is clear that UE receives a different set of NSSRG information based on the different Configured NSSAI it receives over different PLMN.

# 2. Conclusion and proposal(s)

Conclusion 1: The answer to Question#1 is NO. Because the UE is being provided two different set of NSSRG information based on two different Configured NSSAI which is applicable for corresponding PLMN. Hence UE will send the Request NSSAI with slices which shares a common NSSRG information within a PLMN.

Proposal: It is proposed that SA2 agrees to reply the LS on NSSRG restriction across different access types over different PLMNs from CT1 (S2-2300023/ C1-227101) based on the above discussion.