3GPP TSG-WG SA2 Meeting #154-AH	S2-2300934

# e-Meeting, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4020 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wait range during discontinuous coverage. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2, the support of mobility management and power saving with discontinuous coverage is studied and concluded. This CR introduces these conclusions introduced as part of 8.2 in 23.700-28. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of overload control due to discontinuous coverage as per agreement in 23.700-28 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study agreements are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

### 5.4.X Support of discontinuous network coverage

#### 5.4.X.y	Overload control

In order to reduce the impact due to large number of UEs triggering signalling load on the network returning after discontinuous coverage, the AMF determines the wait range based on network configuration and sends the Disco wait range information to the UE via the Registration procedure or UE Configuration Update procedure. If the UE has decided to remain in no service (e.g. by applying power saving) due to discontinuous coverage and wait until the same RAT/PLMN coverage to return, the UE will calculate the wait timer based on Disco wait range information received from the network (e.g. by selecting a random value between 0 and Disco wait range information). The UE starts the respective wait timer when returning to coverage after being in discontinuous coverage to reduce signalling overload on the same RAT/PLMN. The UE will not initiate any NAS signalling when the wait timer is running.

Editor's note:	How to negotiate UE/network support of Disco wait range information is FFS. Whether separate support indication is necessary or common support indication for all features of 5GSAT_ph2 has to be used is FFS.

# * * * * End of changes * * * *