3GPP TSG-WG SA2 Meeting #154-AH-E	S2-2301005

Elbonia, January 16 - 20, 2023	(revision of S2-230xxxx)

Source:	Lenovo

Title:	KI#1: Further analysis on Alternative S-NSSAI determination and provision to UE

Document for:	Discussion and approval

Agenda Item:	9.11.2

Work Item / Release:	eNS_Ph3 / Rel-18

Abstract: this paper proposes an analysis and agreement on open issues related to the Alternative S-NSSAI in KI#1.

# 1. Introduction/Discussion

## 1.1.	Motivation

The FS_eNS_Ph3 study has concluded in TR 23.700-41 V2.0.0 clause 8.1 how to proceed in the normative phase.

However, there are several open issues:

- Does the network (e.g. AMF, NSSF) determine an Alternative S-NSSAI independent which UEs are registered and using the temporarily unavailable SNSSAI?  Or does the network (e.g. AMF, NSSF) determine different Alternative S-NSSAI for each UEs registered currently for the temporarily unavailable SNSSAI?

- How is the Alternative S-NSSAI provided to the UE in different scenarios:

  - In non-roaming scenario;

  - In roaming scenario for VPLMN S-NSSAI;

  - In roaming scenario for HPLMN S-NSSAI?

Apparently further discussions and agreements are needed during the normative phase.

## 1.2.	Analysis

Regarding the issue 1):

- Does the network (e.g. AMF, NSSF) determine an Alternative S-NSSAI on system level, i.e. independent which UEs are registered and using the temporarily unavailable SNSSAI?  Or does the network (e.g. AMF, NSSF) determine different Alternative S-NSSAI for each UEs registered currently for the temporarily unavailable SNSSAI?

The following analysis is provided:

- If the Alternative S-NSSAI is determined on system level (i.e. independent which UEs are registered and using the unavailable SNSSAI), it would result in a single (or small number) of Alternative S-NSSAIs for a single temporary unavailable SNSSAI. It is easier for the network to determine a common Alternative S-NSSAI considering the system deployment and the load conditions (e.g. using the NWDAF analytics).

- If the Alternative S-NSSAI is determined on UE level (i.e. for each UE registration for the unavailable SNSSAI, an individual S-NSSAI is determined), it would result in multiple Alternative S-NSSAIs for a single temporary unavailable SNSSAI. The Alternative S-NSSAI is determined for each UE registration individually considering the UE's subscribed S-NSSAIs. This process is more resource consuming in the network. Also, many UEs are usually subscribed to few S-NSSAIs (single or two S-NSSAIs) and for them anyhow the Alternative S-NSSAI has to be determined as not part of the Subscribed S-NSSAIs.

It is proposed to specify option A in the normative phase. The network determines one or more Alternative S-NSSAIs for temporary unavailable S-NSSAI.  For each registered UE using the temporary unavailable S-NSSAI, the AMF decides the Alternative S-NSSAIs to use. In one example, the network may determine 2 Alternative S-NSSAIs and for some UEs it decides to use one Alternative S-NSSAI and for other UEs the other Alternative S-NSSAI.

Regarding the issue 2):

- How is the Alternative S-NSSAI provided to the UE in different scenarios:

  - In non-roaming scenario;

  - In roaming scenario when the VPLMN S-NSSAI has to be replaced (in any of the LBO or HR traffic case);

  - In roaming scenario when the HPLMN S-NSSAI has to be replaced (in HR traffic case)?

The following analysis is provided:

Scenario A (non-roaming case): the AMF provides the mapping of the unavailable S-NSSAI form the Allowed NSSAI to the Alternative S-NSSAI. As in non-roaming case the mapping of Allowed NSSAI IE cannot be used, it is proposed to introduce a new IE to indicate the mapping of the temporary unavailable S-NSSAI to the Alternative S-NSSAI.

Scenario B (roaming case when the VPLMN S-NSSAI has to be replaced): the AMF provides the Alternative S-NSSAI in a new updated Allowed NSSAI. There is no need to provide the mapping of the unavailable VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI, as the VPLMN has the opportunity to determine any VPLMN S-NSSAI to be used in the VPLMN and use the existing mapping of Allowed NSSAI IE in the signalling to the UE.

Scenario C (roaming case when the HPLMN S-NSSAI has to be replaced): when the AMF gets a trigger that an HPLMN S-NSSAI has to be replaced by an Alternative S-NSSAI, the AMF needs to have means to provide to the UE the (temporary) replacement of the HPLMN S-NSSAI to the Alternative S-NSSAI. Such replacement is needed in the UE in order to keep and use the current URSP rules which include only HPLMN S-NSSAI values. So, such replacement needs to be provided in addition to the existing mapping of Allowed NSSAI IE. Therefore, it is proposed to use a new IE to indicate the mapping of the temporary unavailable S-NSSAI to the Alternative S-NSSAI.

# 2. Proposal

The following is proposed:

Proposal 1:  The network determine one or more Alternative S-NSSAIs on system level. For each registered UE using the temporary unavailable SNSSAI, the AMF decides the Alternative S-NSSAIs to use.

Proposal 2:  for roaming UEs when the VPLMN S-NSSAI has to be replaced, the network provides to the UE a new Allowed NSSAI and the existing mapping of Allowed NSSAI IE.

Proposal 3:  roaming case when the HPLMN S-NSSAI has to be replaced, the network provides to the UE a new IE to indicate the mapping of the temporary unavailable HPLMN S-NSSAI to the Alternative S-NSSAI.