SA WG2 Meeting #S2-154-AH-e	S2-2301097

16 - 20 January, 2023, Electronic

Title:	[DRAFT] Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge data network

Response to:	S6-223487 / S2-2300083

Release:	Release 18

Work Item:	FS_eEDGEAPP, UPEAS

Source:	SA2

To:	SA6

Cc:	SA3

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23501CR3829, 23.502CR3675

1. Overall Description:

SA2 thanks SA6 for their LS on “FS_eEDGEAPP Solution for Support of NAT deployed within the edge data network” (S6-223487 / S2-2300083).

SA2 would like to provide the following answers to the SA6 questions.

Q.1 Can SA2 provide support for the core network APIs (e.g., Nnef_UEId_Get) to resolve NATed IP address and Port number to UE ID as part of Rel-18 normative work?

SA2 is delighted to inform SA6 that SA2 has agreed the attached Rel-18 CRs that extend the Nnef_UEId_Get service operation to allow the AF to query the 5G system with a combination of IP address and Port information in order to obtain an AF-specific UE identifier.

Q.2 SA2 mentioned “The above DNN/S-NSSAI or ipDomain attribute is not expected to be exposed outside the 5GC network”. SA6 understands that DNN, S-NSSAI or ipDomain information can be exposed to AF in the PLMN trusted domain, and DNN/S-NSSAI can be exposed to UE (e.g., in URSP). SA6 would request SA2 to confirm this understanding.

SA2 would like to point out that DNN, S-NSSAI and IP domain are available today in the Nnef_UEId_Get service operation (see TS 23.502 clause 5.2.6.27.2) for use by an AF. Whether these parameters should be exposed to a trusted AF is a deployment decision.

Q.3 Can UE be provided with the ipDomain information together with UE IP address by the core network when UE IP address is allocated by the core network?

The IP domain reflects internal network configuration and is not intended to be disclosed to the UE.

2. Actions:

To SA6 group.

ACTION: 	SA2 respectfully asks SA6 to take into account the feedback above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#155			20 - 24 February 2023			Athens, GR