3GPP TSG-SA2 Meeting #154AHE	S2-2301285

# Elbonia, Jan 16 – 20, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0694 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on PFD Determination analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2022-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The clause 6.16 in TS 23.288 defines the PFD Determination analytics, but the text has misaglignment an inconsistency (e.g., wrong analytics names) throughout the clause. And, the figure also needs to be adjusted to include NFs for data collection. This CR is to correct errors and clarify the procedure for PFD Determination analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct texts and update the figure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and errorneous specification for PFD analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

6.16	PFD Determination Analytics

6.16.1	General

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFDs to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer may forward new or updated PFD information provided by the NWDAF to the UPF via the SMF to detect a known application as defined in TS 23.502 [3].

The service consumer may be an NEF(PFDF).

The consumer of the analytics may indicate in the request or subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-	Application ID

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated PFDs);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a specific S-NSSAI, DNN, and Application ID, and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data for PFD Determination

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that uses the application |
| S-NSSAI | SMF, UPF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF, UPF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Application ID | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

6.16.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.16.3-1. The output analysis can be used to provision new or updated PFDs for known applications.

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application traffic description |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing IP 3-tuples(destination IP address, port, and protocol of uplink packets from UE(s)). |
| > URL list | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | A Domain Name matching criteria and information about applicable protocol(s). |

6.16.4	Procedures

The Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (i. e., NEF(PFDF)) requests or subscribes to the NWDAF to request PFD Determination analytics. The Target of Analytics Reporting is set to Any UE.

This subscription may be triggered by local configuration or OAM. The Analytics Filter Information may optionally include the S-NSSAI and/or DNN. A single Application ID or a list of Application IDs are also provided as part of the Analytics Filter information.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF).

3.	The NWDAF collects required information from the UPF, including URL, Domain name part, and IP 3-tuples of packets of the SDF that does not match with the installed PDRs.

Editor's note:	UPEAS WI is specifying UPF event exposure for NWDAF data collection. It is FFS whether clause 4.15 of TS 23.502 [3] and the other UPEAS normative work address the needs of this new analytic.

4.	The NWDAF subscribes to the SMF and collects required information.

5.	The NWDAF derives PFD Determination analytics, e.g., new or updated PFD information for the existing Application ID.

6.	The NWDAF notifies PFD Determination analytics to the Consumer NF (i. e., NEF(PFDF)).

* * *End of Changes * * *