3GPP TSG-SA WG2#154-AH-e	S2-2301616
Electronic, 16 – 20 Jan, 2023	(merge of S2-2301266r02)

Source:	OPPO, InterDigital Inc., Lenovo, Intel, Samsung, Apple, Nokia, Nokia Shanghai Bell

Title:	Updating to KI#1 Conclusion

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This contribution proposed updating of conclusion for KI#1

1.	Summary

.

Based on the show of hands decision, Option C was selected without objections.

2.	Text proposal

It is proposed to agree the following changes in TS 23.700-85 as below

# * * * * Start of change * * * *

8.1	Conclusions on KI#1

8.1.1	General

The following definitions will be included as part of the normative work on URSP Rules in roaming:

VPLMN specific URSP Rules: A VPLMN specific URSP Rule is applicable when the UE is registered in the VPLMN or its equivalent VPLMN only. VPLMN specific URSP rules are provided from the HPLMN and contains, based on agreements with VPLMN, HPLMN values for Network Slice Selection Policies and DNN Selection Policies. When provided, the Time and Location criteria in each of the RSD contain VPLMN values. It is provided to the UE to route traffic on a PDU Session to a SMF and UPF in the VPLMN.

VPLMN configuration is used to ensure that the local VPLMN values are used when a PDU Session is established for a roaming UE.

When the UE sends a PDU Session Establishment the HPLMN values that are included in the S-NSSAI and DNN need to be translated to VPLMN values to be able to select a SMF at the VPLMN. This means that the AMF select a local DNN that is used to select a SMF in the VPLMN. How the AMF selects a local DNN is to be decided during normative phase. The AMF also sends the local DNN, if decided, as “Selected DNN” to the SMF, while the DNN requested by the UE is sent as “Requested DNN”.

The VPLMN provides the service parameters to HPLMN for HPLMN to generate the VPLMN specific URSP rules. The H-PCF creates VPLMN specific URSP rules corresponding to different VPLMN. The H-PCF may provide the VPLMN specific URSP rules to the UE. This can be triggered by the UE's registration in the VPLMN or it can happen before UE roams into the VPLMN. The URSP Rules received by UE in VPLMN are only applicable when the UE is registered in that VPLMN or its equivalent VPLMNs. If the UE does not find a match using the URSP rules associated with the VPLMN ID or equivalent VPLMN ID, it uses the URSP rules associated with the HPLMN.

NOTE:  	The PCF may provide VPLMN specific URSP rules based on existing PCRT report from PCF such as PLMN Change or other triggers.

8.1.2	Conclusions on the how to identify PLMN specific URSP Rules

To enable the PCF to provide and the UE to identify the PLMN specific URSP Rules, the following conclusion principles apply:

-	VPLMN ID is provided to UE along with the URSP.

-	The PCF provides to UE a list of PSIs associated to the HPLMN ID and a list of PSIs associated to each VPLMN ID with roaming agreements (following the principles of Solution #6).

# * * * * End of changes * * * *